The MERLIN-Expo Root model

V1.1

Author: Taku Tanaka1

1EDF R&D, 6 quai Watier, 78400 Chatou, France

taku.tanaka@edf.fr

Reviewers: Philippe Ciffroy1

Content

Level 1 documentation (basic knowledge on model purpose, applicability and components)	4

1.	Model purpose	4

1.1.	Goal	4

1.2.	Potential decision and regulatory framework(s)	4

2.	Model applicability	4

2.1.	Spatial scale and resolution	4

2.2.	Temporal scale and resolution	4

2.3.	Chemical considered	4

2.4.	Steady-state vs dynamic processes	4

3.	Model components	5

3.1.	Media considered	5

3.2.	Loadings, losses, and exchanges between different media	5

3.3.	Coupling with other models	6

3.4.	Forcing variables	7

3.5.	Parameters	9

3.6.	Intermediate State variables	11

3.7.	Regulatory State variables	15

Level 2 documentation (background science)	17

4.	Processes and assumptions	17

4.1.	Process n°1: Partition between phases	17

4.2.	Process n°2: Xylem influx	18

4.3.	Process n°3: Xylem outflux	19

4.4.	Process n°4: Degradation in root compartment	19

4.5.	Process n°5: Uptake of metals	19

Level 3 documentation (numerical information)	21

5.	Numerical default values (deterministic and/or probabilistic)	21

5.1.	Initialization of mass balance in Media	21

5.2.	Default parameter values	21

5.2.1 Site-specific and plant parameters	21

5.2.2 Parameters related to the partition between phases	23

5.2.3 Parameters related to the xylem influx	39

5.2.4 Parameters related to the chemical degradation	40

5.2.5 Parameters related to the transfer from soil to root governed by the equilibrium transfer factor	41

Level 4 documentation (mathematical information)	43

6.	Mass balance equation	43

7.	Calculation of state variables	44

7.1	Xylem influx (Xylem_Influx)	44

7.1.1	Transpiration stream (Transpiration)	44

7.1.2	Leaf area index for root crops (LAIroot)	45

7.1.3	Distribution coefficient of the pollutant between soil and water (Kd_soil)	45

7.2	Xylem outflux (Xylem_ Outflux)	46

7.2.1	Mass of root per unit area of soil (m_root)	46

7.2.2	Partition coefficient between root and water (K_root_water)	47

7.2.3	Partition coefficient between air and water (K_air_water)	47

7.3	Loss by degradation in root (Loss_deg_root)	48

7.4	Uptake of metals (Uptake_metals)	48

7.5	Concentration in root at harvest (C_root)	49

Reference	50

# Level 1 documentation (basic knowledge on model purpose, applicability and components)

# Model purpose

## Goal

The Root model is used to estimate the time-dependent accumulation (in mass and concentration bases) of organic/metals in the edible part in root crops (such as carrot, radish, turnip, and etc) at harvest.

## Potential decision and regulatory framework(s)

Coupled with the information about the ingestion rate of root crops (kg fresh weight d-1), the root model can estimate the human exposure to organic substances/metals through the ingestion of root crops. This output can be used for evaluating the risk to exceed regulatory thresholds for human health or used as an input for PBPK models.

# Model applicability

## Spatial scale and resolution

The Root model consists of one below-ground ‘root’ compartment for organic substances and metals. The compartment is fixed in space and is treated as being homogeneous, i.e. well-mixed in chemical composition, that is, the Root model considers no spatial variation of chemical concentration in the compartment.

When model users need to consider, in their own scenarios, several regions with different levels of contamination in soil and/or air, they can set up multiple Root models and make the models correspond to different levels of contamination in soil and/or air to cover several regions in their scenarios.

## Temporal scale and resolution

All the transfer processes considered in the Root model are expressed on a daily basis, and thus the model should be run by the daily simulation setting.

## Chemical considered

The Root model can be applicable for a variety of chemicals including many types of organic substances (such as PAHs, PCBs, Dioxins, VOCs) and also metals for which soil-to-plant transfer factors were found in the literature (i.e. Al, As, Cd, Cr, Cu, Fe, Mn, Pb, Zn). However, it should be noted that the model used for organic chemicals is limited to neutral compounds; it is not really applicable to ionic or dissociating compounds. For example, practically all herbicides and all ‘systemic’ pesticides are weak bases (Trapp, 2004). Also many of pharmaceuticals are weak bases or acids.  These electrolytes undergo more complicated processes inside plants than neutral compounds.

## Steady-state vs dynamic processes

The Root model is governed by time-dependent (dynamic) transfer processes with the following exceptions:

- Chemical transfer of organic substances from soil to the root zone: the chemical transfer from soil to the root zone is governed by the time-dependent transpiration stream which takes up water and solutes by roots and transports them into leaves, but also by the equilibrium partition coefficient between soil particles and soil pore-water for organic substances (Kd_soil).

- Chemical transfer of organic substances from roots to above-ground shoot: the chemical transfer from roots to above-ground shoot is governed by the time-dependent transpiration stream (xylem flow) but also by the equilibrium partition coefficient between roots and water (Kroot_water), which is derived from octanol-water partition coefficient (Kow) and air-water partition coefficient (Kair_water).

- Chemical transfer of metals from soil to roots: the transfer of metals into roots is represented by equilibrium transfer factors. The transfer factor is expressed by the ratio of the concentration in root to the concentration in soil.

# Model components

## Media considered

Definition: A ‘Medium’ is defined as an environmental or human compartment assumed to contain a given quantity of the chemical. The quantity of the chemical in the media is governed by loadings/losses (see 3.2 and 3.3) from/to other media and by transformation processes (e.g. degradation).

The Root model is composed by a single ‘Root’ compartment as follows:

- ‘Root’. In this media, a time-dependent chemical mass (mg) in roots is calculated from the mass balance equation.

## Loadings, losses, and exchanges between different media

Definition:

- A ‘Loading’ is defined as the rate of release/input of the chemical of interest to the   receiving system, here the root and fruit media;

- A ‘Loss’ is defined as the rate of output of the chemical of interest from the receiving system;

- An ‘Exchange’ is defined as the transfer of the chemical of interest between two media of the system.

The loading and loss processes considered in the Root model for organic substances are same as those defined in the root compartments of different plant models such as the Fruit, Leaf, and Grain models.

In the Root model, the transfer processes for organic substances are assumed to be different from those for metals. The loading and loss processes for organic substances are based on mechanistic assumptions such as the transpiration stream and the first-order decay rate whereas the transfer of metals from soil to roots is represented only by equilibrium transfer factors.

Each process is presented in the following table par each type of target substances (organic or metal):

Table 1 Loading and loss processes in the Root model

| The loading and loss processes in root compartment |  |
| --- | --- |
| For organic substances | For metals |
| Input from soil to root by transpiration stream | Input from soil to root governed by equilibrium transfer factor |
| Loss from root to above-ground shoot by transpiration stream |  |
| Loss by chemical degradation |  |
| Loss by harvest | Loss by harvest |

In addition to the loading and loss processes, several inter-phase exchange processes are considered in the Root model (for organic substances). The root compartment takes in account the equilibrium partitioning between concentrations in roots (mg kg-1 fresh weight) and in water (xylem sap, mg L-1). The equilibrium partitioning between concentrations in soil pore water (mg m-3) and in soil particles (mg kg-1) is used to calculate the fraction of a contaminant dissolved in water contained in the soil compartment, which is taken upward to root by xylem flow.

In the Root model, all the transfer processes take place only during the growing period of a root crop, that is, only between the germination time and harvest time (the model uses a syntax named “flag_cultivating” to control this). Out of the period, no chemical accumulation arises in the Root model. At harvest time, the chemical mass accumulated in root and root compartments gets completely erased (set to zero), which is expressed in the model by the process of ‘Loss by harvest’ (see Table 1). After the harvest time, the transfer processes restart from the germination time in the next growing season.

Figure 1 and 2 present all the input/loss/exchanges processes for organic substances and metals, respectively.

Figure 1 Input and loss processes in the Root model (for organic substances)

Figure 2 Input and loss processes in the Root model (for metals)

## Coupling with other models

The Root model can potentially be coupled with other models stored in the Merlin-Expo library. These models can estimate loadings as input data used for the Root model.

Table 2 Potential models to be coupled with the Root model

| Coupled model | Loadings estimated by coupled models |
| --- | --- |
| Soil model | The soil model estimates the mass of the organic/metal substance accumulated in the surface soil (or root zone soil). This mass of the substance in soil pore water is used as an input to calculate: - The transfer of organic substances from soil to root by transpiration stream. This total mass of the substance in soil is used as an input to calculate: - The transfer of metals from soil to root governed by equilibrium transfer factors. |

Figure 3 and 4 present the input and loss processes for the Root model together with other models relevant for the processes.

Figure 3 Potential linkages with other models (for organic substances)

Figure 4 Potential linkages with other models (for metals)

## Forcing variables

A ‘Forcing variable’ is defined as an external or exogenous (from outside the model framework) factor that influences the state variables calculated within the model. Such variables include, for example, climatic or environmental conditions (temperature, wind flow, etc.).

Some forcing variables are used to calculate the external loadings from air and soil to the Root model and also the internal processes within the Root model.  They can be given by sets of time-dependent values. The forcing variables can be calculated by other models (the atmosphere and soil models in this case) or can be given by model users (in the case that the users have measurement data or their own estimations). The following forcing variables are defined in the Root model. The column ‘Substance’ allows distinguishing the forcing variables used for organic substances from those for metals.

Table 3 Forcing variables used in the Root model

| Name | Substance | Abbreviation and unit | Description | Given by… |
| --- | --- | --- | --- | --- |
| Concentration in soil | Organic and Metal | C_soil (mg kgdw-1) | It is the concentration of the pollutant in root zone of soil (on dry mass basis). It is used to calculate (i) the transfer of pollutant from soil to root by the transpiration stream (Xylem_influx) and (ii) the metal uptake from soil to roots (Uptake_metals). | Users |
| Actual evapotranspiration | Organic | ET_a (mm d-1) | It is used to calculate the transpiration stream (Transpiration). | Soil model or users |
| Temperature of air | Organic | T_air (°C) | It is used to calculate the saturation vapour pressure of water at a given air temperature (p_water_sat). | Users |

## Parameters

A ‘Parameter’ is defined as a term in the model that is fixed during a model run or simulation but can be changed in different runs as a method for conducting sensitivity analysis or to achieve calibration goals.

The parameters used in the Root model are listed in the following tables. Table 4 presents all the parameters alphabetically by their abbreviations. The column ‘Substance’ allows distinguishing the parameters used for organic substances from those for metals.

Table 4 Parameters used in the Root model

| Name | Abbreviation | Description | Unit | Substance |
| --- | --- | --- | --- | --- |
| Extinction factor for partitioning of evapotranspiration to plant transpiration | alpha_extinction | The extinction factor for total solar irradiance functions as partitioning radiant energy between canopy and soil surface, that is, as partitioning evapotranspiration between soil evaporation and plant transpiration | unitless | Organic |
| Correction factor for density difference between water and n-octanol | delta_density_OW | Factor correcting difference between densities of water and of n-octanol | L kg-1 | Organic |
| Empirical correction factor for differences between solubility in octanol and sorption to root lipids | delta_solubility_lipids_root | Empirical factor correcting differences between solubility in octanol and sorption to root lipids | unitless | Organic |
| Fraction of organic matter in soil | f_OM_soil | - | g g-1 | Organic |
| Air content of root | G_root | Air volume contained in root per fresh weight of root | L kgfw-1 | Organic |
| Henry's law constant | H | Ratio of a chemical’s abundance in gas phase (vapour pressure) to that in aqueous phase (solubility) at equilibrium at a given temperature | Pa m3 mol-1 | Organic |
| Lipid content of root | L_root | Lipid quantity contained in fresh weight of root | kg kgfw-1 | Organic |
| Leaf area index at harvest for root crops | LAI_root_harvest | One-sided leaf area per unit ground surface area at harvest time for root crops | (unitless or m2 (of leaves for root crops) m-2 (of soil)) | Organic |
| Degradation rate in root | lambda_deg_root | First order decay rate of pollutants in root | d-1 | Organic |
| Water-organic carbon partition coefficient (in log10) | log10_K_oc | Ratio of the equilibrium concentration of a chemical in water to that in soil organic carbon | unitless (L kg-1 in normal scale) | Organic |
| Octanol-water partition coefficient (in log 10) | log10_K_ow | Ratio of the equilibrium concentration of a chemical in octanol to that in water | unitless (m3 m-3 in normal scale) | Organic |
| Mass of root per unit area of soil at harvest | m_root_harvest | The maximal root mass per unit ground surface area (m2) at harvest time | kgfw m-2 | Organic and Metal |
| Universal gas constant | R | - | Pa m3 mol-1 K-1 | Organic |
| Surface area of field | S_field | Surface area of cultivated field for the plant of interest | m2 | Organic and Metal |
| Date of germination of a root crop | t_germ_root | Difference between the germination date and the harvest date represents the growth period of a root crop | d | Organic and Metal |
| Date of harvest of a root crop | t_harv_root |  | d | Organic and Metal |
| Transfer factor from soil to root | TF_soil_root | The substance-specific equilibrium factor. The factor is defined as the concentration in root divided by the concentration in soil and thus is empirically derived. | kgdw kgdw-1 | Metal |
| Water content of root | Theta_root | Water volume contained in root per fresh weight of root | L kgfw-1 | Organic |

## Intermediate State variables

An ‘Intermediate State variable’ is defined as a dependent variable calculated within the model. Some State variables are fixed during a model run or simulation because they are calculated only from parameters. Some others are time-dependent because they are calculated not only from parameters but also from time-dependent forcing variables. We distinguish ‘Intermediate State variables’ and ‘Regulatory State variables’. The first ones are generally not used by decision-makers for regulatory purposes but can be used as performance indicators of the model that change over the simulation. The second ones can be used by decision-makers for regulatory purposes.

The state variables for which no forcing variables are required are constant all over the calculation time, while the state variables for which forcing variables are required are time-dependent. They are grouped by the following four categories:

- Parameters related to partition between phases (relevant only for organic substances);

- ‘Xylem influx’: Transfer from soil to root due to xylem flow (relevant only for organic substances);

- ‘Xylem outflux’: Transfer from root to ground-above shoot due to xylem flow (relevant only for organic substances);‘

- ‘Uptake of metals’: Transfer from soil to root governed by the equilibrium transfer factor (relevant only for metals).

The intermediate state variables used in the Root model are listed for each category in the following tables. The calculation processes of the intermediate state variables are depicted by the following three symbols and by arrows:

It should be noted that several state variables such as T_air_Kelvin, K_air_water, Transpiration, and m_root (in abbreviation) are commonly used over different transfer processes.

Table 5 Partition between phases

| State variable n° | Name | Abbreviation and unit | Description | Process followed for calculating the state variable |
| --- | --- | --- | --- | --- |
| 1 | Distribution coefficient of the pollutant between soil and water | Kd_soil (m3 g-1) | Kd_soil is the equilibrium partitioning between concentrations in soil pore water (mg m-3) and in soil solids (mg g-1). |  |
| 2 | Temperature of air (in Kelvin) | T_air_Kelvin (K) | T_air_Kelvin is an expression of temperature of air in Kelvin unit. |  |
| 3 | Partition coefficient between air and water | K_air_water (m3 m-3) | K_air_water is the equilibrium partitioning between concentrations in air and in water. It is an expression of Henry’s law constant in terms of a dimensionless ratio concentration. |  |
| 4 | Partition coefficient between root and water | K_root_water (L kgfw-1) | K_root_water is the equilibrium partitioning between concentrations in roots (mg kg fw-1) and in water or xylem sap (mg L-1). K_root_water considers the sorption to root lipids (estimated by L_root, delta_density_OW, log10_K_ow, and delta_solubility_lipids_root), the dissolution into the aqueous solution of root cells (given by Theta_root), and the partition to the gas phase of root (estimated by G_root and K_air_water). |  |

Table 6 Transfer from soil to root due to xylem flow (‘xylem influx’)

| State variable n° | Name | Abbreviation and unit | Description | Process followed for calculating the state variable |
| --- | --- | --- | --- | --- |
| 5 | Leaf area index for root crops | LAI_root (unitless or m2 leaf.m-2 soil) | LAI_root is a dimensionless quantity that characterizes plant canopies. It is defined as the one-sided green leaf area per unit ground surface area for root crops. The canopy area is assumed to grow linearly over the growing period of root. |  |
| 6 | Transpiration stream | Transpiration (m3 m-2 d-1) | Transpiration is the process of water movement through a plant and its evaporation from aerial parts such as from leaves. It is defined as a function of the actual evapotranspiration (ET_a), Leaf Area Index for root crops (LAI_root) and the extinction factor for partitioning of evapotranspiration between soil evaporation and plant transpiration (alpha_extinction). |  |
| 7 | Xylem influx | Xylem_influx (mg d-1) | Xylem_influx represents the transfer from soil to root by xylem influx (transpiration stream). It is estimated from Transpiration, C_soil, Kd_soil, and the surface area of field (S_field). |  |

Table 7 Transfer from root to above-ground shoot due to xylem flow (‘Xylem outflux’)

| State variable n° | Name | Abbreviation and unit | Description | Process followed for calculating the state variable |
| --- | --- | --- | --- | --- |
| 6 | Transpiration stream | Transpiration (m3 m-2 d-1) | Described in Table 6 | Described in Table 6 |
| 8 | Mass of root per unit area of soil | m_root (kg.fw m-2) | The mass of root is assumed to grow linearly over the growing period of root. |  |
| 9 | Xylem outflux | Xylem_outflux (d-1) | Xylem_ outflux represents the loss rate from root to above-ground shoot due to xylem outflux. |  |

Table 8 Transfer from soil to root governed by the equilibrium transfer factor (‘Uptake of metals’)

| State variable n° | Name | Abbreviation and unit | Description | Process followed for calculating the state variable |
| --- | --- | --- | --- | --- |
| 10 | Uptake of metals | Uptake_metals (mg d-1) | Uptake_metals is the transfer rate of metals from soil to root. It is estimated from TF_soil_root, C_soil, m_root_harvest, Theta_root, the growing period of root, and the surface area of field (S_field). |  |

## Regulatory State variables

An ‘Regulatory State variable’ is defined as a dependent variable calculated within the model. It is generally time-dependent because it is calculated from parameters, but also from time-dependent forcing variables and loadings. We distinguish ‘Intermediate State variables’ and ‘Regulatory State variables’. The first ones are generally not used by decision-makers for regulatory purposes but can be used as performance indicators of the model that change over the simulation. The second ones can be used by decision-makers for regulatory purposes.

The concentration of a target pollutant in the root compartment at harvest time (C_root) is defined as a regulatory state variable. The regulatory state variable is presented in the following table together with other state variables used to calculate it. Figure 5 and Figure 6 summarize the calculation processes for organic substances and metals.

Table 9 Regulatory variable for the Root model

| State variable n° | Name | Abbreviation and unit | Description |
| --- | --- | --- | --- |
| 11 | Quantity in root | Q_root (mg) | Total quantity of the pollutant in root. |
| 12 | Quantity in root at harvest | Q_root_harvest (mg) | Q_root_harvest is the total quantity of the pollutant in root at harvest time. |
| 13 | Concentration in root at harvest | C_root (mg kgfw-1) | C_root is the concentration of the pollutant in root at harvest time. It is calculated from the total quantity of the pollutant in root at harvest time (Q_root_harvest), the mass of root at harvest (m_root_harvest), and the surface area of field (S_field). |

Figure 5 Calculation process for C_root (for organic substances)

Figure 6 Calculation process for C_Root (for metals)

End of Level 1 documentation (basic end-user)

# Level 2 documentation (background science)

# Processes and assumptions

It should be noted that this model was mainly based on the model proposed by Legind and Trapp (2010). Therefore, many assumptions adopted in the model were a priori selected in Legind and Trapp (2010).

## Process n°1: Partition between phases

Motivation

The exchanges of organic compounds between different media can be expressed by equilibrium partition coefficients. In the Root model, following inter-media exchanges are taken into consideration:

- Exchange of contaminants between soil and water

- Exchange of contaminants between root and water

Selected model and assumptions

A chemical equilibrium between two media is expressed by a constant value. The constant is called the partition coefficient, which is given as the concentration ratio between two media. The present model takes into account the following partition coefficients:

- Kd_soil: It is expressed as the concentration ratio between the particulate phase (mg kg-1) and the dissolved phase (mg m-3). For organic pollutants, Kd_soil is derived from the water-organic carbon partition coefficient (K_oc) and the fraction of organic matter in soil particles (f_OM_soil);

- K_root_water: It is expressed as the concentration ratio between roots (mg kg-1 fresh weight) and water (mg L-1). K_root_water considers the sorption to root lipids, the dissolution into the aqueous solution of root cells, and the partition to the gas phase of root;

- K_air_water: It is expressed as the concentration ratio between air and water. For organic pollutants, it is an expression of Henry’s law constant in terms of a dimensionless ratio concentration.

The partition (distribution) of pollutants between two phases is relevant for their transfers into other media (compartments). Each partition coefficient is associated to the following transfer process:

- Transfer of pollutants from soil to root by xylem flow: Kd_soil;

- Transfer of pollutants from root to above-ground shoot by xylem flow: K_root_water.

Alternatives and limits

When equilibrium condition between two phases is not respected (especially for Kd_soil, e.g. just after a direct application), the model must then be considered with caution.

## Process n°2: Xylem influx

Motivation

Xylem flow driven by plant transpiration is a key process in the transfer of pollutants from soil to root with the water taken up.

Selected model and assumptions

In the present model, the uptake of chemical from soil to root is governed by the transpiration stream (Transpiration) and the chemical distribution between soil and water (Kd_soil).

The partition of actual evapotranspiration (ET_a) between soil evaporation and plant transpiration is governed by the solar radiation that can actually reach the soil and that can be spent as latent energy to evaporate water from soil surface. The fraction of solar radiation that can reach soil decreases with the increase of leaf area. Sau et al (2004) and Francisco et al (2008) present models which partition potential evapotranspiration (ET_p) to potential soil evaporation and potential plant transpiration, based on the Ritchie approach (1972, 1985). The model for estimating potential plant transpiration requires as parameters ET_p, the time-dependent Leaf Area Index and the extinction factor. The extinction factor is the coefficient of the canopy for total solar irradiance and functions for partitioning evapotranspiration between soil evaporation and plant transpiration. The time dependent LAI is calculated under the assumption in which aerial plant biomass linearly increases with time until harvest. The present model takes the approach above to estimate actual transpiration stream by assuming that the relation between ET_a and actual transpiration is identical to that between ET_p and potential transpiration.

Alternatives and limits

Several models such as TGD (EC, 2003), CSOIL (Brand et al., 2007) and Xtrafood (Seuntjes et al., 2006) apply the equilibrium partitioning approach for calculating the distribution of chemicals between soil and root. The approach has been questioned for lipophilic chemicals in some studies (Legind and Trapp 2009; Rikken et al. 2001; Trapp 2002; Trapp and Schwartz 2000). Comparing results from the equilibrium approach and the dynamic root model (Trapp 2002) to measured concentrations of benzo(a)pyrene in root crops shows that the equilibrium approach gives a concentration of benzo(a)pyrene in roots that is three orders of magnitude higher than measured. Furthermore, the estimate from the dynamic model lies in the same order of magnitude as the measured value (Legind and Trapp, 2009). Consequently, the present model takes the approach in which the uptake of chemical from soil to root is based on the transpiration flow rate and the chemical distribution between root and soil.

However, the current root uptake mechanism by xylem flux is modelled for lipophilic neutral chemicals based on Trapp (2002) but not for polar chemicals (usually ionic). For the polar substances, the uptake of chemicals into roots is usually slower than that of water. The application of the current model for polar compounds would then be conservative. To be more realistic in a potential future model, it would be necessary to introduce a correction factor that represents the reduced uptake for polar substances.

Kool et al (2014) describes some alternative models for estimating ET partitioning. These are categorized into mechanistic (M) and empirical (P) approaches, or analytical (A) and numerical (N) models. Kool et al (2014) categorizes the following models using the symbols above: Shuttleworth–Wallace (MA); ENWATBAL (MN); Cupid-DPEVAP (MA); SWEAT (MN); TSEB (MA); FAO dual Kc model(PA); HYDRUS-1D (MN). Among these models, FAO dual Kc model (Allen et al, 1998) is the most common model used to partition ET, as it requires relatively few parameters (e.g. wind speed at 2m height, relative humidity) compared to other models. However, the model is empirical and pre-defined crop factors are not always applicable to sites in different contexts. The approach selected for the present model is empirical and requires less parameters than FAO dual Kc model. Therefore, the approach can be useful when the weather data such as wind speed and humidity are not available. A difficulty in the approach may be to give a right value of the extinction factor. Regarding the parameter, however, Sau et al (2004) recommend testing lower values (about 0.5) in a wide range of crops and environments.

## Process n°3: Xylem outflux

Motivation

Transfer of contaminants from root into above-ground shoot is driven by the translocation of contaminants from roots to storage organs. Translocation estimation involves a good knowledge of transport processes occurring in the xylem. In the Root model, the quantity of contaminants exported to the above-ground shoot is considered as a loss from the model system.

Selected model and assumptions

Outflux of the pollutant from the root into above-ground shoot  is governed by the xylem flow driven by transpiration stream (Transpiration) and also by the chemical distribution between roots (mg kg-1 fresh weight) and water (xylem sap, mg L-1).

The detail description about the transpiration stream is given in the previous section (4.2).

Alternatives and limits

See the previous section (4.2).

## Process n°4: Degradation in root compartment

Motivation

Degradation of the pollutant can be a loss term in root.

Selected model and assumptions

Loss by degradation is governed by a degradation rate. The first order degradation rate represents both biotic and abiotic processes. An example of an abiotic process is photodegradation of dioxins in aboveground plant parts. Biotic processes are metabolism in plants, which for most chemicals is unknown, so the degradation rate constant is an optional input. The individual processes that are responsible for degradation (e.g. biodegradation, photolysis) are not distinguished here but they are added into an aggregated loss rate. Degradation is assumed to follow linear first-order kinetics.

## Process n°5: Uptake of metals

Motivation

The bioaccumulation concepts developed for neutral hydrophobic substances (based in particular on diffusion across the lipid biomembranes) are not relevant for metals. For these latter compounds, other processes can significantly influence the accumulation of trace elements by plants like sequestration, detoxification, storage, regulation, etc. The accumulation of metals by plants (and in particular roots) is generally described by soil-to-plant transfer factors at equilibrium.

Description and selected assumptions

The soil-root transfer is governed by a substance-specific equilibrium transfer factor (TF_soil_root).  The factor is defined as the concentration in root (mg kgdw-1) divided by the total concentration in soil (mg kgdw-1) and thus is empirically derived.

Alternatives and limits

CLEA model (Jeffries and Martin, 2008) estimates the concentration of metals in plant tissue (potato, shoot, or fruit). The model assumes that the concentration in the root is directly proportional to the concentration in the soil solution. The concentration in edible plant parts are found by multiplication with the fraction of metal in the roots that reaches the root store, potato, shoot or fruit by xylem or phloem flow.

Since the transfers to root via atmospheric depositions and diffusion are not relevant for the present model, the equilibrium transfer factor (TF_soil_root) will be a dominant parameter to determine the metal level in root. As the parameter in general entails the large variability, a deterministic calculation using a best estimate of the parameter can be insufficient in terms of risk evaluations of metals.

End of Level 2 documentation (end-user with expertise in process)

# Level 3 documentation (numerical information)

# Numerical default values (deterministic and/or probabilistic)

## Initialization of mass balance in Media

The initial mass in the target media, i.e. the total quantity of root (Q_root (mg) must be a priori set in the model. By default, no contaminant accumulation (Q_root =0)  is assumed at the beginning of the simulation.

## Default parameter values

This section proposes default parameter values. Parameters are grouped by the five categories almost as defined in the section 1.1 but the first category “Site-specific and plant parameters” includes not only the site-specific parameters but also the parameters about physiological characteristics of plant (e.g. G_root, Theta_root, m_root_harvest in abbreviation) and plant growth (e.g. t_germ_root, t_harv_root in abbreviation). Those parameters grouped in the first category are commonly used over different transfer processes. At the end of each part, a recommended best estimate and a PDF are proposed.

In the following tables, PDFs are presented by their types and statistical values as follows:

- Normal distribution: N(Mean, Sd(standard deviation));

- Log-normal distribution: LN(GM(geometric mean), GSD(geometric standard deviation)) or LN(5%, 95%);

- Uniform distribution: U(Min(minimum), Max(maximum));

- Triangular distribution: T(Min(minimum), Max(maximum), Mode).

### 5.2.1 Site-specific and plant parameters

#### 5.2.1.1 Surface area of field (S_field)

Physical/chemical/biological/empirical meaning

The Surface of the field under investigation (S_field) represents the dimensions of the region under evaluation. The relevant spatial scale is governed by the homogeneity of the soil and agronomic system under investigation (e.g. homogeneity in contamination levels, agricultural practices, and meteorological conditions). It is advised to define a field surface that show low variations in its land use and contamination level.

Parameter default value and PDF

As this parameter is purely site-specific, neither default best estimate nor PDF is provided here. Therefore it should be defined by model users.

#### 5.2.1.2 Water/lipid/air contents of root (Theta_root, L_root, G_root)

Physical/chemical/biological/empirical meaning

Water/lipid/air contents in root represent respectively the water volume contained in fresh weight of root, the lipid quantity contained in fresh weight of root, the air volume contained in fresh weight of root.

Description of data source

A literature survey was conducted to collect the data of the parameters. The following table presents the parameter values collected from the literature survey and their sources.

Table 10 Values of Theta_root, L_root, G_root collected from the literature survey

| Abbreviation of parameter | Unit | Value | Remark | Original Source |
| --- | --- | --- | --- | --- |
| G_root | L kgfw-1 | 0.1 | Trapp (2002) presents arbitrary values of the gas fraction in root (set as 0.1 L.L-1) and of the root density (set as 1 kg.fw L-1) . Thus in this report, G_root (L.kg fw-1) was calculated by multiplying two values. | Trapp (2002) |
| Theta_root | L kgfw-1 | GM = 0.87, GSD = 1.05, Min = 0.77, Max= 0.95 (n = 39) | IAEA (2010) reports the water content for root crops. These values apply to the edible part of the plant as harvested. | IAEA (2010) |
|  |  | 0.89 | Trapp (2002) presents the average data for carrots that were obtained from Wang and Jones (1994). The data were measured at the time of harvest. | Trapp (2002), Wang and Jones (1994) |
| L_root | Kg kgfw-1 | 0.025 | Trapp (2002) presents a generic value. | Trapp (2002) |

Parameter default value and PDF

The following table lists, for each parameter, a recommended value selected from the previous table and also presents a probability density function (PDF).

Table 11 Default values of Theta_root, L_root, G_root collected from the literature survey

| Abbreviation of parameter | Unit | Best estimate | PDF | Comment |
| --- | --- | --- | --- | --- |
| G_root | L kgfw-1 | 0.1 |  |  |
| Theta_root | L kgfw-1 | 0.87 | LN(GM = 0.87, GSD = 1.05) | Values from IAEA (2010) were selected because they were derived from a number of samples, and thus they can be regarded as generic (representative) values for whole root crops. |
| L_root | Kg kgfw-1 | 0.025 |  |  |

#### 5.2.1.3 Dates of germination/harvest of a root crop (t_germ_root, t_harv_root)

Physical/chemical/biological/empirical meaning

The dates of germination/harvest of a root crop represent the growing period of the root crop of interest. In the Root model, all the transfer processes take place only during the growing period.

Description of data source

The growing periods of crops of interest can vary significantly depending on the types of crops and on the regions. Therefore the dates of germination/harvest of root crops are regarded as site-specific and they should be chosen by model users.

#### 5.2.1.4 Mass of root per unit area of soil at harvest (m_root_harvest)

Physical/chemical/biological/empirical meaning

Mass of root per unit area of soil at harvest represents the maximal root mass per unit ground surface area (m2) at harvest time.

Description of data source and default value

The following table presents the parameter values collected from the FAOStat freely available on internet (http://faostat.fao.org). The geometric means and ranges (min – max) shown in Table 16 were obtained based on the statistics for France and years 1993 to 2012. FAO gives statistics for several root crops, countries and years and data can then be adjusted for defining parameter scenario- specific values. Values estimated for carrots (and turnips) are proposed as default values in the Root model.

Table 12 Default values of m_root_harvest for different root crops (from FAOStat)

| Root species | GM of m_root_harvest (Kgfw m-2) over the period from 1993 to 2012 in France | Range of m_root_harvest (Kgfw m-2) over the period from 1993 to 2012 in France |
| --- | --- | --- |
| Carrots and turnips | 3.6 | 2.1-4.6 |
| Sugar beet | 7.7 | 6.3-9.7 |

Best estimate (m_root_harvest) = 3.6 (Kgfw m-2)

#### PDF (m_root_harvest) = U(min=2.1 , max=4.6) (Kgfw m-2)

### 5.2.2 Parameters related to the partition between phases

#### 5.2.2.1 Fraction of organic matter in soil (f_OM_soil)

Physical/chemical/biological/empirical meaning

Organic carbon is considered the main sorbing phase in soil for neutral organic compounds. As for non-neutral organics, which typically have a log(Kow) greater than 4, their affinity to organic matter tends to be much stronger than to mineral surfaces and the sorption to these latter can be neglected.  Organic carbon content can be divided between amorphous, soft or new and condensed, old or black carbon. Even if this division in types of sedimentary organic matter can be relevant for non-ionic organics, we considered in the model organic carbon as a unique sorbing phase.

The exchanges of organic contaminants between soil pore water and solid soil are assumed to be at equilibrium and represented by partition coefficients at equilibrium (Kd_soil). Interaction of chemicals with solid soil is assumed to be governed by lipophilic sorption onto organic matter. In the present model, the fraction of organic carbon in soil (f_OC_soil), together with the water-organic carbon partition coefficient (K_oc), is used to calculate the partition coefficient at equilibrium Kd_soil.

Organic Matter content in soils depends on natural backgrounds as well as on anthropogenic activities (land use coverage, wet zones management, etc).

Description of data source

Panagos et al (2013) collected data of soil organic carbon and soil erosion in Europe using the European Environment Information and Observation Network for soil (EIONET-SOIL). Panagos et al (2013) presents a best estimate for each European country. Each participating country provided monitoring data on the basis of a grid of 1 km × 1 km cells and for 30 cm soil depth.

For data at a regional scale, specific national monitoring programs can be used. For example, data were collected in the frame of a French program aiming at mapping soil properties at local scales. These data are freely available and can be found in the website www.gissol.fr. Mean and extreme values can thus be calculated for each French region.

Parameter default value and PDF

Table 13 presents the best estimate value and the standard deviation for each country, based on the proposition made by Panagos et al (2013). The data are assumed to follow normal distributions, which must be truncated at zero to avoid negative values. Three examples of default values extracted from the French program Gissol are provided as well in Table 13.

Table 13  Organic fraction in soils in several European countries (from Panagos, 2013) and French regions (from Gissol)

| Country | Number of cells | Best estimate (%) | Standard deviation (%) |
| --- | --- | --- | --- |
| Austria | 55329 | 2.7 | 3.3 |
| Bulgaria | 14101 | 2.1 | 1.0 |
| Denmark | 42919 | 2.0 | 1.5 |
| Estonia | 13379 | 3.5 | 4.6 |
| Ireland | 1322 | 13.3 | 14.1 |
| Italy | 30521 | 3.1 | 2.3 |
| Netherlands | 29866 | 3.5 | 2.6 |
| Norway | 14249 | 3.2 | 3.1 |
| Poland | 220090 | 2.6 | 4.0 |
| Serbia | 1181 | 2.0 | 1.6 |
| Slovakia | 26959 | 1.3 | 1.1 |
| Switzerland | 105 | 4.5 | 5.6 |
| France - Brittany |  | 2.5 | 1.2 |
| France - Alsace |  | 2.5 | 1.2 |
| France - Provence |  | 1.2 | 0.1 |

#### 5.2.2.2 Henry’s law constant (H)

Physical/chemical/biological/empirical meaning

The Henry’s law constant represents the ratio between the vapour pressure and solubility of a chemical at a constant temperature. It describes the capability of the chemical substance to partition between air and water in a binary system. In the present model, the unit is given by Pa.m3.mol-1. When divided by the gas constant R and by temperature, we obtain the dimensionless Henry’s law constant.

The Henry’s law constant can also be defined in the literature as: KH = ca/pg, where ca is the substance concentration in aqueous phase and pg the partial pressure in the gaseous phase. The relationship between H and KH is given by:  H = pg/xa = ρH2O / (MH2Ox KH), where xa is the molar ratio in the aqueous mixture, ρH2O the water density and MH2O the molar mass of water. Under the terminology ‘Henry’s law constant’, both types of values can be found in the literature.

The Henry’s law constant is dependent on the temperature. Values are generally obtained at 25°C. The Henry’s law constant typically increases with increasing temperature and is calculated according to the Van’t Hoff equation often used to model the temperature dependence.

Description of data source

Because experimental data for the Henry’s law constant H are not available for all chemicals in use, several correlations have been developed relating H to molecular descriptors like bond descriptors. Such correlations (called QSAR models) tend to be developed for specific groups or classes of chemicals and can therefore be adapted for some classes of chemicals only. We present here some of the QSAR models that can be used for estimating H values for a given chemical (Table 14) (available in the ChemProp software).

- Meylan and Howard (1991) developed a QSAR model for the estimation of Henry’s law constants based on 59 bond descriptors. The database for training is based on several chemical classes. The model was fitted on 345 chemicals and validated on a independent dataset of 74 substances.  The 345-chemical data set contains a subset of 120 compounds that have both an experimentally measured Henry’s law constants and measured vapor pressures and water solubilities. The standard deviation of the regression model is 0.46 log units.

- Viswanadhan at al (1999) developed a QSAR model based on solvation free energy to estimate Henry’s law constants. Their method (ALOGS) used an extensive atom classification scheme and was fitted on a database containing 265 molecules with experimentally determined solvation free energies. The model was then tested on 27 molecules not present in the training set. The standard deviation of the regression model is 0.86 log units.

- Abraham et al (1994) developed a model based on 5 molecular descriptors, i.e. the excess molar refraction, the dipolarity/polarizability, the effective hydrogen-bond acidity and basicity, and the McGowan characteristic volume. It was fitted on 408 gaseous compounds and the standard deviation of the regression model is 0.15 log units.

- A model was developed by Kühne et al (2005) to estimate the temperature dependency of Henry's law constant in water for organic compounds from the 2D structure. Air/water partition enthalpies of 456 chemicals from various compound classes were fitted to 46 substructure fragments. Application of  the model together with experimental 25°C data to a set of 462 compounds with 2119 experimental Henry's law constants at temperatures below 20°C yields a standard error of 0.21 logarithmic units. The prediction capability is further evaluated using cross validation and permutation.

- A read-across approach was developed by UFZ (personal communication; method is not published yet; details will not be disclosed before publication). The read-across set covers 2354 compounds. Three models are proposed: (i) a Default model (n. of data=1409; rms=0.66); (ii) a High similarity model (n. of data=1127; rms=0.54); (iii) a Low similarity model (n. of data=2352; rms=1.23). Structurally similar compounds in a reference set are looked up via comparison of atom-centered fragments (ACF). The experimental values of the similar compounds are weighted by their similarity. The final result is a weighted average of different runs.

Other models based on the estimation of both water solubility and vapor pressure are also available.

The QSAR models that are indicated above are based on linear regressions fitted by ordinary least squares. Assuming identical, independent and normally distributed errors, the in a QSAR prediction Log H can be defined as the predictive distribution by the predictive mean  and standard error of predictions :

Where  is the student t-distribution with n-k-1 degrees of freedom, n is the number of data in the training set, k is the number of descriptors in the model (and k+1 is the intercept plus the number of descriptors).

The QSAR models that are indicated above generally provide an estimation of the standard error of predictions  by the Mean Squared error MSE. Since MSE is an expectation value, it is subject to estimation error that could be taken into account. The uncertainty on MSE can be calculated from a Bayesian point of view, assuming that the uncertainty of MSE has a scaled inverse Chi distribution. A re-analysis of raw data used in the training set would however be necessary to calculate this posterior distribution. Therefore, in the present model, the MSE uncertainty is not included and the standard error of predictions  is assumed to be equal to the MSE value provided in each QSAR model description.

Table 14 QSAR and read-across models available for calculating Henry’s law constant

| Source | Descriptors | N. of data in the training set | MSE | - 5th-95th percentile |
| --- | --- | --- | --- | --- |
| Meylan and Howard, 1991 | 59 chemical bonds | 345 (and 74 compounds used in the prediction set) | 0.46 | 0.76 |
| Viswanadhan et al, 2006 | No information | 265 (and 26 compounds used in the prediction set) | 0.86 |  |
| Abraham et al, 1999 | 5 descriptors (excess molar refraction,dipolarity/polarizability, effective hydrogen-bond acidity and basicity, McGowan characteristic volume) | 408 | 0.15 | 0.25 |
| Kühne et al, 2005 | 46 substructure fragments. | 456 | 0.21 | 0.34 |
| UFZ, personal communication | Read-across | 2354 |  |  |

Parameter default value and PDF

According to the approach described above, the default best estimates and PDFs of the parameter are given on a log scaled basis for several key substances in Table 15. All the calculations were computed on the ChemProp software that is freely available on request (http://www.ufz.de/index.php?en=6738). The ChemProp returns a result for each compound separately, by applying the methods in the order listed below. The first valid result is accepted. Default order: 1 - Meylan and Howard; 2 - Viswanadhan et al, 2006; 3 - Abraham et al, 1999. By comparison, the result obtained according to Kühne et al, 2005 and to UFZ (personal communication) is also given. The Meylan method is those that is proposed by default in the MERLIN-Expo model. However, end-users are encouraged to run the ChemProp software with alternative methods (that are not reported comprehensively here) to check the concordance between several approaches and to evaluate the plausibility of estimations (see for example some discrepancies for pesticides in Table 15).

Table 15  Henry’s law constant of selected substances

| Chemical class | Substance | Model | Best estimate – log10H | 5th-95th percentile - log10H | 5th-95th percentile – H (Pa.m3.mol-1) |
| --- | --- | --- | --- | --- | --- |
| PAH | Anthracene | Meylan | 0.71 | -4.6.10-2 ; 1.47 | 0.9;29.8 |
|  |  | UFZ-Read-across | 0.46 |  |  |
|  | Benzo(a)pyrene | Meylan | -1.09 | -1.85 ; -3.2.10-1 | 1.4.10-2 ; 4.7.10-1 |
|  |  | UFZ-Read-across | -0.87 |  |  |
|  | Benzo(b)fluoranthene | Meylan | -1.09 | -1.85 ; -3.2.10-1 | 1.4.10-2 ; 4.7.10-1 |
|  |  | UFZ-Read-across | -1.2 |  |  |
|  | Benzo(k)fluoranthene | Meylan | -1.09 | -1.85 ; -3.2.10-1 | 1.4.10-2 ; 4.7.10-1 |
|  |  | UFZ-Read-across | -1.2 |  |  |
|  | Fluoranthene | Meylan | -7.6.10-2 | -8.4.10-1 ; 6.8.10-1 | 0.15 ; 4.8 |
|  |  | UFZ-Read-across | 4.4.10-2 |  |  |
|  | Naphtalene | Meylan | 1.1 | 3.4.10-1 ; 1.9 | 2.2 ; 73.1 |
|  |  | UFZ-Read-across | 1.27 |  |  |
| PCB | PCB28 | Meylan | 1.23 | 4.7.10-1 ; 2 | 3 ; 98.7 |
|  |  | UFZ-Read-across | 1.32 |  |  |
|  | PCB 52 | Meylan | 1.1 | 3.4.10-1 ; 1.9 | 2.2 ; 73.2 |
|  |  | UFZ-Read-across | 1.27 |  |  |
|  | PCB101 | Meylan | 0.97 | 2.1.10-1 ; 1.7 | 1.6 ; 54.2 |
|  |  | UFZ-Read-across | 1.15 |  |  |
|  | PCB118 | Meylan | 0.97 | 2.1.10-1 ; 1.7 | 1.6 ; 54.2 |
|  |  | UFZ-Read-across | 1.15 |  |  |
|  | PCB138 | Meylan | 0.84 | 8.4.10-2 ; 1.6 | 1.2 ; 40.2 |
|  |  | UFZ-Read-across | 0.8 |  |  |
|  | PCB153 | Meylan | 0.84 | 8.4.10-2 ; 1.6 | 1.2 ; 40.2 |
|  |  | UFZ-Read-across | 0.8 |  |  |
|  | PCB180 | Meylan | 0.71 | -4.6.10-1 ; 1.5 | 0.9 ; 29.8 |
|  |  | UFZ-Read-across | 0.7 |  |  |
| Pesticides | Alachlor | Meylan | -2.65 | -3.4 ; -1.9 | 3.9.10-4 ; 1.3.10-2 |
|  |  | UFZ-Read-across | -2.08 |  |  |
|  | Atrazine | Meylan | -3.35 | -4.1 ; -2.6 | 7.8.10-5 ; 2.6.10-3 |
|  |  | UFZ-Read-across | -3.15 |  |  |
|  | Chlordane | Meylan | 0.85 | 9.4.10-2 ; 1.6 | 1.2 ; 41.1 |
|  |  | UFZ-Read-across | 0.9 |  |  |
|  | Chlorpyrifos | Meylan | -0.6 | -1.4 ; 1.6.10-1 | 4.4.10-2 ; 1.46 |
|  |  | UFZ-Read-across | -0.38 |  |  |
|  | DDT | Meylan | 0.19 | -5.7.10-1 ; 9.5.10-1 | 0.27 ; 9 |
|  |  | UFZ-Read-across | -7.6.10-2 |  |  |
|  | Dieldrin | Meylan | -1.27 | -2 ; -5.1.10-1 | 9.4.10-3 ; 0.31 |
|  |  | UFZ-Read-across | -0.13 |  |  |
|  | Diuron | Meylan | -4.27 | -5 ; -3.5 | 9.4.10-6 ; 3.1.10-4 |
|  |  | UFZ-Read-across | -4.74 |  |  |
|  | Endosulfan | Meylan | -2.04 | -2.8 ; -1.3 | 1.6.10-3 ; 5.3.10-2 |
|  |  | UFZ-Read-across | -0.79 |  |  |
|  | Hexachlorocyclohexane (lindane) | Meylan | 1.41 | 6.5.10-1 ; 2.2 | 4.5 ; 149.4 |
|  |  | UFZ-Read-across | -0.65 |  |  |
|  | Isopruturon | Meylan | -3.7 | -4.5 ; -3 | 3.3.10-5 ; 1.1.10-3 |
|  |  | UFZ-Read-across | -4.62 |  |  |
|  | Malathion | Meylan | -4.07 | -4.8 ; -3.3 | 1.5.10-5 ; 4.9.10-4 |
|  |  | UFZ-Read-across | -2.22 |  |  |
|  | Parathion | Meylan | -1.52 | 2.3 ; -0.77 | 5.2.10-3 ; 0.17 |
|  |  | UFZ-Read-across | -1.51 |  |  |
|  | Pentachlorophenol | Meylan | -1.9 | -2.7 ; -1.1 | 2.2.10-3 ; 7.3.10-2 |
|  |  | UFZ-Read-across | -0.42 |  |  |
| Brominated flame retardants | Pentabromo diphenylether | Meylan | -0.93 | -1.7 ; -1.6.10-1 | 2.1.10-2 ; 0.68 |
|  |  | UFZ-Read-across | -1.6.10-2 |  |  |
|  | Hexabromobiphenyl | Meylan | -7.8.10-1 | -1.5 ; -1.6.10-2 | 2.9.10-2 ; 0.96 |
|  |  | UFZ-Read-across | n.a. |  |  |
| VOCs | Benzene | Meylan | 2.73 | 2 ; 3.5 | 94.3 ; 3121.3 |
|  |  | UFZ-Read-across | 2.75 |  |  |
|  | 1,2-Dichloroethane | Meylan | 3.09 | 2.3 ; 3.9 | 215.9 ; 7150.6 |
|  |  | UFZ-Read-across | 2.00 |  |  |
|  | Dichloromethane | Meylan | 2.96 | 2.2 ; 3.7 | 160.1 ; 5300.8 |
|  |  | UFZ-Read-across | 2.41 |  |  |
|  | Hexachlorobenzene (HCB) | Meylan | 1.95 | 1.2 ; 2.7 | 15.6 ; 518 |
|  |  | UFZ-Read-across | 1.37 |  |  |
|  | Hexachlorobutadiene | Meylan | 3.03 | 2.3 ; 3.8 | 188.1 ; 6227.8 |
|  |  | UFZ-Read-across | 2.63 |  |  |
|  | Pentachlorobenzene | Meylan | 2.08 | 1.3 ; 2.8 | 21.1 ; 698.8 |
|  |  | UFZ-Read-across | 1.77 |  |  |
|  | Trichlorobenzene | Meylan | 2.34 | 1.58 ; 3.1 |  |
|  |  | UFZ-Read-across | 2.18 |  |  |
|  | Trichloromethane (chloroform) | Meylan | 2.51 | 1.75 ; 3.3 | 56.8 ; 1880.8 |
|  |  | UFZ-Read-across | 2.58 |  |  |
| Phthalate | Dibutylphthalate (DBP) | Meylan | -0.9 | -1.67 ; -0.15 | 2.2.10-2 ; 0.72 |
|  |  | UFZ-Read-across | -0.71 |  |  |
|  | Di(2-ethylhexyl)phthalate (DEHP) | Meylan | 7.4.10-2 | -0.69 ; 0.83 | 0.2 ; 6.8 |
|  |  | UFZ-Read-across | -0.31 |  |  |
| Dioxins | 2,3,7,8-TCDD | Meylan | -0.44 | -1.21 ; 0.31 | 6.2.10-2 ; 2.06 |
|  |  | UFZ-Read-across | 0.1 |  |  |
|  | 1,2,3,7,8-PeCDD | Meylan | -0.58 | -1.34 ; 0.18 | 4.6.10-2 ; 1.53 |
|  |  | UFZ-Read-across | 0.18 |  |  |
|  | 1,2,3,7,8-HxCDD | Meylan | -0.7 | -1.47 ; 5.4.10-2 | 3.4.10-2 ; 1.13 |
|  |  | UFZ-Read-across | -0.37 |  |  |
| Phenols - Alkylphenols | 2,4,6-tri-tert-butylphenol | Meylan | -5.7.10-3 | -0.77 ; 0.75 | 0.17 ; 5.68 |
|  |  | UFZ-Read-across | n.a. |  |  |
|  | Nonylphenol | Meylan | -0.22 | -0.98 ; 0.54 | 0.11 ; 3.5 |
|  |  | UFZ-Read-across | -0.41 |  |  |
|  | 2-Octylphenol | Meylan | -0.35 | -1.11 ; 0.41 | 7.8.10-2 ; 2.6 |
|  |  | UFZ-Read-across | 1.31 |  |  |

#### 5.2.2.3 Octanol-water partition coefficient in log 10 (log10_K_ow)

Physical/chemical/biological/empirical meaning

The partition coefficient of a substance between water and a lipophilic solvent (n-octanol) characterizes the equilibrium distribution of the chemical between the two phases. The partition coefficient between water and n-octanol (Kow) is defined as the ratio of the equilibrium concentrations of a chemical in octanol saturated with water and water saturated with octanol. The parameter characterizes the equilibrium distribution of the chemical between the two phases and hence represents the degree to which a chemical prefers organic material to water.

In accordance with Nernst’s law K_ow is independent from compound absolute concentration in n-octanol and water. K_ow is usually weakly dependent on temperature, and experiments to measure K_ow values are set to standard conditions i.e. 25°C and 1 atm (OECD protocol). Large organic compounds or highly nonpolar compounds exhibit a high degree of hydrophobicity. Such hydrophobic substances tend to partition to octanol rather than to water and hence have large values of Kow. In contrast, smaller or polar substances are less hydrophobic and have smaller values of Kow. The range of Kow is many orders of magnitude for variety of organic compounds.

Literature value with log K_ow > 6 may contain significant error and thus should be carefully checked.

Kow is not an accurate determinant of lipophilicity for ionizable compounds because it only correctly describes the partition coefficient of neutral (uncharged) molecules. For example, the parameter is not a good predictor of drug behaviours in the changing pH environments of the body because the majority of them are ionizable. The partition coefficient between root and water (K_root_water) is governed by the lipid content of the plant organs. For calculating these partition coefficients, empirical relationships are generally proposed, using the Octanol-water partition coefficient as a surrogate for Lipid-water partition.

Kow is not an accurate determinant of lipophilicity for ionizable compounds because it only correctly describes the partition coefficient of neutral (uncharged) molecules. For example, the parameter is not a good predictor of drug behaviors in the changing pH environments of the body because the majority of them are ionizable.

Description of data source

Experimental protocols to measure Kow are well established and standardised, for example, in OECD guidelines (OECD, 2006; OECD, 2004; OECD 1995). When the parameter is measured following such well-established experimental protocols, it can be assumed that the obtained values of Kow are not likely to be very variable. The measurement values of Kow are available for many of well-recognized toxic substances. Therefore, in this document, measurement data were selected in preference for the determination of default Kow values.

The measurement data were obtained from OECD (2004) and from the software called EPI-Suite developed by US-EPA (http://www.epa.gov/opptintr/exposure/pubs/episuite.htm). The measurement data in EPI-Suite were initially entered by junior and senior scientists using many sources for which the data had already been carefully evaluated, and then were checked by senior scientists.  The QSAR approach used in the EPI-Suite (i.e. “KOWWIN”) was applied to estimate Kow in the case where no measurement was available. The approach uses a "fragment constant" methodology to predict Kow.  In the "fragment constant" method, a structure is divided into fragments (atom or larger functional groups) and coefficient values of each fragment or group are summed together to yield the log Kow estimate. KOWWIN’s methodology is known as an Atom/Fragment Contribution (AFC) method.  Coefficients for individual fragments and groups were derived by multiple regression. Meylan and Howard (1995) presents a more complete description of KOWWIN’s methodology. Analysis of applicability domain of KOWWIN model (Nikolova and Jaworska, 2005) was based on training set of 2434 compounds (r2 = 0,981 and RMSE (root mean squared error) = 0,22) revealed 186 different fragments and 322 different correction factors, resulting in a 508-dimensional descriptor space. The validation set consisted of 10 910 substances, in which the log Kow values vary between -4.99 and 11.71.

Assuming identical, independent and normally distributed errors, the uncertainty in a QSAR prediction log Kow can be defined as the predictive distribution by the predictive mean log Kow and standard error of predictions K_ow:

log K_owp~log K_owp+tn-k-1.SE (log K_owp)

Where  is the student t-distribution with n-k-1 degrees of freedom, n is the number of data in the training set, k is the number of descriptors in the model (and k+1 is the intercept plus the number of descriptors).

The QSAR model used here provides an estimation of the standard error of predictions SE (log K_owp) by the Mean Squared error (MSE).

Parameter default value and PDF

Table 16 presents the values of Kow for well-recognized toxic substances with experimental data (when available), simulated data generated by KOWWIN and uncertainty derived from the approach presented above. Experimental references sited by EPI-Suite are all documented in the software (freely downloadable from the link shown above).

Table 16  Kow of selected substances

| Chemical class | Substance | Log Kow experimental estimate (ref in appendix 1) | Log Kow KOWWIN estimate | 5th-95th percentile (KOWWIN) |
| --- | --- | --- | --- | --- |
| PAH | Anthracene | 4.45 | 4.35 | 3.99 - 4.71 |
|  | Benzo(a)pyrene | 6.13 | 6.11 | 5.75 - 6.47 |
|  | Benzo(b)fluoranthene | 5.78 | 6.11 | 5.75 - 6.47 |
|  | Benzo(k)fluoranthene | 6.11 | 6.11 | 5.75 - 6.47 |
|  | Fluoranthene | 5.16 | 4.93 | 4.57 - 5.29 |
|  | Naphthalene | 3.30 | 3.17 | 2.81 - 3.53 |
| PCB | PCB28 | 5.62 | 5.69 | 5.33 - 6.05 |
|  | PCB 52 | 6.09 | 6.34 | 5.98 - 6.70 |
|  | PCB101 | 6.80 | 6.98 | 6.62 - 7.34 |
|  | PCB118 | 7.12 | 6.98 | 6.62 - 7.34 |
|  | PCB138 | 7.44 | 7.62 | 7.26 - 7.98 |
|  | PCB153 | 7.75 | 7.62 | 7.26 - 7.98 |
|  | PCB180 | - | 8.27 | 7,91 – 8,63 |
| Pesticides | Alachlor | 3.37 | 3.52 | 3.01 - 3.73 |
|  | Atrazine | 2.61 | 2.82 | 2.46 - 3.18 |
|  | Chlordane | 6.16 | 6.26 | 5.90 - 6.62 |
|  | Chlorpyrifos | 4.96 | 4.66 | 4.30 - 5.02 |
|  | DDT | 6.91 | 6.79 | 6.43 - 7.15 |
|  | Dieldrin | 5.40 | 5.45 | 5.09 - 5.81 |
|  | Diuron | 2.68 | 2.67 | 2.31 - 3.03 |
|  | Endosulfan | 3.83 | 3.50 | 3.14 - 3.86 |
|  | Hexachlorocyclohexane (lindane) | 3.72 | 4.26 | 3.90 - 4.62 |
|  | Isoproturon | 2.87 | 2.84 | 2.48 - 3.20 |
|  | Malathion | 2.36 | 2.29 | 1.93 - 2.65 |
|  | Parathion | 3.83 | 3.73 | 3.37 - 4.09 |
|  | Pentachlorophenol | 5.12 | 4.74 | 4.38 - 5.10 |
| Brominated flame retardants | Pentabromo diphenylether | - | 7.66 | 7,30 – 8,02 |
|  | Hexabromobiphenyl | - | 9.10 | 8,74 – 9,46 |
| VOCs | Benzene | 2.13 | 1.99 | 1.63 - 2.35 |
|  | 1,2-Dichloroethane | 1.48 | 1.83 | 1.47 - 2.19 |
|  | Dichloromethane | 1.25 | 1.34 | 0.98 - 1.70 |
|  | Hexachlorobenzene (HCB) | 5.73 | 5.86 | 5.50 -6.22 |
|  | Hexachlorobutadiene | 4.78 | 4.72 | 4.36 - 5.08 |
|  | Pentachlorobenzene | 5.17 | 5.22 | 4.86 - 5.58 |
|  | Trichlorobenzene | 4.05 | 3.93 | 3.57 - 4.29 |
|  | Trichloromethane (chloroform) | 1.97 | 1.52 | 1.16 - 1.88 |
| Phthalate | Dibutylphthalate (DBP) | 5.53 | 4.61 | 4.25 - 4.97 |
|  | Di(2-ethylhexyl)phthalate (DEHP) | 7.60 | 8.39 | 8.03 - 8.75 |
| Dioxins | 2,3,7,8-TCDD | 6.80 | 6.92 | 6.56 - 7.28 |
|  | 1,2,3,7,8-PeCDD | 6.64 | 7.56 | 7.20 - 7.92 |
|  | 1,2,3,6,8-HxCDD | 7.80 | 8.21 | 7.85 - 8.57 |
| Phenols - Alkylphenols | 2,4,6-tri-tert-butylphenol | 6.06 | 6.39 | 6.03 - 6.75 |
|  | Nonylphenol | 5.76 | 5.99 | 5.63 - 6.35 |
|  | 2-Octylphenol | - | 5.50 | 5,14 – 5,86 |

#### 5.2.2.4 Water-organic carbon partition coefficient in log10 (log10_K_oc)

Physical/chemical/biological/empirical meaning

Organic carbon is assumed to be the main particulate media interacting with hydrophobic chemicals potentially present in soil. The Water-Organic Carbon partition coefficient represents the ratio at equilibrium of the chemical associated to particulate organic matter and present in water respectively.

The exchanges of contaminants between Pore Water and Soil particles are assumed to be at equilibrium and represented by a Partition coefficient at equilibrium Kd_soil_organic. Interactions of chemicals with particles are assumed to be governed by lipophilic sorption onto organic matter. The K_oc parameter, together with the organic fraction in soil, allows calculating the partition coefficient at equilibrium Kd_soil.

The main limitation of the approach described above is that the variability in the soil composition is only described by the content of organic carbon. The validity of this assumption can be disputable especially for ionizable compounds (Ter Laak et al, 2006). In particular, the organic carbon content alone as the descriptor of soil composition is not sufficient to predict the soil–water distribution of chemicals that do not exclusively adsorb to organic matter. Furthermore, the composition of organic carbon itself can vary substantially and influence sorption. Complexation is another process neglected by the model, although it may have significant impact for some compounds.

Description of data source

Because experimental K_oc data are not available for all chemicals in use, numerous correlations have been developed relating K_oc to molecular descriptors like the Octanol-Water partition coefficient K_ow. Such correlations (called QSAR models) tend to be developed for specific groups or classes of chemicals and can therefore be adapted for some classes of chemicals only. We present here some of the QSAR models that can be used for estimating K_oc values for a given chemical (Table 17).

- A classical hydrophobic approach based on Kow and on a decision tree was proposed by Sablić et al (1995, 1996). Log Koc is estimated by a hierarchical decision tree, offering 20 different equations in total. The first equation applies the topological index 1χ, while the other 19 equations correlate log Koc to log Kow. For non-polar compounds, the more precise but also restricted model is the one with 1χ, if it cannot be applied, a more general, less precise equation is used.

- Schüürmann et al (2006) developed another model for non-ionic organic compounds. Literature data of logKoc for 571 organic chemicals (subdivided in 457 for training set and 114 for predictive set) were fitted to 29 parameters. The general form of the model is:

with 3 variable Pi (molecular weight, bond connectivity, molecular E-state), 21 fragment correction factors Fj, 4 structural indicator variables Ik. The data set compounds are neutral organics (except for partial ionization of acids and bases at soil pH) that include the following atom types: C, H, N, O, P, S, F, Cl, and Br. Because the training set contained no organoiodine compounds, I is not included, but it is likely that a simple extension (see below) will provide reasonable estimates for compounds with iodine attached to aliphatic or aromatic carbon. The range confidence is checked for the molecular correction factors by comparing the frequency of these substructures in the molecule to the training set. The compound will be checked versus the original training set compounds by means of second order ACFs (Atom Centered Fragments). The applicability domain is then classified as: (i) In: All ACFs are matching including the number of occurrences; (ii) Borderline in: Either the frequency of at least one substructure of the compound exceeds the range of occurrences in the training set, or one substructure is not in the training set at all; (iii) Borderline out: More than one substructure is not in the training set at all, but all 1st order ACFs are matching; (iv) Out: There is mismatch even with 1st order ACFs.

- Tao et al (1999) developed another model based on logKoc data for 592 organic chemicals (subdivided in 430 for training set and 162 for predictive set) and on 98 parameters (74 fragment constants and 24 structural factors).

- Huuskonen (1999) developed a model based on atom-type electrotopological state indices, involving 12 parameters (connectivity index 1χ, 11 atom-type E-state indices). It was tested on logKoc data for 201 organic pesticides (subdivided in 143 for training set and 58 for predictive set). The general form of the model is:

where Si are the atom-type E-state values..

- Poole and Poole (1999) developed a solvatation-based model to predict logK_oc. After removal of the outliers, the model is under the form:

where V is the McGowan’s characteristic volume, E is the excess molar refraction, A and B(O)  are the solute’s effective hydrogen-bond acidity and hydrogen-bond basicity.

- Franco et al (2008, 2009) developed a QSAR model for ionizable compounds (monovalent organic acids and bases). The classical Kow model is applied here, but the Kow value accounts for the distribution of the chemical between neutral and ionic forms. The neutral and ionic fractions are calculated from the substance pKa and the surrounding pH, according to the Henderson-Hasselbalch relationship:

for acids;

for bases.

Thus, supplying of a valid pKa and a pH is required for running the model. The models do not work for neutral compounds without specification of pKa.

The QSAR models that are indicated above are based on linear regressions fitted by ordinary least squares. Assuming identical, independent and normally distributed errors, the uncertainty in a QSAR prediction Log Koc,p can be defined as the predictive distribution by the predictive mean  and standard error of predictions :

Where  is the student t-distribution with n-k-1 degrees of freedom, n is the number of data in the training set, k is the number of descriptors in the model (and k+1 is the intercept plus the number of descriptors).

The QSAR models that are indicated above generally provide an estimation of the standard error of predictions  by the Mean Squared error MSE. Since MSE is an expectation value, it is subject to estimation error that could be taken into account. The uncertainty on MSE can be calculated from a Bayesian point of view, assuming that the uncertainty of MSE has a scaled inverse Chi distribution. A re-analysis of raw data used in the training set would however be necessary to calculate this posterior distribution. Therefore, in the present model, the MSE uncertainty is not included and the standard error of predictions  is assumed to be equal to the MSE value provided in each QSAR model description.

Table 17  QSAR available for calculating logKoc

| Source | Descriptors | N. of data in the training set | MSE | Applicability domain | - 5th-95th percentile |
| --- | --- | --- | --- | --- | --- |
| Sablić et al (1995, 1996) | 1: topological index 1χ | 81 | 0.264 | Predominantly hydrophobics - 3 to 22 atoms of carbon or halogenes with 1<logKoc<6,5 | 0.44 |
|  | 1: octanol-water partition coefficient Kow | 81 | 0.451 | Predominantly hydrophobics | 0.75 |
|  |  | 390 | 0.557 | Nonhydrophobics with -2<logKow<8 | 0.92 |
|  |  | 54 | 0.401 | Phenols, Anilines, Benzonitriles, Nitrobenzenes with 1<logKow<5 | 0.67 |
|  |  | 216 | 0.425 | Acetanilides, Carbamates, Esters, Phenylureas, Phosphates, Triazines, Triazoles, Uracils with -1<logKow<8 | 0.70 |
|  |  | 36 | 0.388 | Alcohols, Organic acids with -1<logKow<5 | 0.66 |
|  |  | 21 | 0.339 | Acetanilides | 0.59 |
|  |  | 13 | 0.397 | Alcohols | 0.71 |
|  |  | 28 | 0.491 | Amides | 0.84 |
|  |  | 20 | 0.341 | Anilines | 0.59 |
|  |  | 43 | 0.408 | Carbamates | 0.69 |
|  |  | 20 | 0.242 | Dinitroanilines | 0.42 |
|  |  | 25 | 0.463 | Esters | 0.79 |
|  |  | 10 | 0.583 | Nitrobenzenes | 1.08 |
|  |  | 23 | 0.336 | Organic acids | 0.58 |
|  |  | 24 | 0.373 | Phenols, Benzonitriles | 0.64 |
|  |  | 52 | 0.335 | Phenylureas | 0.56 |
|  |  | 41 | 0.452 | Phosphates | 0.76 |
|  |  | 16 | 0.379 | Triazines | 0.67 |
|  |  | 15 | 0.482 | Triazoles | 0.85 |
| Schüürmann et al, 2006 | 29 descriptors : molecular weight, bond connectivity, molecular E-state, 24 fragment corrections representing polar groups, one indicator for nonpolar and weakly polar compounds | 457 (and 114 compounds used in the prediction set) | 0.467 | Neutral organics (except for partial ionization of acids and bases at soil pH) with atom types C, H, N, O, P, S, F, Cl, Br | 0.77 |
| Tao et al, 1999 | 98 descriptors (74 fragment constants and 24 structural factors) | 430 (and 162 compounds used in the prediction set) | 0.366 | Organics with K_oc over 7.65 log-units | 0.60 |
| Huuskonen, 2003 | 12 structural parameters (connectivity index 1χ, 11 atom-type E-state indices) | 143 (and 58 compounds used in the prediction set) | 0.40 | Organic pesticides (with logKoc ranging from 0.42 to 5.31) | 0.66 |
| Poole et al, 1999 | 4 descriptors | 131 | 0.248 |  | 0.41 |
| Franco et al, 2008, 2009 | 3 descriptors or conditions (octanol-water partition coefficient Kow, pKa, pH) | 44 (10 acids, 12 bases, different pH) | 0.32 | Ionizable monovalent organic acids and bases | 0.54 |

Parameter default value and PDF

According to the approach described above, the default best estimates and PDFs for the Log scaled K_oc parameter are given for several key substances in Table 18. All the calculations were computed on the ChemProp software that is freely available on request (http://www.ufz.de/index.php?en=6738). When the Schüürmann’s approach indicates ‘In’ for the applicability domain, it was the preferred approach because it explicitly checks the validity domain. Instead, the Sablic approach was considered. For pesticides, the Huuskonen’s approach was considered because it was specifically developed for this class of chemicals. The grey lines indicate the methods that are proposed by default in the MERLIN-Expo tool. However, end-users are encouraged to run the ChemProp software with alternative methods (that are not reported comprehensively here) to check the concordance between several approaches and to evaluate the plausibility of estimations.

Table 18 Log10 Koc of selected substances

| Chemical class | Substance | Model | Applicability domain | Best estimate | 5th-95th percentile |
| --- | --- | --- | --- | --- | --- |
| PAH | Anthracene | Sablić – Equ. 1 |  | 4.3 | 3.86-4.74 |
|  |  | Schüürmann | In | 4.08 | 3,31-4,85 |
|  | Benzo(a)pyrene | Sablić – Equ. 1 |  | 5.86 | 5,42-6,3 |
|  |  | Schüürmann | In | 5.7 | 4,93-6,47 |
|  | Benzo(b)fluoranthene | Sablić – Equ. 1 |  | 5.86 | 5,42-6,3 |
|  |  | Schüürmann | In | 5.18 | 4,41-5,95 |
|  | Benzo(k)fluoranthene | Sablić – Equ. 1 |  | 5.86 | 5,42-6,3 |
|  |  | Schüürmann | In | 5.18 | 4,41-5,95 |
|  | Fluoranthene | Sablić – Equ. 1 |  | 4.83 | 4,39-5,27 |
|  |  | Schüürmann | In | 4.23 | 3,46-5 |
|  | Naphtalene | Sablić – Equ. 1 |  | 3.28 | 2,84-3,72 |
|  |  | Schüürmann | In | 3.12 | 2,35-3,89 |
| PCB | PCB28 | Sablić – Equ. 1 |  | 4.43 | 3,99-4,87 |
|  |  | Schüürmann | In | 4.26 | 3,49-5,03 |
|  | PCB 52 | Sablić – Equ. 1 |  | 4.64 | 4,2-5,08 |
|  |  | Schüürmann | In | 4.63 | 3,86-5,4 |
|  | PCB101 | Sablić – Equ. 1 |  | 4.85 | 4,41-5,29 |
|  |  | Schüürmann | In | 5.00 | 4,23-5,77 |
|  | PCB118 | Sablić – Equ. 1 |  | 4.85 | 4,41-5,29 |
|  |  | Tao |  | 6.12 | 5,52-6,72 |
|  |  | Poole |  | 5.61 | 5,2-6,02 |
|  | PCB138 | Sablić – Equ. 1 |  | 5.08 | 4,64-5,52 |
|  |  | Schüürmann | In | 5.36 | 4,59-6,13 |
|  | PCB153 | Sablić – Equ. 1 |  | 5.07 | 4,63-5,51 |
|  |  | Schüürmann | Border In | 5.37 | 4,6-6,14 |
|  |  | Tao |  | 6.63 | 6,03-7,23 |
|  |  | Poole |  | 5.88 | 5,47-6,29 |
|  | PCB180 | Sablić – Equ. 1 |  | 5.29 | 4,85-5,73 |
|  |  | Schüürmann | Border In | 5.73 | 4,96-6,5 |
|  |  | Tao |  | 7.14 | 6,54-7,74 |
|  |  | Poole |  | 6.26 | 5,85-6,67 |
| Pesticides | Alachlor | Huuskonen |  | 2.83 | 2,17-3,49 |
|  | Atrazine |  |  | 2.44 | 1,78-3,1 |
|  | Chlordane |  |  | 5.15 | 4,49-5,81 |
|  | Chlorpyrifos |  |  | 3.6 | 2,94-4,26 |
|  | DDT |  |  | 4.72 | 4,06-5,38 |
|  | Dieldrin |  |  | 4.49 | 3,83-5,15 |
|  | Diuron |  |  | 2.29 | 1,63-2,95 |
|  | Endosulfan |  |  | 4.04 | 3,38-4,7 |
|  | Hexachlorocyclohexane (lindane) |  |  | 3.7 | 3,04-4,36 |
|  | Isopruturon |  |  | 2.05 | 1,39-2,71 |
|  | Malathion |  |  | 2.4 | 1,74-3,06 |
|  | Parathion |  |  | 2.82 | 2,16-3,48 |
|  | Pentachlorophenol |  |  | 3.46 | 2,8-4,12 |
| Brominated flame retardants | Pentabromo diphenylether | Sablić – Equ. 3 |  | 4.74 | 3.82-5,66 |
|  |  | Schüürmann | Border Out | 5.34 | 4,57-6,11 |
|  | Hexabromobiphenyl | Sablić – Equ. 1 |  | 5.08 | 4,64-5,52 |
|  |  | Schüürmann | Border Out | 6.32 | 5,55-7,09 |
| VOCs | Benzene | Sablić – Equ. 1 |  | 2.26 | 1,82-2,7 |
|  |  | Schüürmann | In | 2.18 | 1,41-2,95 |
|  | 1,2-Dichloroethane | Sablić – Equ. 1 |  | 1.70 | 1,26-2,14 |
|  |  | Schüürmann | In | 1.85 | 1,08-2,62 |
|  | Dichloromethane | Sablić – Equ. 1 |  | 1.44 | 1-1,88 |
|  | Hexachlorobenzene (HCB) | Sablić – Equ. 1 |  | 3.54 | 3,1-3,98 |
|  |  | Schüürmann | Border In | 4.37 | 3,6-5,14 |
|  | Hexachlorobutadiene | Sablić – Equ. 1 |  | 3.02 | 2,58-3,46 |
|  | Pentachlorobenzene | Sablić – Equ. 1 |  | 3.32 | 2,88-3,76 |
|  |  | Schüürmann | In | 4.01 | 3,24-4,78 |
|  | Trichlorobenzene | Sablić – Equ. 1 |  | 2.89 | 2,45-3,33 |
|  |  | Schüürmann | In | 3.28 | 2,51-4,05 |
|  | Trichloromethane (chloroform) | Sablić – Equ. 1 |  | 1.6 | 1,16-2,04 |
| Phthalate | Dibutylphthalate (DBP) | Sablić – Equ. 1 | Chemical domain mismatch : at least one substructure not represented | 3.03 | 2,59-3,47 |
|  | Di(2-ethylhexyl)phthalate (DEHP) | Sablić – Equ. 1 |  | 4.54 | 4,1-4,98 |
|  |  | Schüürmann | In | 4.15 | 3,38-4,92 |
| Dioxins | 2,3,7,8-TCDD | Sablić – Equ. 1 |  | 3.91 | 3,47-4,35 |
|  |  | Schüürmann | In | 4.62 | 3,85-5,39 |
|  | 1,2,3,7,8-PeCDD | Sablić – Equ. 1 |  | 4.26 | 3,82-4,7 |
|  |  | Schüürmann | Border Out | 4.98 | 4,21-5,75 |
|  | 1,2,3,6,8-HxCDD | Sablić – Equ. 3 |  | 4.62 | 3,7-5,54 |
| Phenols - Alkylphenols | 2,4,6-tri-tert-butylphenol | Sablić – Equ. 3 | Chemical domain borderline approached : at least one substructure occurrence outside thresholds | 4.35 | 3,43-5,27 |
|  | Nonylphenol | Sablić – Equ. 3 |  | 3.82 | 2,9-4,74 |
|  |  | Schüürmann | Border Out | 3.62 | 2,85-4,39 |
|  | 2-Octylphenol | Sablić – Equ. 3 |  | 3.58 | 2,66-4,5 |
|  |  | Schüürmann | Border Out | 3.46 | 2,69-4,23 |

#### 5.2.2.5 Empirical correction factors

Physical/chemical/biological/empirical meaning

Several empirical correction factors are used in the Root model to correct differences between:

- solubility in octanol and sorption to root lipids (delta_solubility_lipids_root)

- densities of water and of octanol (delta_density_OW)

Description of data source and Parameter default value

A literature survey was conducted to collect the data of the parameters. The following table presents the parameter values collected from the literature survey and their sources. Only single value was found for each parameter and hence the value was chosen as a best estimate.

Table 19 Default values of delta_solubility_lipids_root and delta_density_OW

| Abbreviation of parameter | Unit | Best estimate | Remark | Original Source |
| --- | --- | --- | --- | --- |
| delta_solubility_lipids_root | unitless | 0.77 | Trapp (2002) presents the value 0.77 originated from Briggs et al (1982). | Trapp (2002), Briggs et al (1982) |
| Delta_density_OW | L kg-1 | 1.22 | Equals to 1 divided by octanol density (0.82 kg L-1). | - |

#### 5.2.2.6 Universal gas constant (R)

Physical/chemical/biological/empirical meaning

R is the gas constant in the ideal gas equation, pV = nRT. In the current model, it is used to convert Henry’s law constant into a dimensionless value, which corresponds to the partition coefficient between air and water (K_air_water).

Parameter default value and PDF

R has a value of 8.314 in the unit of Pa m3 mol-1 K-1.

### 5.2.3 Parameters related to the xylem influx

#### 5.2.3.1 Extinction factor for partitioning of evapotranspiration to plant transpiration (alpha_extinction)

Physical/chemical/biological/empirical meaning

alpha_extinction represents the extinction factor for total solar irradiance which functions as partitioning radiant energy between canopy and soil surface, that is, as partitioning evapotranspiration between soil evaporation and plant transpiration.

Description of data source

A literature survey was conducted to collect the data of the parameters. The following table presents the parameter values collected from the literature survey and their sources.

Table 20 Values of alpha_extinction collected from the literature survey

| Abbreviation of parameter | Unit | Value | Remark | Original Source |
| --- | --- | --- | --- | --- |
| alpha_extinction | unitless | 0.5 | The authors recommend, in their conclusions, testing lower values (close to 0.5) in a wide range of crops and environments. | Sau et al (2004) |
|  |  | Mid-value = 0.9, Min = 0.3, Max = 1.5 | Tahiri (2006) mentions that measured values of the extinction coefficient vary from about 0.3 to 1.5 depending on crop species and stand types (external structure of plants). The mid-value was then added to the two values. | Tahiri et al (2006) |

Parameter default value and PDF

A mid value between the value from Sau et al (2004) and the mid-value calculated based on Tahiri et al (2006) was selected as a best estimate because there is no proof to decide which is better. A PDF was proposed according to the range suggested by Tahiri et al (2006).

Best estimate (alpha_extinction) = 0.7 (unitless)

PDF (alpha_extinction) = U(min=0.3 , max=1.5) (kg.dw m-3)

#### 5.2.3.2 Leaf Area Index at harvest for root crops (LAI_root_harvest)

Physical/chemical/biological/empirical meaning

The LAI_harvest_root represents one-sided root area (m2) per unit ground surface area (m2) at harvest time. Leaf Area Index is an important parameter in the Root model, as it influences transpiration rates.

Description of data source

Breuer et al (2003) collected 5 LAI values a root crop. Table 21 summarizes the statistical values of the LAI from this study.

Table 21 Leaf Area Index at harvest (root crops)

| Number of data | Value | Source |
| --- | --- | --- |
| 5 (all the values are for beets) | Mean = 3.8, Sd = 1.0, Min = 2.4, Max = 5.1 | Breuer et al, 2003 * The estimations of GM and GSD were conducted by the document authors |

Parameter default value and PDF

The values obtained from Breuer et al (2003) were used here as default values.

Best estimate (LAI_root_harvest) = 3.8 (unitless)

PDF (LAI_root_harvest) =  T(Min = 2.4, Max = 5.1, Mode = 3.8)  (unitless)

### 5.2.4 Parameters related to the chemical degradation

#### 5.2.4.1 Degradation rate in root (lambda_deg_root)

Physical/chemical/biological/empirical meaning

It represents first order decay rate of pollutants in root. The value varies depends on chemicals of interest.

Description of data source

A literature survey was conducted to collect the data of the parameters. The following table presents the parameter values collected from the literature survey and their sources.

Table 22 Values of lambda_deg_root collected from the literature survey

| Abbreviation of parameter | Unit | Value | Remark | Original Source |
| --- | --- | --- | --- | --- |
| lambda_deg_root | d-1 | 0.39 | Trapp et al (1994) determined the metabolism rate from measured data (Bromacil in soybeans). The measurements consist of data for stems and roots. The average half-life over stems and roots was found to be 1.8 day. Then metabolism rate was determined by the equation, t_1/2 (half life) = ln2 / K_deg (degradation rate). | Trapp et al (1994) |
|  |  | 0.15 | Legind et al (2011) present a plant metabolism rate (applied to root, stem, leaf, and fruit) for methomyl (at 20°C) which corresponds to a half-life of 4.6 days. The value is representative of a range of measured values. | Legind et al (2011) |
|  |  | 0 | Rein et al (2011) present 0 as a generic value for root, stem, leaf, and corn degradation rates. | Rein et al (2011) |

Parameter default value and PDF

The following table lists, for each parameter, a recommended value selected from the previous table and also presents a probability density function (PDF). The null values that are proposed correspond to conservative assumptions because they express no degradation in the plant.

Table 23 Default values of lambda_deg_root

| Abbreviation of parameter | Unit | Best estimate | PDF | Comment |
| --- | --- | --- | --- | --- |
| lambda_deg_root | d-1 | 0 | - | A generic value was selected because the parameter is substance-specific and thus the measured values for methomyl and bromacil can not represent all other organic substances. |

### 5.2.5 Parameters related to the transfer from soil to root governed by the equilibrium transfer factor

#### 5.2.5.1 Transfer factor from soil to root (TF_soil_root)

Physical/chemical/biological/empirical meaning

The transfer factor from soil to plant is the equilibrium ratio of the metal concentration in root to the metal concentration in soil. This parameter is then calculated by:

TFsoil_root=CrootCsoil

where Croot denotes the concentration of metal in the edible part of a root crop (mg kgdw-1) and Csoil denotes the total concentration of metal in soil (mg kgdw-1).

When Croot is obtained on a basis of fresh weight, it is necessary to convert the unit from fresh into dry weight by using the data of dry matter fraction in a root crop of interest.

TF_soil_root may depend on many factors such as soil texture, pH in soil, type of plants, and the experimental conditions. Precise estimation of TF_soil_root based on these factors remains uncertain, due to the high variability observed in experimental data.

Description of data source

A literature survey was conducted to collect the data of TF_soil_fruit. In addition of the literature survey, the database called BAPPET (Base de données sur les teneurs en Eléments Traces métalliques de Plantes Potagères, http://www.developpementdurable. gouv.fr/BAPPET-BAse-de-donnees-sur-les.html) was used to obtain the values. The BAPPET stores many experimental data of metal concentration in plants and in soil and those concentrations allows the calculation of TF_soil_fruit by the equation above. The concentration of metal in a plant is given in the database in function of the metal considered, of factors associated with the plant/soil (e.g. type of plant/soil), of environmental context (e.g. industrial, rural, urban, and artificial), and of experimental conditions (e.g. mode of extraction of metal: total extraction, semi-total extraction, and partial). The BAPPET compiles the data together with the information about such differing factors.

Parameter default value and PDF

Most of values of TF_soil_root were calculated. The data of Croot and Csoil, which are used to calculate the parameter, were obtained from scientific articles.

The generated parameter values were all transformed into log-scaled values (natural logarithm). Assuming that the transformed values follow the normality, a standard deviation (SD) method was applied to detect outliers according to Seo (2006). In the method, the values greater than X + 2SD or the values smaller than X – 2SD were regarded as outliers (X : mean, SD : standard deviation).

After removing the outliers, the Goodness-of-fit test was used with significant level 5% to evaluate the adjustment of distributions on the parameter values. Log-normal distribution was a priori chosen as the form of PDF. When the log-normality was found to be invalid, the Gamma and Weibull distributions were tested. Table 24 presents a best estimate and a PDF of TF_soil_root for each selected metal. WE signifies the Weibull distribution (given by shape (value on the left) and scale (value on the right))

Table 24  Default values of TF_soil_root for selected metals

| Metal | Best estimate | PDF | Number of data |
| --- | --- | --- | --- |
| Al | 2.20E-04 | LN(2.20E-04, 2.10) | 9 |
| As | 3.99E-03 | LN(3.99E-03, 4.23) | 24 |
| B | 4.07 | LN(4.07, 2.43) | 39 |
| Cd | 3.90E-01 | LN(3.90E-01, 2.35) | 99 |
| Cr | 1.10E-02 | LN(1.10E-02, 12.75) | 19 |
| Cu | 1.54E-01 | LN(1.54E-01, 2.84) | 73 |
| Fe | 2.09E-03 | LN(2.09E-03, 4.19) | 16 |
| Mn | 1.30E-01 | WE(9.02E-01, 2.66E-01) | 35 |
| Pb | 1.38E-02 | LN(1.38E-02, 4.81)* | 99 |
| Zn | 2.51E-01 | LN(2.51E-01, 2.22) | 103 |

* P values calculated for three distributions (Log normal, Gamma, and Weibull) are lower than the level of significance (0.05). Hence the distribution evaluated by the highest P value was selected.

End of Level 3 documentation (end-user with expertise in parameterization)

# Level 4 documentation (mathematical information)

# Mass balance equation

The Root model consists of a single compartment for both organic substances and metals. In the compartment, a mass balance, which accounts for loading, loss, and accumulation of the pollutant, is taken into account. Following equations express mass balances in the root compartment. It should be noted that the loss by harvest is not included in the mass balance equations because the loss process is considered only at the end of the plant growing period during which pollutants are accumulated in plant:

- Mass balance equations used for organic substances

- Mass balance equations used for metals

Where

- Qroot (mg) : Total quantity of the pollutant in root

- Xylem_Influx (mg d-1) : Transfer from soil to root by xylem influx

- Xylem_ Outflux (d-1) : Loss rate from root to above-ground shoot by xylem outflux

- loss_deg_root (d-1) : Loss by degradation in root

- Uptake_metals (mg d-1) : Transfer from soil to root by the equilibrium transfer factor

# Calculation of state variables

This chapter presents the equations to calculate the state variables used in the Root model. Those equations are grouped by differing transfer processes shown in the equations (1), (2), and (3). Together with the equations, state variables are depicted by the following three symbols:

## Xylem influx (Xylem_Influx)

Xylem_influx represents the transfer of the pollutant from soil to root, driven by transpiration stream.

(3)

Where

- Xylem_Influx (mg d-1) : Transfer from soil to root by xylem influx

- Transpiration (m3 m-2 d-1) : Transpiration stream

- Kdsoil (m3 g-1) :  Distribution coefficient of the pollutant between soil and water

- Csoil (mg kgdw-1 ) : Concentration of the pollutant in root zone of soil (on dry mass basis)

- Sfield (m2) : Surface area of field

- 1000 is the unit correction factor with the unit of g kg-1

### Transpiration stream (Transpiration)

The plant transpiration can be estimated from actual evapotranspiration as a function of LAI (leaf area index) and an extinction factor:

(4)

Where

- Transpiration (m3 m-2 d-1) : Transpiration stream

- ETa (mm d-1): Actual evapotranspiration

- αextinction (unitless) :  Extinction factor for partitioning of evapotranspiration to plant transpiration

- LAIroot (unitless or m2 leaf.m-2 soil): Leaf area index for root crops

- 0.001 is the unit correction factor with the unit of m mm-1

### Leaf area index for root crops (LAIroot)

LAI is defined as the one-sided green leaf area per unit ground surface area for root crops. The canopy area is assumed to grow linearly over the growing period of root:

(5)

Where

- LAIroot (unitless or m2 leaf.m-2 soil): Leaf area index for root crops

- tharv_root (d) : Date of harvest of a root crop

- tgerm_root (d) : Date of germination of a root crop

- LAIroot_harvest (m2 m-2) : Leaf area index at harvest for root crops

- t (d) : The current day of the year

### Distribution coefficient of the pollutant between soil and water (Kd_soil)

Kd_soil represents equilibrium partitioning between concentrations in soil pore water (mg m-3) and in soil solids (mg g-1). It is derived from the fraction of organic matter in soil and the partition coefficient between water and organic carbon:

(6)

Where

- Kdsoil (m3 g-1) :  Distribution coefficient of the pollutant between soil and water

- fOM_soil (g g-1) : Fraction of organic matter in soil

- logKoc (unitless) : Water-organic carbon partition coefficient (in log10)

- has the unit of L kg-1

- 10-6 is the unit correction factor with the unit of m3 L-1. kg g-1

## Xylem outflux (Xylem_ Outflux)

Xylem_ outflux represents the loss rate from root to root due to xylem and phloem flows (d-1):

(7)

Where

- Transpiration (m3 m-2 d-1) : Transpiration stream

- Kroot_water (L kgfw-1) : Partition coefficient between root and water

- mroot (kgfw m-2) : Mass of root per unit area of soil

- 0.001 is the unit correction factor with the unit of m3 L-1

### Mass of root per unit area of soil (m_root)

The mass of root is assumed to grow linearly over the growing period of root:

(8)

Where

- mroot (kgfw m-2): Mass of root per unit area of soil

- mroot_harvest (kgfw m-2) : Mass of root per unit area of soil at harvest

- tharv_root (d) : Date of harvest of a root crop

- tgerm_root (d) : Date of germination of a root crop

### Partition coefficient between root and water (K_root_water)

K_root_water is the equilibrium partitioning between concentrations in roots (mg kg fw-1) and in water or xylem sap (mg L-1). K_root_water considers the sorption to root, the dissolution into the aqueous solution of root cells, and the partition to the gas phase of root.

(9)

Where

- Kroot_water (L kgfw-1) : Partition coefficient between root and water

- θroot (L kgfw-1) : Water content of root

- Lroot (kg kgfw-1) : Lipid content of root

- δdensity_OW (L kg-1) : Correction for density difference between water and n-octanol

- logKow (m3 m-3) : Octanol-water partition coefficient (in log 10)

- δsolubility_lipids_root (unitless) : Empirical correction factor for differences between solubility in octanol and sorption to root lipids

- Groot (L kgfw-1) : Air content of root

- Kair_water (m3 m-3) : Partition coefficient between air and water

- has the unit of m3 m-3

### Partition coefficient between air and water (K_air_water)

K_air_water is the equilibrium partitioning between concentrations in air and in water. It is an expression of Henry’s law constant in terms of a dimensionless ratio concentration:

(10)

Where

- Kair_water (m3 m-3) : Partition coefficient between air and water

- H (Pa m3 mol-1) : Henry’s law constant

- R (Pa m3 mol-1 K-1) : Universal gas constant

- Tair_Kelvin (K) : Temperature of air (in Kelvin)

(11)

Where

- Tair_Kelvin (K) : Temperature of air (in Kelvin)

- Tair (°C) : Temperature of air (in Celsius)

## Loss by degradation in root (Loss_deg_root)

The degradation of the pollutant in root is expressed by the first order decay rate (d-1):

- λdeg_root (d-1) : Degradation rate in root (See equation (1))

## Uptake of metals (Uptake_metals)

The uptake of metals from soil into roots is governed by a substance-specific equilibrium transfer factor:

(12)

Where

- Uptakemetals (mg d-1) : Uptake of metals

- TFsoil_root (kgdw kgdw-1) : Transfer factor from soil to root

- ϴroot (L kgfw-1) : Water content of root

- tharv_root (d) : Date of harvest of a root crop

- tgerm_root (d) : Date of germination of a root crop

- mroot_harvest (kgfw m-2) : Mass of root per unit area of soil at harvest

- Csoil (mg kgdw-1 ) : Concentration in soil (on dry mass basis)

- Sfield (m2) : Surface area of field

## Concentration in root at harvest (C_root)

The concentration of the pollutant in root at harvest time (C_root) is calculated as follows:

(13)

Where

- Croot (mg kgfw-1) : Concentration in root at harvest

- Qroot_harvest (mg) : Quantity in root at harvest, which is derived from Qroot

# Reference

Abraham MH, Andonian-Haftvan J, Whiting GS, Leo A, Taft RS (1994) Hydrogen bonding. Part 34. The factors that influence the solubility of gases and vapors in water at 298 K, and a new method for its determination. J. Chem. Soc. Perkin Trans. 2 1777-1791.

Allen RG, Pereira LS, Raes D, Smith M (1998) Crop evapotranspiration – Guidelines for computing crop water requirements – FAO Irrigation and drainage paper 56. FAO – Food and Agriculture Organization of the United Nations, Rome.

Baes CF, Sharp RD (1983) A proposal for estimation of Soil Leaching and Leaching Constants for Use in Assessment Models. Journal of Environment Quality 12 (1): 17-28.

Brand E, Otte PF, Lijzen JPA (2007) CSOIL 2000: an exposure model for human risk assessment of soil contamination. RIVM Report 711701054/2007. Bilthoven, NL, National Institute of Public Health and the Environment.

Breuer L, Eckhardt K, Frede HG (2003) Plant parameter values for models in temperate climates. Ecological Modelling 169: 237-293.

Briggs GG, Bromilow RH, Evans AA (1982) Relationships between lipophilicity and root uptake and translocation of non-ionised chemicals by barleya. Pestic. Sci. 13: 495-504.

Briggs GG, Bromilow RH, Evans AA, Williams M (1983) Relationships between lipophilicity and the distribution of non-ionised chemicals in barley shoots following uptake by the roots. Pestic. Sci. 14: 492-500.

Bromilow RH, Chamberlain K (1995) Principles governing uptake and transport of chemicals. In S. Trapp and JC McFarlane (Eds.), Plant contamination. Modeling and simulation of organic chemical processes: Florida: CRC Press.

Franco A, Fu W, Trapp S (2009) Influence of soil pH on the sorption of ionizable chemicals: Modeling advances. Environ. Toxicol. Chem. 28: 458-464.

Franco A, Trapp S (2008) Estimation of the soil-water partition coefficient normalized to organic carbon for ionizable organic chemicals. Environ. Toxicol. Chem. 27: 1995-2004.

Huuskonen J (2003) Prediction of soil sorption coefficient of organic pesticides from the atom-type electrotopological state indices. Environ. Toxicol. Chem. 22: 816-820.

IAEA (2010) Handbook of parameter values for the prediction of radionuclide transfer in terrestrial and freshwater environments. Technical Reports Series No.472.

Kühne R, Ebert R-U, Schüürmann G (2005) Prediction of the temperature dependency of Henry's Law constant from chemical structure. Environ. Sci. Technol. 39: 6705-6711.

Legind CN, Trapp S (2009) Modeling the exposure of children and adults via diet to chemicals in the environment with crop-specific models. Environ. Pollut. 157: 778-785. Legind CN, Trapp S (2009) Modeling the exposure of children and adults via diet to chemicals in the environment with crop-specific models. Environmental Pollution 157, Supporting information.

Legind CN, Kennedy CM, Rain A, Snyder N, Trapp S (2011) Dynamic plant uptake model applied for drip irrigation of an insecticide to pepper fruit plants. Pest. Manag. Sci. 67: 521-527.

Legind CN, Trapp S (2010) Review of features, events and processes incorporated in existing models for chemical uptake into plants and animals – Proposal of the conceptual and mathematical 2-FUN model for assessing transfer of chemicals to plants and animals. Written as Deliverable 2.4 in the framework of the 2-FUN project (FP6 Project-2005-Global-4 Integrated Project - Contract n°: 036976).

Meylan WM, Howard PH (1991) Bond contrlbution method for estimating Henry’s law constants. Environmental Toxicology and Chemistry 10: 283-1293. Meylan WM, Howard PH (1995) Atom/fragment contribution method for estimating octanol-water partition coefficients. J. Pharm. Sci. 84: 83-92. Nikolova N, Jaworska J (2005) An approach to determining applicability domains for QSAR group contribution models: An analysis of SRC KOWWIN. ATLA 33: 461-470. OECD (1995) OECD guidelines for the testing of chemicals - Partition Coefficient (n-Octanol/Water): Shake Flask Method. OECD (2004) OECD guidelines for the testing of chemicals - Partition Coefficient (n-Octanol/Water): High Performance Liquid Chromatography (HPLC) Method. OECD (2006) OECD guidelines for the testing of chemicals - Partition Coefficient (1-Octanol/Water): Slow-Stirring Method.

Panagos P, Hiederer R, Van Liedekerke M, Bampa F (2013) Estimating soil organic carbon in Europe based on data collected through an European network. Ecological Indicators 24: 439–450.

Rein A, Legind CN, Trapp S (2011) New concepts for dynamic plant uptake models. SAR and QSAR in Environmental Research 22: 191-215.

Rikken MGJ, Lijzen JPA, Cornelese AA (2001) Evaluation of model concepts on human exposure. RIVM Report 711701022/2007. Bilthoven, NL, National Institute of Public Health and the Environment.

Ritchie JT (1972) Model for predicting evapotranspiration from a row crop with incomplete cover. Water Resour. Res. 8: 1204–1213.

Ritchie JT (1985) A user-oriented model of the soil water balance in wheat. p. 293–305. In W Day and RK Atkin (ed.) Wheat growth and modelling. NATO-ASI Science, New York. Sablic A, Güsten H, Verhaar H, Hermens J (1995) QSAR modelling of soil sorption. Improvements and systematics of log KOC vs. log KOW correlations. Chemosphere 31: 4489- 4514, Corrigendum: Vol. 33 (1996): p2577.

Sau F, Boote KJ, Bostick WM, Jones JW, Minguez MI (2004) Testing and Improving Evapotranspiration and Soil Water Balance of the DSSAT Crop Models. Agronomy Journal 96: 1243-1257.

Schüürmann G, Ebert R-U, Kühne R (2006) Prediction of the sorption of organic compounds into soil organic matter from molecular structure. Environ. Sci. Technol. 40: 7005-7011.

Seo S (2006) A Review and Comparison of Methods for Detecting Outliers in Univariate Data Sets. University of Pittsburgh, United States.

Seuntjes P, Steurbaut W, Vangronsveld J (2006) Chain model for the impact analysis of contaminants in primary food products. D/2006/1191/29. Brussels, Belgian Science Policy.

Tahiri AZ, Anyoji H, Yasuda H (2006) Fixed and variable light extinction coefficients for estimating plant transpiration and soil evaporation under irrigated maize. Agricultural water management 84: 186-192.

Trapp S (2002) Dynamic root uptake model for neutral lipophilic organics. Environmental Toxicology and Chemistry 21: 203-206. Trapp S (2003) Modellering af optagelse aforganiske stoffer i grøntsager ogfrugt, Teknologiudviklingsprogrammet for jord- og grundvandsforurening. Miljøprojekt Nr. 765. Trapp S (2004) Plant uptake and transport models for neutral and ionic chemicals. Environ Sci & Pollut Res 11(1): 33-39.

Trapp S, Matthies M (1995) Generic one-compartment model for uptake of organic-chemicals by foliar vegetation, Environ. Sci. Technol. 29: 2333–2338, Erratum Vol.30: p360.

Trapp S, McFarlane C, Matthies M (1994) Model for uptake of xenobiotics into plants: validation with bromacil experiments. Environmental Toxicology and Chemistry 13: 413-422.

Trapp S, Schwartz S (2000) Proposals to overcome limitations in the EU chemical risk assessment scheme. Chemosphere 41(7): 965-971.

Viswanadhan VN, Ghose AK, Singh UC, Wendoloski JJ (1999) Prediction of solvation free energies of small organic molecules. Additive-constitutive models based on molecular fingerprints and atomic constants. J. Chem. Inform. Comput. Sci. 39: 405-412.

Wang M, Jones KC (1994) Uptake of chlorobenzenes by carrots from spiked and sewage sludgeamended soil. Environ. Sci. Technol. 28:1260-1267.

2Chemical classes in the Meylan’s database with indication of the number of chemicals: Alkanes 16 ; Alkenes 20 ; Alkynes 7 ; Acids, aliphatic 6; Alcohols 18; Aldehydes 17; Esters 27; Ethers 16; Epoxides 2; Ketones 9; Halomethanes 22; Haloethanes 20; Halopropanes 11; Halobutanes 9; Other haloalkanes 4; Haloalcohols 5; Haloalkenes 12; Aliphatic amines 13; Nitriles 5; Other aliphatic nitrogen compounds 11; Aliphatic sulfur compounds 8; Five-member aromatic rings 3; P yridines 12; Benzene and alkylated; benzenes 13; Halogenated benzenes 12; Anilines 3; Phenols 8; Biphenyls 3; Polyaromatics 13; Other aromatics 14; Pesticides 6

The topological index was introduced by Kier et Hall (1990, 1999) following the suggestions of Randic (1975). nχ is a n-order topological index where n represents the number of atoms (except H) linked to each atom (except H) belonging to the molecule.

Defined as molecules containing only C, H and halogen (F, Cl, Br, I)

Defined as all the molecules that contains other atoms than C, H and halogen (F, Cl, Br, I). Does not imply anything about their lipophilicity.