The MERLIN-Expo Leaf model

V1.1

Author: Taku Tanaka1

1EDF R&D, 6 quai Watier, 78400 Chatou, France

taku.tanaka@edf.fr

Reviewers: Philippe Ciffroy1

Content

Level 1 documentation (basic knowledge on model purpose, applicability and components)	5

1.	Model purpose	5

1.1.	Goal	5

1.2.	Potential decision and regulatory framework(s)	5

2.	Model applicability	5

2.1.	Spatial scale and resolution	5

2.2.	Temporal scale and resolution	5

2.3.	Chemical considered	5

2.4.	Steady-state vs dynamic processes	5

3.	Model components	6

3.1.	Media considered	6

3.2.	Loadings, losses, and exchanges between different media	6

3.3.	Coupling with other models	8

3.4.	Forcing variables	9

3.5.	Parameters	11

3.6.	Intermediate State variables	14

3.7.	Regulatory State variables	26

Level 2 documentation (background science)	28

4.	Processes and assumptions	28

4.1.	Process n°1: Partition between phases	28

4.2.	Process n°2: Xylem influx	29

4.3.	Process n°3: Xylem outflux	30

4.4.	Process n°4: Degradation in root and leaf compartments	30

4.5.	Process n°5: Diffusion between leaf and air	31

4.6.	Process n°6: Interception of irrigated water	32

4.7.	Process n°7: Deposition from air to leaf	32

4.8.	Process n°8: Uptake of metals	33

4.9.	Process n°9: Weathering (wash off)	34

Level 3 documentation (numerical information)	35

5.	Numerical default values (deterministic and/or probabilistic)	35

5.1.	Initialization of mass balance in Media	35

5.2.	Default parameter values	35

5.2.1 Site-specific and plant parameters	35

5.2.2 Parameters related to the partition between phases	41

5.2.3 Parameters related to the xylem influx	56

5.2.4 Parameters related to the chemical degradation	58

5.2.5 Parameters related to the diffusion between leaf and air	59

5.2.6 Parameters related to the dry/wet depositions of aerosols intercepted by leaves and related to the interception of irrigated water	61

5.2.7 Parameters related to the transfer from soil to leaf governed by the equilibrium transfer factor	62

5.2.8 Parameters related to the transfer from leaves to soil by weathering	64

Level 4 documentation (mathematical information)	66

6.	Mass balance equation	66

7.	Calculation of state variables	67

7.1	Xylem influx (Xylem_Influx)	67

7.1.1	Transpiration stream (Transpiration)	68

7.1.2	Leaf area index for leaf (LAIleaf)	68

7.1.3	Distribution coefficient of the pollutant between soil and water (Kd_soil)	69

7.2	Xylem outflux (Xylem_ Outflux)	69

7.2.1	Mass of root for a leafy crop per unit area of soil (m_root_leaf)	70

7.2.2	Partition coefficient between root and water (K_root_water)	70

7.2.3	Partition coefficient between air and water (K_air_water)	71

7.3	Loss by degradation in root/leaf (Loss_deg_root/Loss_deg_Leaf)	71

7.4	Diffusion upwards/downwards (Diffusion_upwards/Diffusion_downwards)	72

7.4.1	Mass of leaf per unit area of soil (m_leaf)	72

7.4.2	Partition coefficient between leaf and air (K_leaf_air)	73

7.4.3	Conductance of leaf (g_leaf)	73

7.4.4	Permeability of leaf (P_leaf)	74

7.4.5	Total permeability of the cuticle pathway (P_cuticle)	74

7.4.6	Permeability of the air boundary layer around plant (P_air)	75

7.4.7	Permeability of the cuticle (P_cuticle)	75

7.4.8	Permeability of water inside plant (P_water)	75

7.4.9	Effective diffusion coefficient in water (D_water)	76

7.4.10	Permeability of the stomata (P_stomata)	76

7.4.11	Conductance of the stomatal pathway for the chemical (g_stomata)	77

7.4.12	Conductance of the stomatal pathway for water (g_H2O)	77

7.4.13	Concentration of water vapour in air at saturation (C_H2O_sat)	78

7.4.14	Concentration of water vapour in air at saturation (C_H2O_sat)	78

7.4.15	Partition coefficient between leaf and water (K_leaf_water)	79

7.5	Dry/wet deposition from air to leaf (Depostion_dry_aero_Intercepted/Deposition_wet_aero_Intercepted)	79

7.5.1	Dry/wet interception fraction onto leaves (f_dry_interception_leaf/f_wet_interception_leaf)	80

7.6	Interception of irrigated water (Irrigation_intercepted)	80

7.7	Uptake of metals (Uptake_metals)	81

7.8	Loss from leaf to soil by weathering (Weathering)	82

7.9	Concentration in leaf at harvest (C_leaf)	82

Reference	83

Level 1 documentation (basic knowledge on model purpose, applicability and components)

# Model purpose

## Goal

The Leaf model is used to estimate the time-dependent accumulation (in mass and concentration bases) of organic/metals in the edible part in leaves at harvest (especially for leaf crops like salads, cabbage, spinach, etc).

## Potential decision and regulatory framework(s)

Coupled with the information about the ingestion rate of leafy crops (kg fresh weight d-1), the Leaf model can estimate the human exposure to organic substances/metals through the ingestion of leafy crops. This output can be used for evaluating the risk to exceed regulatory thresholds for human health or used as an input for PBPK models.

# Model applicability

## Spatial scale and resolution

The Leaf model for organic substances consists of two compartments. One is the below-ground compartment representing the root part, and the other is the above-ground compartment representing the leaf part. The model for metals, on the other hand, considers only a leaf compartment. The compartments are fixed in space and are treated as being homogeneous, i.e. well-mixed in chemical composition, that is, the Leaf model considers no spatial variation of chemical concentration in those compartments.

When model users need to consider, in their own scenarios, several regions with different levels of contamination in soil and/or air, they can set up multiple Leaf models and make the models correspond to different levels of contamination in soil and/or air to cover several regions in their scenarios.

## Temporal scale and resolution

All the transfer processes considered in the Leaf model are expressed on a daily basis, and thus the model should be run by the daily simulation setting.

## Chemical considered

The Leaf model can be applicable for a variety of chemicals including many types of organic substances (such as PAHs, PCBs, Dioxins, VOCs) and also metals for which root-to-leaf transfer factors were found in the literature (i.e. Al, As, Cd, Cr, Cu, Fe, Mn, Pb, Zn). However, it should be noted that the model used for organic chemicals is limited to neutral compounds; it is not really applicable to ionic or dissociating compounds. For example, practically all herbicides and all ‘systemic’ pesticides are weak bases (Trapp, 2004). Also many of pharmaceuticals are weak bases or acids.  These electrolytes undergo more complicated processes inside plants than neutral compounds.

## Steady-state vs dynamic processes

The Leaf model is governed by time-dependent (dynamic) transfer processes with the following exceptions:

- Chemical transfer of organic substances from soil to the root zone: the chemical transfer from soil to the root zone is governed by the time-dependent transpiration stream which takes up water and solutes by roots and transports them into leaves, but also by the equilibrium partition coefficient between soil particles and soil pore-water for organic substances (Kd_soil).

- Chemical transfer of organic substances from roots to leaves: the chemical transfer from roots to leaves is governed by the time-dependent transpiration stream (xylem flow) but also by the equilibrium partition coefficient between roots and water (Kroot_water), which is derived from octanol-water partition coefficient (Kow) and air-water partition coefficient (Kair_water).

- Chemical transfer of metals from soil to leaves: the transfer of metals into leaves is represented by equilibrium transfer factors. The transfer factor is expressed by the ratio of the concentration in leaf to the concentration in soil.

# Model components

## Media considered

Definition: A ‘Medium’ is defined as an environmental or human compartment assumed to contain a given quantity of the chemical. The quantity of the chemical in the media is governed by loadings/losses (see 3.2 and 3.3) from/to other media and by transformation processes (e.g. degradation).

For organics, the Leaf model includes the following media (compartment):

- ‘Root’. In this media, a time-dependent chemical mass (mg) in roots is calculated from the mass balance equation.

- ‘Leaf’. In this media, a time-dependent chemical mass (mg) in leaves is calculated from the mass balance equation.

## Loadings, losses, and exchanges between different media

Definition:

- A ‘Loading’ is defined as the rate of release/input of the chemical of interest to the   receiving system, here the root and fruit media;

- A ‘Loss’ is defined as the rate of output of the chemical of interest from the receiving system;

- An ‘Exchange’ is defined as the transfer of the chemical of interest between two media of the system.

In the Leaf model, the transfer processes for organic substances are assumed to be different from those for metals. The loading and loss processes for organic substances are considered in both root and leaf compartments whereas those for metals are modelled only in the leaf compartment (the root compartment is not explicitly considered because the transfer of metals from soil to leaves is represented by equilibrium transfer factors).

Each process is presented in the following table par each type of target substances (organic or metal) and also per each compartment as follows:

Table 1 Loading and loss processes in the Leaf model

| The loading and loss processes in root compartment |  |
| --- | --- |
| For organic substances | For metals |
| Input from soil to root by transpiration stream | No transfer modelled |
| Loss from root to leaf by transpiration stream |  |
| Loss by chemical degradation |  |
| Loss by harvest |  |
| The loading and loss processes in leaf compartment |  |
| For organic substances | For metals |
| Input from root to leaf by transpiration stream | Input from soil to leaf governed by equilibrium transfer factor |
| Input by dry deposition of aerosol in atmosphere | Input by dry deposition of aerosol present in atmosphere |
| Input by wet deposition of aerosol in atmosphere (with rain-water drops during the precipitation event) | Input by wet deposition of aerosol in atmosphere (with rain-water drops during the precipitation event) |
| Input by gaseous diffusion from air to leaf |  |
| Input by interception of irrigated water | Input by interception of irrigated water |
| Loss by gaseous diffusion from leaf to air |  |
| Loss by chemical degradation |  |
| Loss by weathering (by wash-off from leaf to soil) | Loss by weathering (by wash-off from leaf to soil) |
| Loss by harvest | Loss by harvest |

In addition to the loading and loss processes, several inter-phase exchange processes are considered in the Leaf model (for organic substances). The root compartment takes in account the equilibrium partitioning between concentrations in roots (mg kg-1 fresh weight) and in water (xylem sap, mg L-1) and the leaf compartment considers the partitioning between concentrations in leaves (mg kg-1 fresh weight) and in air (mg m-3 air). The equilibrium partitioning between concentrations in soil pore water (mg m-3) and in soil particles (mg kg-1) is used to calculate the fraction of a contaminant dissolved in water contained in the soil compartment, which is taken upward to root by xylem flow.

In the Leaf model, all the transfer processes take place only during the growing period of a leafy crop, that is, only between the germination time and harvest time (the model uses a syntax named “flag_cultivating” to control this). Out of the period, no chemical accumulation arises in the Leaf model. At harvest time, the chemical mass accumulated in root and leaf compartments gets completely erased (set to zero), which is expressed in the model by the process of ‘Loss by harvest’ (see Table 1). After the harvest time, the transfer processes restart from the germination time in the next growing season.

Figure 1 and 2 present all the input/loss/exchanges processes for organic substances and metals, respectively.

Figure 1 Input and loss processes in the Leaf model (for organic substances)

Figure 2 Input and loss processes in the Leaf model (for metals)

## Coupling with other models

The Leaf model can potentially be coupled with other models stored in the Merlin-Expo library. These models can estimate loadings as input data used for the Leaf model.

Table 2 Potential models to be coupled with the Leaf model

| Coupled model | Loadings estimated by coupled models |
| --- | --- |
| Atmosphere model | The atmosphere model estimates the concentration of the organic/metal substance in the atmospheric zone. This concentration is used as an input to calculate: (i) Loading of organic/metals present in the atmosphere by the aerosol dry deposition (ii) Loading of organic/metals present in the atmosphere by the aerosol wet deposition (with rain-water drops during the precipitation event) (iii) Loading/Loss of organic substances by diffusion between contaminant present in leaf air fraction and in atmosphere under gaseous phase |
| Soil model | The soil model estimates the mass of the organic/metal substance accumulated in the surface soil (or root zone soil). This mass of the substance in soil pore water is used as an input to calculate: - The transfer of organic substances from soil to root by transpiration stream. This total mass of the substance in soil is used as an input to calculate: - The transfer of metals from soil to leaf governed by equilibrium transfer factors. |
| River model | The river model estimates the concentration of the organic/metal substance in the raw water. This concentration is used as an input to calculate the transfer of the pollutant to leaves by interception of irrigated water. |

Figure 3 and 4 present the input and loss processes for the Leaf model together with other models relevant for the processes.

Figure 3 Potential linkages with other models (for organic substances)

Figure 4 Potential linkages with other models (for metals)

## Forcing variables

A ‘Forcing variable’ is defined as an external or exogenous (from outside the model framework) factor that influences the state variables calculated within the model. Such variables include, for example, climatic or environmental conditions (temperature, wind flow, etc.).

Some forcing variables are used to calculate the external loadings from air and soil to the Leaf model and also the internal processes within the Leaf model.  They can be given by sets of time-dependent values. The forcing variables can be calculated by other models (the Atmosphere, Soil and River models in this case) or can be given by model users (in the case that the users have measurement data or their own estimations). The following forcing variables are defined in the Leaf model. The column ‘Substance’ allows distinguishing the forcing variables used for organic substances from those for metals.

Table 3 Forcing variables used in the Leaf model

| Name | Substance | Abbreviation and unit | Description | Given by… |
| --- | --- | --- | --- | --- |
| Gaseous concentration in the atmosphere | Organic | C_gas_atm (mg m-3) | It is used to calculate the diffusion from gaseous atmosphere to leaf (Diffusion_downwards). | Atmosphere model or users |
| Concentration in raw water | Organic and Metal | C_water (mg m3) | It is used to calculate the transfer of pollutant to leaves by the interception of irrigated water (Irrigation_ intercepted). | River model or users |
| Surface dry deposition flux of contaminated aerosols | Organic and Metal | Dry_deposition (mg d-1 m-2) | It is used to calculate the dry deposition flux of contaminated aerosols intercepted by leaves (Dry_deposition_intercepted). | Atmosphere model or users |
| Surface wet deposition flux of contaminated aerosols | Organic and Metal | Wet_deposition_aerosol (mg d-1 m-2) | It is used to calculate the wet deposition flux of contaminated aerosols intercepted by leaves (Wet_deposition_aerosol_intercepted) | Atmosphere model or users |
| Concentration in soil | Organic and Metal | C_soil (mg kgdw-1) | It is the concentration of the pollutant in root zone of soil (on dry mass basis). It is used to calculate (i) the transfer of pollutant from soil to root by the transpiration stream (Xylem_influx) and (ii) the metal uptake from soil to leaves (Uptake_metals). | Soil model or Users |
| Actual evapotranspiration | Organic | ET_a (mm d-1) | It is used to calculate the transpiration stream (Transpiration). | Soil model or users |
| Relative humidity | Organic | rh (unitless) | It is used to calculate the conductance of the stomatal pathway for water (g_H2O). | Users |
| Temperature of air | Organic | T_air (°C) | It is used to calculate the saturation vapour pressure of water at a given air temperature (p_water_sat). | Users |
| Irrigation rate of cultivated fields | Organic and Metal | Irrigation_rate (m d-1) | It is used to calculate the transfer of pollutant to leaves by the interception of irrigated water (Irrigation_ intercepted). | Users |

## Parameters

A ‘Parameter’ is defined as a term in the model that is fixed during a model run or simulation but can be changed in different runs as a method for conducting sensitivity analysis or to achieve calibration goals.

The parameters used in the Leaf model are listed in the following tables. Table 4 presents all the parameters alphabetically by their abbreviations. The column ‘Substance’ allows distinguishing the parameters used for organic substances from those for metals.

Table 4 Parameters used in the Leaf model

| Name | Abbreviation | Description | Unit | Substance |
| --- | --- | --- | --- | --- |
| Extinction factor for partitioning of evapotranspiration to plant transpiration | alpha_extinction | The extinction factor for total solar irradiance functions as partitioning radiant energy between canopy and soil surface, that is, as partitioning evapotranspiration between soil evaporation and plant transpiration | unitless | Organic |
| Diffusion coefficient of oxygen in water | D_O2_water | - | m2 d-1 | Organic |
| Correction factor for density difference between water and n-octanol | delta_density_OW | Factor correcting difference between densities of water and of n-octanol | L kg-1 | Organic |
| Empirical correction factor for differences between solubility in octanol and sorption to leaf lipids | delta_solubility_lipids_leaf | Empirical factor correcting differences between solubility in octanol and sorption to leaf lipids | unitless | Organic |
| Empirical correction factor for differences between solubility in octanol and sorption to root lipids | delta_solubility_lipids_root | Empirical factor correcting differences between solubility in octanol and sorption to root lipids | unitless | Organic |
| Thickness of the water layer in leaf | Delta_x_leaf | - | m | Organic |
| Fraction of organic matter in soil | f_OM_soil | - | g g-1 | Organic |
| Air content of leaf | G_leaf | Air volume contained in leaf per fresh weight of leaf | L kgfw-1 | Organic |
| Air content of root | G_root | Air volume contained in root per fresh weight of root | L kgfw-1 | Organic |
| Henry's law constant | H | Ratio of a chemical’s abundance in gas phase (vapour pressure) to that in aqueous phase (solubility) at equilibrium at a given temperature | Pa m3 mol-1 | Organic |
| Depth of the root zone | h_root | Root depth where root uptake of the pollutant occurs | m | Organic and Metal |
| Lipid content of leaf | L_leaf | Lipid quantity contained in fresh weight of leaf | kg kgfw-1 | Organic |
| Lipid content of root | L_root | Lipid quantity contained in fresh weight of root | kg kgfw-1 | Organic |
| Leaf area index at harvest for leafy crops | LAI_leaf_harvest | One-sided leaf area per unit ground surface area at harvest time for leafy crops | (unitless or m2 (of leaf) m-2 (of soil)) | Organic |
| Degradation rate in leaf | lambda_deg_leaf | First order decay rate of pollutants in leaf | d-1 | Organic |
| Degradation rate in root | lambda_deg_root | First order decay rate of pollutants in leaf root | d-1 | Organic |
| Weathering (wash-off) loss rate from leaf to soil | lambda_weathering_leaf | Loss rate of the pollutant from leaf surface into soil by the wash-off process (climate process like rain and wind). | d-1 | Organic and Metal |
| Water-organic carbon partition coefficient (in log10) | log10_K_oc | Ratio of the equilibrium concentration of a chemical in water to that in soil organic carbon | unitless (L kg-1 in normal scale) | Organic |
| Octanol-water partition coefficient (in log 10) | log10_K_ow | Ratio of the equilibrium concentration of a chemical in octanol to that in water | unitless (m3 m-3 in normal scale) | Organic |
| Molar mass of water | M_H2O | - | g mol-1 | Organic |
| Mass of leaf per unit area of soil at harvest | m_leaf_harvest | The maximal leaf mass per unit ground surface area (m2) at harvest time | kgfw m-2 | Organic and Metal |
| Molar mass of the contaminant | M_molar | - | g mol-1 | Organic |
| Molar mass of dioxygen | M_O2 | - | g mol-1 | Organic |
| Mass of root for leafy crops per unit area of soil at harvest | m_root_leaf_harvest | The maximal root mass of leafy crops per unit ground surface area (m2) at harvest time | kgfw m-2 | Organic |
| Interception coefficient for dry deposits | mu_dry | Empirically derived interception (absorption) coefficient for dry deposits | m2 kgdw-1 | Organic and Metal |
| Interception coefficient for wet deposits | mu_wet | Empirically derived interception (absorption) coefficient for wet deposits | m2 kgdw-1 | Organic and Metal |
| Permeability of the cell wall | P_cell_wall | - | m d-1 | Organic |
| Universal gas constant | R | - | Pa m3 mol-1 K-1 | Organic |
| Dry density of soil | rho_soil_dry | The mass of solid part of soil per unit volume of soil consisting of air, water, and solid parts | kgdw m-3 | Organic and Metal |
| Surface area of field | S_field | Surface area of cultivated field for the plant of interest | m2 | Organic and Metal |
| Date of germination of a leafy crop | t_germ_leaf | Difference between the germination date and the harvest date represents the growth period of a leafy crop | d | Organic and Metal |
| Date of harvest of a leafy crop | t_harv_leaf |  | d | Organic and Metal |
| Transfer factor from soil to leaf | TF_soil_leaf | The substance-specific equilibrium factor. The factor is defined as the concentration in leaf divided by the concentration in soil and thus is empirically derived. | kgdw kgdw-1 | Metal |
| Water content of leaf | Theta_leaf | Water volume contained in leaf per fresh weight of leaf | L kgfw-1 | Organic and Metal |
| Water content of root | Theta_root | Water volume contained in root per fresh weight of root | L kgfw-1 | Organic |

## Intermediate State variables

An ‘Intermediate State variable’ is defined as a dependent variable calculated within the model. Some State variables are fixed during a model run or simulation because they are calculated only from parameters. Some others are time-dependent because they are calculated not only from parameters but also from time-dependent forcing variables. We distinguish ‘Intermediate State variables’ and ‘Regulatory State variables’. The first ones are generally not used by decision-makers for regulatory purposes but can be used as performance indicators of the model that change over the simulation. The second ones can be used by decision-makers for regulatory purposes.

The state variables for which no forcing variables are required are constant all over the calculation time, while the state variables for which forcing variables are required are time-dependent. They are grouped by the following seven categories:

- Parameters related to partition between phases (relevant only for organic substances);

- ‘Xylem influx’: Transfer from soil to root due to xylem flow (relevant only for organic substances);

- ‘Xylem outflux’: Transfer from root to leaf due to xylem flow (relevant only for organic substances);

- ‘Diffusion_upwards /Diffusion_downwards’: Upward/downward diffusions between leaf and air (relevant only for organic substances);

- ‘Irrigation_ intercepted’: Transfer to leaf by the leaf interception of irrigated water (relevant for organic substances and metals);

- ‘Dry/wet intercepted depositions : Dry/wet depositions of aerosols intercepted by leaves (relevant for organic substances and metals);

- ‘Uptake of metals’: Transfer from soil to leaf governed by the equilibrium transfer factor (relevant only for metals).

The intermediate state variables used in the Leaf model are listed for each category in the following tables. The calculation processes of the intermediate state variables are depicted by the following three symbols and by arrows:

It should be noted that several state variables such as T_air_Kelvin, K_air_water, Transpiration, and m_leaf (in abbreviation) are commonly used over different transfer processes.

Table 5 Partition between phases

| State variable n° | Name | Abbreviation and unit | Description | Process followed for calculating the state variable |
| --- | --- | --- | --- | --- |
| 1 | Distribution coefficient of the pollutant between soil and water | Kd_soil (m3 g-1) | Kd_soil is the equilibrium partitioning between concentrations in soil pore water (mg m-3) and in soil solids (mg g-1). |  |
| 2 | Temperature of air (in Kelvin) | T_air_Kelvin (K) | T_air_Kelvin is an expression of temperature of air in Kelvin unit. |  |
| 3 | Partition coefficient between air and water | K_air_water (m3 m-3) | K_air_water is the equilibrium partitioning between concentrations in air and in water. It is an expression of Henry’s law constant in terms of a dimensionless ratio concentration. |  |
| 4 | Partition coefficient between root and water | K_root_water (L kgfw-1) | K_root_water is the equilibrium partitioning between concentrations in roots (mg kg fw-1) and in water or xylem sap (mg L-1). K_root_water considers the sorption to root lipids (estimated by L_root, delta_density_OW, log10_K_ow, and delta_solubility_lipids_root), the dissolution into the aqueous solution of root cells (given by Theta_root), and the partition to the gas phase of root (estimated by G_root and K_air_water). |  |
| 5 | Partition coefficient between leaf and water | K_leaf_water (L kgfw-1) | K_leaf_water is the partitioning between concentrations in leaves (mg kg fw-1) and in water (mg L-1). K_leaf_water considers the sorption to leaf lipids (estimated by L_leaf, delta_density_OW, log10_K_ow, and delta_solubility_lipids_leaf), the dissolution into the aqueous solution of leaf cells (given by Theta_leaf), and the partition to the gas phase of leaf (estimated by G_leaf and K_air_water). |  |
| 6 | Partition coefficient between leaf and air | K_leaf_air (m3 kgfw-1) | K_leaf_air is the partitioning between concentrations in leaves (mg kg fw-1) and in air (mg m-3). |  |

Table 6 Transfer from soil to root due to xylem flow (‘Xylem influx’)

| State variable n° | Name | Abbreviation and unit | Description | Process followed for calculating the state variable |
| --- | --- | --- | --- | --- |
| 7 | Leaf area index for leafy crops | LAI_leaf (unitless or m2 leaf.m-2 soil) | LAI_leaf is a dimensionless quantity that characterizes plant canopies. It is defined as the one-sided green leaf area per unit ground surface area for leafy crops. The canopy area is assumed to grow linearly over the growing period of leaf. |  |
| 8 | Transpiration stream | Transpiration (m3 m-2 d-1) | Transpiration is the process of water movement through a plant and its evaporation from aerial parts such as from leaves. It is defined as a function of the actual evapotranspiration (ET_a), Leaf Area Index for leafy crops (LAI_leaf) and the extinction factor for partitioning of evapotranspiration between soil evaporation and plant transpiration (alpha_extinction). |  |
| 9 | Xylem influx | Xylem_influx (mg d-1) | Xylem_influx represents the transfer from soil to root by xylem influx (transpiration stream). It is calculated from Transpiration, C_soil, Kd_soil, and the surface area of field (S_field). |  |

Table 7 Transfer from root to leaf due to xylem flow (‘Xylem outflux’)

| State variable n° | Name | Abbreviation and unit | Description | Process followed for calculating the state variable |
| --- | --- | --- | --- | --- |
| 8 | Transpiration stream | Transpiration (m3 m-2 d-1) | Described in Table 6 | Described in Table 6 |
| 10 | Mass of root for a leafy crop per unit area of soil | m_root_leaf (kg.fw m-2) | The mass of root for a leafy crop is assumed to grow linearly over the growing period of Leaf. |  |
| 11 | Xylem outflux | Xylem_outflux (d-1) | Xylem_ outflux represents the loss rate from root to leaf due to xylem outflux. |  |

Table 8 Upward/downward diffusions between leaf and air (‘Diffusion_upwards /Diffusion_downwards’)

| State variable n° | Name | Abbreviation and unit | Description | Process followed for calculating the state variable |
| --- | --- | --- | --- | --- |
| 7 | Leaf area index for leaf crops | LAI_leaf (unitless or m2 leaf.m-2 soil) | Described in Table 6 | Described in Table 6 |
| 12 | Mass of leaf per unit area of soil | m_Leaf (kgfw m-2) | The mass of leaf for a leafy crop is assumed to grow linearly over the growing period of Leaf. |  |
| 13 | Permeability of the air boundary layer around plant | P_air (m d-1) | A resistance of 1/200 m s-1 was estimated as typical for a chemical with a molecular mass of 300 g mol-1 for the air boundary layer (Thompson 1983). The ratio of square molar masses is used to derive the permeability in air for any chemicals. |  |
| 14 | Permeability of the cuticle | P_cuticle (m d-1) | A regression equation was derived for the permeability of citrus cuticles (Kerler and Schonherr 1988). |  |
| 15 | Effective diffusion coefficient in pure water | D_water (m2 d-1) | D_water is the effective diffusion coefficient of the pollutant in pure water. It can be related to the ratio between the square root of the pollutant molar mass and that of dioxygen. |  |
| 16 | Permeability of water inside plant | P_water (m d-1) | The permeability of water inside plant is related to the effective diffusion coefficient of a chemical in water. |  |
| 17 | Total permeability of the cuticle pathway | P_cuticle_tot (m d-1) | P_cuticle_tot is assumed to be the sum of four permeabilities in the form of parallel resistance, the permeability of the air boundary layer around plant (P_air), the permeability of water inside plant (P_water), the permeability of the cuticle (P_cuticle), and the permeability of the cell water (P_cell_wall). |  |
| 8 | Transpiration stream | Transpiration (m3 m-2 d-1) | Described in Table 6 | Described in Table 6 |
| 18 | Saturation vapour pressure of water at given temperature | p_water_sat (Pa) | p_water_sat is the maximum saturation pressure of the water vapour in moist air. It depends on the air temperature. |  |
| 2 | Temperature of air (in Kelvin) | T_air_Kelvin (K) | Described in Table 5 | Described in Table 5 |
| 19 | Concentration of water vapour in air at saturation | C_H2O_sat (kg m-3) | C_H2O_sat is measured as the water vapour at saturation per unit volume of air. |  |
| 20 | Conductance of the stomatal pathway for water | g_H2O (m d-1) | g_H2O is the measure of the rate of passage of water through the stomata of leaf. |  |
| 21 | Conductance of stomatal pathway for the chemical | g_stomata (m d-1) | g_stomata is the measure of the rate of passage of the pollutant through the stomata of leaf. |  |
| 22 | Permeability of the stomata | P_stomata (m d-1) | P_stomata is the permeability of the stomatal pathway. It is the product of the stomatal conductance (g_stomata) times the partition coefficient between air and water (K_air_water). |  |
| 23 | Permeability of leaf | P_leaf (m d-1) | P_leaf is the sum of the following permeabilities in the form of a series resistance: the cuticle pathway (P_cuticle_tot) and the stomata permeability (P_stomata). |  |
| 24 | Conductance of leaf | g_leaf (m d-1) | g_leaf is the measure of the rate of passage of the pollutant through leaf. It is a synonym of permeability. The difference between conductivity and permeability is that the ‘permeability’ is related to concentrations in water, while ‘conductance’ is related to concentrations in gas phase. Hence g_leaf can be obtained from the permeability of leaf (P_leaf) times the partition coefficient between air and water (K_air_water). |  |
| 25 | Diffusion upwards | Diffusion_upwards (d-1) | Diffusion_upwards represents the loss rate from leaf to air by diffusion. |  |
| 26 | Diffusion downwards | Diffusion_downwards (mg d-1) | Diffusion_downwards represents the transfer from air to leaf by diffusion. |  |

Table 9: Transfer to leaf by the leaf interception of irrigated water (‘Irrigation _intercepted’)

| State variable n° | Name | Abbreviation and unit | Description | Process followed for calculating the state variable |
| --- | --- | --- | --- | --- |
| 12 | Mass of leaf per unit area of soil | m_leaf (kgfw m-2) | Described in Table 8 | Described in Table 8 |
| 27 | Wet interception fraction onto leaves | f_wet_interception_leaf (unitless) | f_wet_interception_leaf represents the fraction of pollutant transferred onto leaves by the interception of wet aerosol deposition. |  |
| 28 | Transfer of the pollutant to leaves by the interception of irrigated water | Irrigation_ intercepted (mg d-1) | Irrigation intercepted represents the transfer of the pollutant from irrigated water (coming from river water) to leaves due to the interception of irrigated water |  |

Table 10 Dry/wet depositions of aerosols intercepted by leaves (‘Dry/wet depositions aerosols intercepted’)

| State variable n° | Name | Abbreviation and unit | Description | Process followed for calculating the state variable |
| --- | --- | --- | --- | --- |
| 12 | Mass of leaf per unit area of soil | m_leaf (kgfw m-2) | Described in Table 8 | Described in Table 8 |
| 27 | Wet interception fraction onto leaves | f_wet_interception_leaf (unitless) | Described in Table 9 | Described in Table 9 |
| 29 | Dry interception fraction onto leaves | f_dry_interception_leaf (unitless) | f_dry_interception_leaf represents the fraction of pollutant transferred onto leaves by interception dry aerosol deposition. |  |
| 30 | Dry deposition aerosols intercepted | Dry_deposition_intercepted (mg d-1) | Dry_deposition_intercepted represents the transfer from air to leaf by the leaf interception of the dry deposition of contaminated aerosols. |  |
| 31 | Wet deposition aerosols intercepted | Wet_deposition_aerosol_intercepted (mg d-1) | Wet_deposition_aerosol_intercepted represents the transfer from air to leaf by the leaf interception of the wet deposition of contaminated aerosols. |  |

Table 11 Transfer from soil to leaf governed by the equilibrium transfer factor (‘Uptake of metals’)

| State variable n° | Name | Abbreviation and unit | Description | Process followed for calculating the state variable |
| --- | --- | --- | --- | --- |
| 32 | Uptake of metals | Uptake_metals (mg d-1) | Uptake_metals is the transfer rate of metals from soil to leaf. It is estimated from TF_soil_leaf, C_soil, m_leaf_harvest, Theta_leaf, and the growing period of leaf, and the surface area of field (S_field). |  |

## Regulatory State variables

An ‘Regulatory State variable’ is defined as a dependent variable calculated within the model. It is generally time-dependent because it is calculated from parameters, but also from time-dependent forcing variables and loadings. We distinguish ‘Intermediate State variables’ and ‘Regulatory State variables’. The first ones are generally not used by decision-makers for regulatory purposes but can be used as performance indicators of the model that change over the simulation. The second ones can be used by decision-makers for regulatory purposes.

The concentration of a target pollutant in the leaf compartment at harvest time (C_leaf) is defined as a regulatory state variable. The regulatory state variable is presented in the following table together with other state variables used to calculate it. Figure 5 and Figure 6 summarize the calculation processes for organic substances and metals.

Table 12 Regulatory variable for the Leaf model

| State variable n° | Name | Abbreviation and unit | Description |
| --- | --- | --- | --- |
| 33 | Quantity in roots for a leafy crop (only for organic) | Q_root_leaf (mg) | Total quantity of the pollutant in roots for a leafy crop. |
| 34 | Quantity in leaf | Q_leaf (mg) | Total quantity of the pollutant in leaf. |
| 35 | Quantity in leaf at harvest | Q_leaf_harvest (mg) | Q_leaf_harvest is the total quantity of the pollutant in leaf at harvest time. |
| 36 | Concentration in leaf at harvest | C_leaf (mg kgfw-1) | C_leaf is the concentration of the pollutant in leaf at harvest time. It is calculated from the total quantity of the pollutant in leaf at harvest time (Q_leaf_harvest), the mass of leaf at harvest (m_leaf_harvest), and the surface area of field (S_field). |

Figure 5 Calculation process for C_leaf (for organic substances)

Figure 6 Calculation process for C_Leaf (for metals)

End of Level 1 documentation (basic end-user)

Level 2 documentation (background science)

# Processes and assumptions

It should also be noted that this model was built mainly based on the model proposed by Legind and Trapp (2010). Therefore, many assumptions adopted in the model were a priori selected in Legind and Trapp (2010).

## Process n°1: Partition between phases

Motivation

The exchanges of organic compounds between different media can be expressed by equilibrium partition coefficients. In the Leaf model, following inter-media exchanges are taken into consideration:

- Exchange of contaminants between soil and water

- Exchange of contaminants between root and water

- Exchange of contaminants between leaf and air

Selected model and assumptions

A chemical equilibrium between two media is expressed by a constant value. The constant is called the partition coefficient, which is given as the concentration ratio between two media. The present model takes into account the following partition coefficients:

- Kd_soil: It is expressed as the concentration ratio between the particulate phase (mg kg-1) and the dissolved phase (mg m-3). For organic pollutants, Kd_soil is derived from the water-organic carbon partition coefficient (K_oc) and the fraction of organic matter in soil particles (f_OM_soil).

- K_root_water: It is expressed as the concentration ratio between roots (mg kg-1 fresh weight) and water (mg L-1). K_root_water considers the sorption to root lipids, the dissolution into the aqueous solution of root cells, and the partition to the gas phase of root.

- K_leaf_water: It is expressed as the concentration ratio between leaves (mg kg-1 fresh weight) and water (mg L-1). K_leaf_water considers the sorption to leaf lipids, the dissolution into the aqueous solution of leaf cells, and the partition to the gas phase of leaf.

- K_air_water: It is expressed as the concentration ratio between air and water. For organic pollutants, it is an expression of Henry’s law constant in terms of a dimensionless ratio concentration

- K_leaf_air: It is expressed as the concentration ratio between leaf (mg kg-1 fresh weight) and air (mg m-3) which is calculated from K_leaf_water and K_air_water.

The partition (distribution) of pollutants between two phases is relevant for their transfers into other media (compartments). Each partition coefficient is associated to the following transfer process:

- Transfer of pollutants from soil to root by xylem flow: Kd_soil;

- Transfer of pollutants from root to leaf by xylem flow and phloem flow: K_root_water;

- Transfer of pollutant from leaf to air by diffusion: K_leaf_air, which is derived from K_leaf_water and K_air_water).

Alternatives and limits

When equilibrium condition between two phases is not respected (especially for Kd_soil, e.g. just after a direct application), the model must then be considered with caution.

## Process n°2: Xylem influx

Motivation

Xylem flow driven by plant transpiration is a key process in the transfer of pollutants from soil to root with the water taken up.

Selected model and assumptions

In the present model, the uptake of chemical from soil to root is governed by the transpiration stream (Transpiration) and the chemical distribution between soil and water (Kd_soil).

The partition of actual evapotranspiration (ET_a) between soil evaporation and plant transpiration is governed by the solar radiation that can actually reach the soil and that can be spent as latent energy to evaporate water from soil surface. The fraction of solar radiation that can reach soil decreases with the increase of leaf area. Sau et al (2004) and Francisco et al (2008) present the models which partition potential evapotranspiration (ET_p) to potential soil evaporation and potential plant transpiration, based on the Ritchie approach (Ritchie 1972 and 1985). The model for estimating potential plant transpiration requires as parameters ET_p, the time-dependent Leaf Area Index and the extinction factor. The extinction factor is the coefficient of the canopy for total solar irradiance and functions for partitioning evapotranspiration between soil evaporation and plant transpiration. The time- dependent LAI is calculated under the assumption in which aerial plant biomass linearly increases with time until harvest. The present model takes the approach above to estimate actual transpiration stream by assuming that the relation between ET_a and actual transpiration is identical to that between ET_p and potential transpiration.

Alternatives and limits

Several models such as TGD (EC, 2003), CSOIL (Brand et al., 2007) and Xtrafood (Seuntjes et al., 2006) apply the equilibrium partitioning approach for calculating the distribution of chemicals between soil and root. The approach has been questioned for lipophilic chemicals in some studies (Legind and Trapp 2009; Rikken et al. 2001; Trapp 2002; Trapp and Schwartz 2000). Comparing results from the equilibrium approach and the dynamic root model (Trapp 2002) to measured concentrations of benzo(a)pyrene in root crops shows that the equilibrium approach gives a concentration of benzo(a)pyrene in roots that is three orders of magnitude higher than measured. Furthermore, the estimate from the dynamic model lies in the same order of magnitude as the measured value (Legind and Trapp, 2009). Consequently, the present model takes the approach in which the uptake of chemical from soil to root is based on the transpiration flow rate and the chemical distribution between root and soil.

However, the current root uptake mechanism by xylem flux is modelled for lipophilic neutral chemicals based on Trapp (2002) but not for polar chemicals (usually ionic). For the polar substances, the uptake of chemicals into roots is usually slower than that of water. The application of the current model for polar compounds would then be conservative. To be more realistic in a potential future model, it would be necessary to introduce a correction factor that represents the reduced uptake for polar substances.

Kool et al (2014) describes some alternative models for estimating ET partitioning. These are categorized into mechanistic (M) and empirical (P) approaches, or analytical (A) and numerical (N) models. Kool et al (2014) categorizes the following models using the symbols above: Shuttleworth–Wallace (MA); ENWATBAL (MN); Cupid-DPEVAP (MA); SWEAT (MN); TSEB (MA); FAO dual Kc model(PA); HYDRUS-1D (MN). Among these models, FAO dual Kc model (Allen et al, 1998) is the most common model used to partition ET, as it requires relatively few parameters (e.g. wind speed at 2m height, relative humidity) compared to other models. However, the model is empirical and pre-defined crop factors are not always applicable to sites in different contexts. The approach selected for the present model is empirical and requires less parameters than FAO dual Kc model. Therefore, the approach can be useful when the weather data such as wind speed and humidity are not available. A difficulty in the approach may be to give a right value of the extinction factor. Regarding the parameter, however, Sau et al (2004) recommend testing lower values (about 0.5) in a wide range of crops and environments.

## Process n°3: Xylem outflux

Motivation

A fraction of contamination in leaf is originated from the translocation of contaminants from roots to storage organs. Translocation estimation involves a good knowledge of transport processes occurring in the xylem.

Selected model and assumptions

Outflux of the pollutant from the root into the leaf compartment is governed by the xylem flow driven by transpiration stream (Transpiration) and also by the chemical distribution between roots (mg kg-1 fresh weight) and water (xylem sap, mg L-1).

The detail description about the transpiration stream is given in the previous section (4.2).

Alternatives and limits

See the previous section (4.2).

## Process n°4: Degradation in root and leaf compartments

Motivation

Degradation of the pollutant can be a loss term in leaf and root.

Selected model and assumptions

Loss by degradation is governed by a degradation rate. The first order degradation rate represents both biotic and abiotic processes. An example of an abiotic process is photodegradation of dioxins in aboveground plant parts. Biotic processes are metabolism in plants, which for most chemicals is unknown, so the degradation rate constant is an optional input. The individual processes that are responsible for degradation (e.g. biodegradation, photolysis) are not distinguished here but they are added into an aggregated loss rate. Degradation is assumed to to follow linear first-order kinetics.

## Process n°5: Diffusion between leaf and air

Motivation

Some pollutants that are highly volatile (or Semi Volatile Organic Compounds – SVOCs) can be   exchanged between leaf surfaces and atmosphere.

Description and selected assumptions

The Fick’s 1st law of diffusion describes the exchange between two compartments (phases) through a layer as follows:

dm1→2dt=-A×DΔx×(C1-C2)

where dm1→2dt is the flux of chemical from phase 1 to phase 2 (mg d-1), A is the exchange area (m2), D is the diffusion coefficient (m2 d-1), Δx is the diffusion length (m), (C1 – C2) is the concentration gradient between phase 1 and phase 2 (mg m-3). The ratio of the diffusion coefficient D to the diffusion length Δx is commonly named “permeability” in unit of m d-1, synonyms of which are exchange velocity, transfer velocity or conductivity.

The diffusion of chemical between air and leaf is controlled by the conductance of leaf, which is related to the permeability of leaf. The difference between the conductance and permeability is that the former is related to concentrations in the gas phase, while the latter is related to concentrations in water. The diffusive uptake of chemicals by leaf from air mleaf can be expressed as:

dmleafdt=A×g×(Cair-Cleaf/Kleaf_air)

Where A is the leaf surface area (m2), g is the conductance of leaf (m d-1), Cair is the concentration in air, Cleaf is the concentration in leaf, and Kleaf_air is the partition coefficient between leaf and air.

Major resistances that control the exchange between plant and air are: air boundary layer, stomata and cuticle resistances (Riederer 1995). The present model considers the permeability of these three layers together with cell wall permeability, and water layer permeability to model the diffusion between leaf and air. Among them, the permeability of the air boundary layer (P_air), of the cuticle (P_cuticle), of the water layer (P_water), and of the cell wall (P_cell_wall) are aggregated as total permeability of cuticle pathway (P_cuticule_tot). Overall permeability of leaf (P_leaf) can be depicted, as in Figure 7, by several resistors in a series circuit according to the assimilation into Ohm’s law. The overall permeability is assumed to be the addition of two permeabilities such as the permeability of stomata (P_stomata) and of the cuticle pathway (P_cuticule_tot).

The following figure depicts the overall permeability relevant for diffusion between air and leaf.

Figure 7 Permeability relevant for diffusion between air and leaf

Alternatives and limits

The present model estimates the gaseous diffusion by calculating time-dependent and chemical specific permeability of plant tissue, whereas other models (TGD and CSOIL) applies a fixed gas deposition rate.

## Process n°6: Interception of irrigated water

Motivation

Irrigation with poor-quality water is one way that food crops can become contaminated. Food crops can take up contaminants not only through the root uptake of contaminated water irrigated to soil but also through the interception of irrigated water by leaves.

Description and selected assumptions

Transfer of pollutants onto leaves via interception of irrigated water is determined by the interception fraction by leaves for wet deposits, the concentration of the pollutant in irrigated water, and the irrigation rate. The interception fraction by leaves can be quantified by the Chamberlain’s relationship (Chamberlain, 1970). Further description about this relationship is given in the next section (4.7).

## Process n°7: Deposition from air to leaf

Motivation

Some experimental observations showed that the fraction of atmospheric contaminants intercepted by plants (essentially leaves) is not negligible and should be taken into account in the mass budget calculations. For example, several experiments were conducted with radionuclides as tracers of atmospheric aerosols (e.g. Pinder et al, 1988a; Pinder and McLeod, 1988b; Hoffman et al, 1992; Hoffman et al, 1995; Pröhl and Hoffman, 1996; Vandecasteele et al, 2002) and showed that, under some conditions (e.g. high Leaf area Index), the main part of atmospheric particulate contamination is intercepted by plants. For organic pollutants, the relative importance of intercepted particles versus gaseous diffusion obviously depends of the partitioning in the atmosphere between aerosols and gas phase. For some pollutants mainly associated to atmospheric particles (e.g. PAHs), the interception of particulate dry or wet deposits can play a predominant role (Smith and Jones, 2000; Barber et al, 2004). It is then justified to include this process in the modelling of the leaf contamination.

Description and selected assumptions

The present model takes into account the transfer of pollutants on leaves by the dry and wet particle depositions from atmosphere into leaf. The wet particle deposition is driven by rain events and the dry deposition is driven by gravitational deposition of aerosols. In the present model, both the fractions of dry and wet deposits intercepted by leaf are quantified by the Chamberlain’s relationship (Chamberlain, 1970). This relationship relates the interception fraction (i.e. the ratio between the intercepted quantity and the total deposited quantity) and the above-ground biomass and involves a parameter called interception coefficient. Aerial biomass is a time-dependent variable because it is assumed to increase linearly until harvest. This relationship reflects the fact that the interception fraction increases as the plant grows (and the surface of leaves increases). It was for example implemented in the urban multimedia model developed by Diamond et al (2001) (only for dry deposits). The Chamberlain’s model was experimentally supported for pasture grass and other leafy crops.

Alternatives and limits

Some models such as SimpleBox (Brandes et al., 1996) and Caltox (Mckone, 1993) consider that pollutants falling from the atmosphere under dry and wet deposits (as well as irrigation inputs) integrally and directly reach soil, that is, the interception of pollutants by plants is ignored. On the other hand, TrimFate (US-EPA, 2002) and the present model consider the interception of pollutants by plants. However, interception processes are described in different manners among those models. In TrimFate, the fraction of dry deposits intercepted by plants is assumed to be constant, whereas the fraction of wet deposits intercepted by plants is calculated according to the Leaf Area Index, the “vegetation-dependant leaf-wetting factor” (i.e. a retention factor, in m) and amount of rainfall during rainfall events.

The approach chosen in the leaf MERLIN-Expo model is consistent with those selected in the Soil MERLIN-Expo model and able to calculate the fraction of dry and wet deposits reaching soil surface.

## Process n°8: Uptake of metals

Motivation

The bioaccumulation concepts developed for neutral hydrophobic substances (based in particular on diffusion across the lipid biomembranes) are not relevant for metals. For these latter compounds, other processes can significantly influence the accumulation of trace elements by plants like sequestration, detoxification, storage, regulation, etc. The accumulation of metals by plants (and in particular leaves) is generally described by soil-to-plant transfer factors at equilibrium.

Description and selected assumptions

As for the uptake of metals into leaf, the present model considers the constant soil – leaf transfer and the time-dependent air-Leaf transfer by aerial dry/wet depositions. The soil-leaf transfer is governed by a substance-specific equilibrium transfer factor (TF_soil_leaf).  The factor is defined as the concentration in leaf (mg kgdw-1) divided by the total concentration in soil (mg kgdw-1) and thus is empirically derived. The air-leaf transfer is governed by wet and dry particle depositions as for organic substances (see 4.6).

Alternatives and limits

CLEA model (Jeffries and Martin, 2008) estimates the concentration of metals in plant tissue (potato, shoot, or fruit). The model assumes that the concentration in the root is directly proportional to the concentration in the soil solution. The concentration in edible plant parts are found by multiplication with the fraction of metal in the roots that reaches the root store, potato, shoot or fruit by xylem or phloem flow.

When the metal contamination in soil is relatively higher than that in air, the equilibrium transfer factor will be a dominant parameter to determine the metal level in leaf. As the parameter in general entails the large variability, a deterministic calculation using a best estimate of the parameter can be insufficient in terms of risk evaluations of metals.

## Process n°9: Weathering (wash off)

Motivation

Intercepted fraction of pollutants can reach soil with certain delay by the transfer from leaves to soil by climate process (wind, rain, and etc), called “weathering” (wash off).

Description and selected assumptions

Weathering is a process of clearance of contamination from plant leaves. ‘Weathering’ includes a variety of processes like wash-off of previously intercepted material by rain or irrigation or surface abrasion by wind. The magnitude of weathering depends on several factors including meteorological conditions (rain and wind), chemical solubility, strength of adsorption to the plant surface, degree of penetration into the inner leaf structure, plant senescence and defoliation, etc.

Given the complexity of such processes, it is common (especially in radioecology where the concept of weathering was more deeply investigated) to simulate weathering by a global first-order rate constant.

Alternatives and limits

The transfer of pollutants from leaves to soil can take place not only through the weathering process but also through the litter fall. However, the present model does not consider the latter process for simplification.

End of Level 2 documentation (end-user with expertise in process)

Level 3 documentation (numerical information)

# Numerical default values (deterministic and/or probabilistic)

## Initialization of mass balance in Media

The initial mass in the target media, i.e. the total quantity of leaf and leaf roots (Q_leaf and Q_leaf_root (mg), respectively) must be a priori set in the model. By default, no contaminant accumulation (Q_leaf and Q_leaf_root = 0) is assumed at the beginning of the simulation.

## Default parameter values

This section proposes default parameter values. Parameters are grouped by the eight categories almost as defined in the section 1.1 but the first category “Site-specific and plant parameters” includes not only the site-specific parameters but also the parameters about physical characteristics of plant (e.g. G_leaf, Theta_leaf, m_leaf_harvest in abbreviation) and plant growth (e.g. t_germ_leaf, t_harv_leaf in abbreviation). Those parameters grouped in the first category are commonly used over different transfer processes. At the end of each part, a recommended best estimate and a PDF are proposed.

In the following tables, PDFs are presented by their types and statistical values as follows:

- Normal distribution: N(Mean, Sd(standard deviation));

- Log-normal distribution: LN(GM(geometric mean), GSD(geometric standard deviation)) or LN(5%, 95%);

- Uniform distribution: U(Min(minimum), Max(maximum));

- Triangular distribution: T(Min(minimum), Max(maximum), Mode).

### 5.2.1 Site-specific and plant parameters

#### 5.2.1.1 Surface area of field (S_field)

Physical/chemical/biological/empirical meaning

The Surface of the field under investigation (S_field) represents the dimensions of the region under evaluation. The relevant spatial scale is governed by the homogeneity of the soil and agronomic system under investigation (e.g. homogeneity in contamination levels, agricultural practices, and meteorological conditions). It is advised to define a field surface that show low variations in its land use and contamination level.

Parameter default value and PDF

As this parameter is purely site-specific, neither default best estimate nor PDF is provided here. Therefore it should be defined by model users.

#### 5.2.1.2 Air/water/lipid contents of leaf (G_leaf, Theta_leaf, L_leaf) and Air/water/lipid contents of root (G_root, Theta_root, L_root)

Physical/chemical/biological/empirical meaning

Air/water/lipid contents of leaf (root) represent respectively the air volume contained in leaf (root) per fresh weight of leaf (root), the water volume contained in leaf (root) per fresh weight of leaf (root), and the lipid quantity contained in fresh weight of leaf (root).

Description of data source

A literature survey was conducted to collect the data of the parameters. The following table presents the parameter values collected from the literature survey and their sources.

Table 15 Values of G_leaf, Theta_leaf, L_leaf, G_root, Theta_root, L_root collected from the literature survey

| Abbreviation of parameter | Unit | Value | Remark | Original Source |
| --- | --- | --- | --- | --- |
| G_leaf | L kgfw-1 | 0.1 | The supporting information of Legind and Trapp (2009) cites Trapp and Matthies (1995) for this parameter. However, the value of leaf air content is not presented in Trapp and Matthies (1995). Thus, this value is interpreted as a generic value given by an expert judgement made in Legind and Trapp, 2009). | Legind and Trapp (2009) - supporting information |
| G_root | L kgfw-1 | 0.1 | Trapp (2002) presents arbitrary values of the gas fraction in root (set as 0.1 L.L-1) and of the root density (set as 1 kg.fw L-1) . Thus in this report, G_root (L.kg fw-1) was calculated by multiplying two values. | Trapp (2002) |
| Theta_leaf | L kgfw-1 | GM = 0.92, GSD = 1.0, Min = 0.84, Max = 0.97 (n = 88) | IAEA (2010) reports the values of the water content for leafy crops together with non-leafy vegetables. These values apply to the edible part of the plant as harvested. | IAEA (2010) |
|  |  | 0.8 | The supporting information of Legind and Trapp (2009) cites Trapp and Matthies (1995) for this parameter. The value was an estimate which was used for calculations by PLANIX model. | Legind and Trapp (2009) - supporting information, Trapp And Matthies (1995) |
| Theta_root | L kgfw-1 | GM = 0.87, GSD = 1.05, Min = 0.77, Max= 0.95 (n = 39) | IAEA (2010) reports the water content for root crops. These values apply to the edible part of the plant as harvested. | IAEA (2010) |
|  |  | 0.89 | Trapp (2002) presents the average data for carrots that were obtained from Wang and Jones (1994). The data were measured at the time of harvest. | Trapp (2002), Wang and Jones (1994) |
| L_leaf | Kg kgfw-1 | 0.02 | The supporting information of Legind and Trapp (2009) cites Trapp and Matthies (1995) for this parameter. The value was an estimate which was used for calculations by PLANIX model. | Legind and Trapp (2009) - supporting information, Trapp and Matthies (1995) |
| L_root | Kg kgfw-1 | 0.025 | Trapp (2002) presents a generic value. | Trapp (2002) |

Parameter default value and PDF

The following table lists, for each parameter, a recommended value selected from the previous table and also presents a probability density function (PDF).

Table 16 Default values of G_leaf, Theta_leaf, L_leaf, G_root, Theta_root, L_root

| Abbreviation of parameter | Unit | Best estimate | PDF | Comment |
| --- | --- | --- | --- | --- |
| G_leaf | L kgfw-1 | 0.1 |  |  |
| G_root | L kgfw-1 | 0.1 |  |  |
| Theta_leaf | L kgfw-1 | 0.92 | LN(GM = 0.92, GSD = 1.0) | Values from IAEA (2010) were selected because they were derived from a number of samples, and thus they can be regarded as representative values for leafy crops. |
| Theta_root | L kgfw-1 | 0.87 | LN(GM = 0.87, GSD = 1.05) | Values from IAEA (2010) were selected because they were derived from a number of samples, and thus they can be regarded as generic (representative) values for whole root crops. |
| L_leaf | Kg kgfw-1 | 0.02 |  |  |
| L_root | Kg kgfw-1 | 0.025 |  |  |

#### 5.2.1.3 Dates of germination/harvest of a leafy crop (t_germ_leaf, t_harv_leaf)

Physical/chemical/biological/empirical meaning

The dates of germination/harvest of a leafy crop represent the growing period of the leafy of interest. In the Leaf model, all the transfer processes take place only during the growing period.

Description of data source

The growing periods of crops of interest can vary significantly depending on the types of crops and on the regions. Therefore the dates of germination/harvest of leafy crops are regarded as site-specific and they should be chosen by model users.

#### 5.2.1.4 Mass of leaf per unit area of soil at harvest (m_leaf_harvest)

Physical/chemical/biological/empirical meaning

Mass of leaf per unit area of soil at harvest represents the maximal leaf mass per unit ground surface area (m2) at harvest time.

Description of data source and default value

The following table presents the parameter values collected from the FAOStat freely available on internet (http://faostat.fao.org). The geometric means and ranges (min – max) shown in Table 17 were obtained based on the statistics for France and years 1993 to 2012. FAO gives statistics for several leafy crops, countries and years and data can then be adjusted for defining parameter scenario- specific values. It should be noted that Table 17 lists the values not only for leafy crops consumed by people (e.g. cabbage, lettuce, spinach) but also for crops consumed by livestock animals (e.g. cabbage for fodder, clover and grasses used as forage and silage). Values estimated for lettuce (and chicory) are proposed as default values in the Leaf model.

Table 17 Default values of m_leaf_harvest for different leaves (from FAOStat)

| Leaf species | GM of m_leaf_harvest (Kgfw m-2) over the period from 1993 to 2012 in France | Range of m_leaf_harvest (Kgfw m-2) over the period from 1993 to 2012 in France |
| --- | --- | --- |
| Cabbages for fodder | 4.5 | 4.1-5.0 |
| Cabbages and other brassicas | 2.4 | 2.0-3.1 |
| Lettuce and chicory | 2.7 | 2.5-2.9 |
| Spinach | 1.8 | 1.5-2.2 |
| Forage and silage, clover | 3.5 | 2.8-3.9 |
| Forage and silage, grasses nes | 3.2 | 2.6-4.0 |

Best estimate (m_leaf_harvest) = 2.7 (Kgfw m-2)

#### PDF (m_leaf_harvest) = U(min=2.5 , max=2.9) (Kgfw m-2)

#### 5.2.1.5 Mass of root for leafy crops per unit area of soil at harvest (m_root_leaf_harvest)

Physical/chemical/biological/empirical meaning

Mass of root for leafy crops per unit area of soil represents the maximal root mass of leafy crops per unit ground surface area (m2) at harvest time.

Description of data source and default value

In general, the patterns of above-ground biomass (shoot mass) are reasonably well understood, whereas the knowledge of below-ground biomass (root mass) is very limited probably because of the methodological difficulties to measure it or/and also of the poor economic interest to do it. Due to such an informative constraint, the target parameter can be estimated using the information of shoot/root ratio (defined as the shoot biomass divided by the root biomass).

Peach et al (2000) investigated, for field grown plants (carrot, onion, cabbage), the relative contribution of above-ground and below-ground competitions between plants to the edge effects, in which a plant grows more when it is located adjacent to a gap, by using aerial and soil partitions. In the investigation, the effect of aerial and soil partitions on the shoot/root ratio for those plants was analysed.

It was found from the study that the measured shoot/root ratios for cabbage are not sensitive to types of partition (including no partition), ranging between 15.5 and 19.5. With a mid-value over the range, the target parameter can be calculated by the following relationship, assuming that the water contents in leaf and root are almost identical (See 5.2.1.2):

mroot_leaf_harvest=mleaf_harvestRshoot_root

Where

- mleaf_harvest : the maximal leaf mass per unit ground surface area (m2) at harvest time (See 5.2.1.4)

- Rshoot_root : shoot/root dry weight ratio (mid-value = 17.5)

The default values of the parameter were then obtained from the mid-value of shoot/root dry weight ratio (=17.5) and the default values of m_leaf_harvest that are presented in the section 5.2.1.4.

Best estimate (m_root_leaf_harvest) = 0.15 (Kg.fw m-2)

PDF (m_root_leaf_harvest) = U(min=0.14 , max=0.17) (Kg.fw m-2)

### 5.2.2 Parameters related to the partition between phases

#### 5.2.2.1 Fraction of organic matter in soil (f_OM_soil)

Physical/chemical/biological/empirical meaning

Organic carbon is considered the main sorbing phase in soil for neutral organic compounds. As for non-neutral organics, which typically have a log(Kow) greater than 4, their affinity to organic matter tends to be much stronger than to mineral surfaces and the sorption to these latter can be neglected.  Organic carbon content can be divided between amorphous, soft or new and condensed, old or black carbon. Even if this division in types of sedimentary organic matter can be relevant for non-ionic organics, we considered in the model organic carbon as a unique sorbing phase.

The exchanges of organic contaminants between soil pore water and solid soil are assumed to be at equilibrium and represented by partition coefficients at equilibrium (Kd_soil). Interaction of chemicals with solid soil is assumed to be governed by lipophilic sorption onto organic matter. In the present model, the fraction of organic carbon in soil (f_OC_soil), together with the water-organic carbon partition coefficient (K_oc), is used to calculate the partition coefficient at equilibrium Kd_soil.

Organic Matter content in soils depends on natural backgrounds as well as on anthropogenic activities (land use coverage, wet zones management, etc).

Description of data source

Panagos et al (2013) collected data of soil organic carbon and soil erosion in Europe using the European Environment Information and Observation Network for soil (EIONET-SOIL). Panagos et al (2013) presents a best estimate for each European country. Each participating country provided monitoring data on the basis of a grid of 1 km × 1 km cells and for 30 cm soil depth.

For data at a regional scale, specific national monitoring programs can be used. For example, data were collected in the frame of a French program aiming at mapping soil properties at local scales. These data are freely available and can be found in the website www.gissol.fr. Mean and extreme values can thus be calculated for each French region.

Parameter default value and PDF

Table 18 presents the best estimate value and the standard deviation for each country, based on the proposition made by Panagos et al (2013). The data are assumed to follow normal distributions, which must be truncated at zero to avoid negative values. Three examples of default values extracted from the French program Gissol are provided as well in Table 18.

Table 18  Organic fraction in soils in several European countries (from Panagos, 2013) and French regions (from Gissol)

| Country | Number of cells | Best estimate (%) | Standard deviation (%) |
| --- | --- | --- | --- |
| Austria | 55329 | 2.7 | 3.3 |
| Bulgaria | 14101 | 2.1 | 1.0 |
| Denmark | 42919 | 2.0 | 1.5 |
| Estonia | 13379 | 3.5 | 4.6 |
| Ireland | 1322 | 13.3 | 14.1 |
| Italy | 30521 | 3.1 | 2.3 |
| Netherlands | 29866 | 3.5 | 2.6 |
| Norway | 14249 | 3.2 | 3.1 |
| Poland | 220090 | 2.6 | 4.0 |
| Serbia | 1181 | 2.0 | 1.6 |
| Slovakia | 26959 | 1.3 | 1.1 |
| Switzerland | 105 | 4.5 | 5.6 |
| France - Brittany |  | 2.5 | 1.2 |
| France - Alsace |  | 2.5 | 1.2 |
| France - Provence |  | 1.2 | 0.1 |

#### 5.2.2.2 Henry’s law constant (H)

Physical/chemical/biological/empirical meaning

The Henry’s law constant represents the ratio between the vapour pressure and solubility of a chemical at a constant temperature. It describes the capability of the chemical substance to partition between air and water in a binary system. In the present model, the unit is given by Pa.m3.mol-1. When divided by the gas constant R and by temperature, we obtain the dimensionless Henry’s law constant.

The Henry’s law constant can also be defined in the literature as: KH = ca/pg, where ca is the substance concentration in aqueous phase and pg the partial pressure in the gaseous phase. The relationship between H and KH is given by:  H = pg/xa = ρH2O / (MH2Ox KH), where xa is the molar ratio in the aqueous mixture, ρH2O the water density and MH2O the molar mass of water. Under the terminology ‘Henry’s law constant’, both types of values can be found in the literature.

The Henry’s law constant is dependent on the temperature. Values are generally obtained at 25°C. The Henry’s law constant typically increases with increasing temperature and is calculated according to the Van’t Hoff equation often used to model the temperature dependence.

Description of data source

Because experimental data for the Henry’s law constant H are not available for all chemicals in use, several correlations have been developed relating H to molecular descriptors like bond descriptors. Such correlations (called QSAR models) tend to be developed for specific groups or classes of chemicals and can therefore be adapted for some classes of chemicals only. We present here some of the QSAR models that can be used for estimating H values for a given chemical (Table 19) (available in the ChemProp software).

- Meylan and Howard (1991) developed a QSAR model for the estimation of Henry’s law constants based on 59 bond descriptors. The database for training is based on several chemical classes. The model was fitted on 345 chemicals and validated on a independent dataset of 74 substances.  The 345-chemical data set contains a subset of 120 compounds that have both an experimentally measured Henry’s law constants and measured vapor pressures and water solubilities. The standard deviation of the regression model is 0.46 log units.

- Viswanadhan at al (1999) developed a QSAR model based on solvation free energy to estimate Henry’s law constants. Their method (ALOGS) used an extensive atom classification scheme and was fitted on a database containing 265 molecules with experimentally determined solvation free energies. The model was then tested on 27 molecules not present in the training set. The standard deviation of the regression model is 0.86 log units.

- Abraham et al (1994) developed a model based on 5 molecular descriptors, i.e. the excess molar refraction, the dipolarity/polarizability, the effective hydrogen-bond acidity and basicity, and the McGowan characteristic volume. It was fitted on 408 gaseous compounds and the standard deviation of the regression model is 0.15 log units.

- A model was developed by Kühne et al (2005) to estimate the temperature dependency of Henry's law constant in water for organic compounds from the 2D structure. Air/water partition enthalpies of 456 chemicals from various compound classes were fitted to 46 substructure fragments. Application of  the model together with experimental 25°C data to a set of 462 compounds with 2119 experimental Henry's law constants at temperatures below 20°C yields a standard error of 0.21 logarithmic units. The prediction capability is further evaluated using cross validation and permutation.

- A read-across approach was developed by UFZ (personal communication; method is not published yet; details will not be disclosed before publication). The read-across set covers 2354 compounds. Three models are proposed: (i) a Default model (n. of data=1409; rms=0.66); (ii) a High similarity model (n. of data=1127; rms=0.54); (iii) a Low similarity model (n. of data=2352; rms=1.23). Structurally similar compounds in a reference set are looked up via comparison of atom-centered fragments (ACF). The experimental values of the similar compounds are weighted by their similarity. The final result is a weighted average of different runs.

Other models based on the estimation of both water solubility and vapor pressure are also available.

The QSAR models that are indicated above are based on linear regressions fitted by ordinary least squares. Assuming identical, independent and normally distributed errors, the in a QSAR prediction Log H can be defined as the predictive distribution by the predictive mean  and standard error of predictions :

Where  is the student t-distribution with n-k-1 degrees of freedom, n is the number of data in the training set, k is the number of descriptors in the model (and k+1 is the intercept plus the number of descriptors).

The QSAR models that are indicated above generally provide an estimation of the standard error of predictions  by the Mean Squared error MSE. Since MSE is an expectation value, it is subject to estimation error that could be taken into account. The uncertainty on MSE can be calculated from a Bayesian point of view, assuming that the uncertainty of MSE has a scaled inverse Chi distribution. A re-analysis of raw data used in the training set would however be necessary to calculate this posterior distribution. Therefore, in the present model, the MSE uncertainty is not included and the standard error of predictions  is assumed to be equal to the MSE value provided in each QSAR model description.

Table 19 QSAR and read-across models available for calculating Henry’s law constant

| Source | Descriptors | N. of data in the training set | MSE | - 5th-95th percentile |
| --- | --- | --- | --- | --- |
| Meylan and Howard, 1991 | 59 chemical bonds | 345 (and 74 compounds used in the prediction set) | 0.46 | 0.76 |
| Viswanadhan et al, 1999 | No information | 265 (and 26 compounds used in the prediction set) | 0.86 |  |
| Abraham et al, 1994 | 5 descriptors (excess molar refraction,dipolarity/polarizability, effective hydrogen-bond acidity and basicity, McGowan characteristic volume) | 408 | 0.15 | 0.25 |
| Kühne et al, 2005 | 46 substructure fragments. | 456 | 0.21 | 0.34 |
| UFZ, personal communication | Read-across | 2354 |  |  |

Parameter default value and PDF

According to the approach described above, the default best estimates and PDFs of the parameter are given on a log scaled basis for several key substances in Table 24. All the calculations were computed on the ChemProp software that is freely available on request (http://www.ufz.de/index.php?en=6738). The ChemProp returns a result for each compound separately, by applying the methods in the order listed below. The first valid result is accepted. Default order: 1 - Meylan and Howard, 1991; 2 - Viswanadhan et al, 1999; 3 - Abraham et al, 1994. By comparison, the result obtained according to Kühne et al, 2005 and to UFZ (personal communication) is also given. The Meylan method is those that is proposed by default in the MERLIN-Expo model. However, end-users are encouraged to run the ChemProp software with alternative methods (that are not reported comprehensively here) to check the concordance between several approaches and to evaluate the plausibility of estimations (see for example some discrepancies for pesticides in Table 20).

Table 20  Henry’s law constant of selected substances

| Chemical class | Substance | Model | Best estimate – log10H | 5th-95th percentile - log10H | 5th-95th percentile – H (Pa.m3.mol-1) |
| --- | --- | --- | --- | --- | --- |
| PAH | Anthracene | Meylan | 0.71 | -4.6.10-2 ; 1.47 | 0.9;29.8 |
|  |  | UFZ-Read-across | 0.46 |  |  |
|  | Benzo(a)pyrene | Meylan | -1.09 | -1.85 ; -3.2.10-1 | 1.4.10-2 ; 4.7.10-1 |
|  |  | UFZ-Read-across | -0.87 |  |  |
|  | Benzo(b)fluoranthene | Meylan | -1.09 | -1.85 ; -3.2.10-1 | 1.4.10-2 ; 4.7.10-1 |
|  |  | UFZ-Read-across | -1.2 |  |  |
|  | Benzo(k)fluoranthene | Meylan | -1.09 | -1.85 ; -3.2.10-1 | 1.4.10-2 ; 4.7.10-1 |
|  |  | UFZ-Read-across | -1.2 |  |  |
|  | Fluoranthene | Meylan | -7.6.10-2 | -8.4.10-1 ; 6.8.10-1 | 0.15 ; 4.8 |
|  |  | UFZ-Read-across | 4.4.10-2 |  |  |
|  | Naphtalene | Meylan | 1.1 | 3.4.10-1 ; 1.9 | 2.2 ; 73.1 |
|  |  | UFZ-Read-across | 1.27 |  |  |
| PCB | PCB28 | Meylan | 1.23 | 4.7.10-1 ; 2 | 3 ; 98.7 |
|  |  | UFZ-Read-across | 1.32 |  |  |
|  | PCB 52 | Meylan | 1.1 | 3.4.10-1 ; 1.9 | 2.2 ; 73.2 |
|  |  | UFZ-Read-across | 1.27 |  |  |
|  | PCB101 | Meylan | 0.97 | 2.1.10-1 ; 1.7 | 1.6 ; 54.2 |
|  |  | UFZ-Read-across | 1.15 |  |  |
|  | PCB118 | Meylan | 0.97 | 2.1.10-1 ; 1.7 | 1.6 ; 54.2 |
|  |  | UFZ-Read-across | 1.15 |  |  |
|  | PCB138 | Meylan | 0.84 | 8.4.10-2 ; 1.6 | 1.2 ; 40.2 |
|  |  | UFZ-Read-across | 0.8 |  |  |
|  | PCB153 | Meylan | 0.84 | 8.4.10-2 ; 1.6 | 1.2 ; 40.2 |
|  |  | UFZ-Read-across | 0.8 |  |  |
|  | PCB180 | Meylan | 0.71 | -4.6.10-1 ; 1.5 | 0.9 ; 29.8 |
|  |  | UFZ-Read-across | 0.7 |  |  |
| Pesticides | Alachlor | Meylan | -2.65 | -3.4 ; -1.9 | 3.9.10-4 ; 1.3.10-2 |
|  |  | UFZ-Read-across | -2.08 |  |  |
|  | Atrazine | Meylan | -3.35 | -4.1 ; -2.6 | 7.8.10-5 ; 2.6.10-3 |
|  |  | UFZ-Read-across | -3.15 |  |  |
|  | Chlordane | Meylan | 0.85 | 9.4.10-2 ; 1.6 | 1.2 ; 41.1 |
|  |  | UFZ-Read-across | 0.9 |  |  |
|  | Chlorpyrifos | Meylan | -0.6 | -1.4 ; 1.6.10-1 | 4.4.10-2 ; 1.46 |
|  |  | UFZ-Read-across | -0.38 |  |  |
|  | DDT | Meylan | 0.19 | -5.7.10-1 ; 9.5.10-1 | 0.27 ; 9 |
|  |  | UFZ-Read-across | -7.6.10-2 |  |  |
|  | Dieldrin | Meylan | -1.27 | -2 ; -5.1.10-1 | 9.4.10-3 ; 0.31 |
|  |  | UFZ-Read-across | -0.13 |  |  |
|  | Diuron | Meylan | -4.27 | -5 ; -3.5 | 9.4.10-6 ; 3.1.10-4 |
|  |  | UFZ-Read-across | -4.74 |  |  |
|  | Endosulfan | Meylan | -2.04 | -2.8 ; -1.3 | 1.6.10-3 ; 5.3.10-2 |
|  |  | UFZ-Read-across | -0.79 |  |  |
|  | Hexachlorocyclohexane (lindane) | Meylan | 1.41 | 6.5.10-1 ; 2.2 | 4.5 ; 149.4 |
|  |  | UFZ-Read-across | -0.65 |  |  |
|  | Isopruturon | Meylan | -3.7 | -4.5 ; -3 | 3.3.10-5 ; 1.1.10-3 |
|  |  | UFZ-Read-across | -4.62 |  |  |
|  | Malathion | Meylan | -4.07 | -4.8 ; -3.3 | 1.5.10-5 ; 4.9.10-4 |
|  |  | UFZ-Read-across | -2.22 |  |  |
|  | Parathion | Meylan | -1.52 | 2.3 ; -0.77 | 5.2.10-3 ; 0.17 |
|  |  | UFZ-Read-across | -1.51 |  |  |
|  | Pentachlorophenol | Meylan | -1.9 | -2.7 ; -1.1 | 2.2.10-3 ; 7.3.10-2 |
|  |  | UFZ-Read-across | -0.42 |  |  |
| Brominated flame retardants | Pentabromo diphenylether | Meylan | -0.93 | -1.7 ; -1.6.10-1 | 2.1.10-2 ; 0.68 |
|  |  | UFZ-Read-across | -1.6.10-2 |  |  |
|  | Hexabromobiphenyl | Meylan | -7.8.10-1 | -1.5 ; -1.6.10-2 | 2.9.10-2 ; 0.96 |
|  |  | UFZ-Read-across | n.a. |  |  |
| VOCs | Benzene | Meylan | 2.73 | 2 ; 3.5 | 94.3 ; 3121.3 |
|  |  | UFZ-Read-across | 2.75 |  |  |
|  | 1,2-Dichloroethane | Meylan | 3.09 | 2.3 ; 3.9 | 215.9 ; 7150.6 |
|  |  | UFZ-Read-across | 2.00 |  |  |
|  | Dichloromethane | Meylan | 2.96 | 2.2 ; 3.7 | 160.1 ; 5300.8 |
|  |  | UFZ-Read-across | 2.41 |  |  |
|  | Hexachlorobenzene (HCB) | Meylan | 1.95 | 1.2 ; 2.7 | 15.6 ; 518 |
|  |  | UFZ-Read-across | 1.37 |  |  |
|  | Hexachlorobutadiene | Meylan | 3.03 | 2.3 ; 3.8 | 188.1 ; 6227.8 |
|  |  | UFZ-Read-across | 2.63 |  |  |
|  | Pentachlorobenzene | Meylan | 2.08 | 1.3 ; 2.8 | 21.1 ; 698.8 |
|  |  | UFZ-Read-across | 1.77 |  |  |
|  | Trichlorobenzene | Meylan | 2.34 | 1.58 ; 3.1 |  |
|  |  | UFZ-Read-across | 2.18 |  |  |
|  | Trichloromethane (chloroform) | Meylan | 2.51 | 1.75 ; 3.3 | 56.8 ; 1880.8 |
|  |  | UFZ-Read-across | 2.58 |  |  |
| Phthalate | Dibutylphthalate (DBP) | Meylan | -0.9 | -1.67 ; -0.15 | 2.2.10-2 ; 0.72 |
|  |  | UFZ-Read-across | -0.71 |  |  |
|  | Di(2-ethylhexyl)phthalate (DEHP) | Meylan | 7.4.10-2 | -0.69 ; 0.83 | 0.2 ; 6.8 |
|  |  | UFZ-Read-across | -0.31 |  |  |
| Dioxins | 2,3,7,8-TCDD | Meylan | -0.44 | -1.21 ; 0.31 | 6.2.10-2 ; 2.06 |
|  |  | UFZ-Read-across | 0.1 |  |  |
|  | 1,2,3,7,8-PeCDD | Meylan | -0.58 | -1.34 ; 0.18 | 4.6.10-2 ; 1.53 |
|  |  | UFZ-Read-across | 0.18 |  |  |
|  | 1,2,3,7,8-HxCDD | Meylan | -0.7 | -1.47 ; 5.4.10-2 | 3.4.10-2 ; 1.13 |
|  |  | UFZ-Read-across | -0.37 |  |  |
| Phenols - Alkylphenols | 2,4,6-tri-tert-butylphenol | Meylan | -5.7.10-3 | -0.77 ; 0.75 | 0.17 ; 5.68 |
|  |  | UFZ-Read-across | n.a. |  |  |
|  | Nonylphenol | Meylan | -0.22 | -0.98 ; 0.54 | 0.11 ; 3.5 |
|  |  | UFZ-Read-across | -0.41 |  |  |
|  | 2-Octylphenol | Meylan | -0.35 | -1.11 ; 0.41 | 7.8.10-2 ; 2.6 |
|  |  | UFZ-Read-across | 1.31 |  |  |

#### 5.2.2.3 Octanol-water partition coefficient in log 10 (log10_K_ow)

Physical/chemical/biological/empirical meaning

The partition coefficient of a substance between water and a lipophilic solvent (n-octanol) characterizes the equilibrium distribution of the chemical between the two phases. The partition coefficient between water and n-octanol (Kow) is defined as the ratio of the equilibrium concentrations of a chemical in octanol saturated with water and water saturated with octanol. The parameter characterizes the equilibrium distribution of the chemical between the two phases and hence represents the degree to which a chemical prefers organic material to water.

In accordance with Nernst’s law K_ow is independent from compound absolute concentration in n-octanol and water. K_ow is usually weakly dependent on temperature, and experiments to measure K_ow values are set to standard conditions i.e. 25°C and 1 atm (OECD protocol). Large organic compounds or highly nonpolar compounds exhibit a high degree of hydrophobicity. Such hydrophobic substances tend to partition to octanol rather than to water and hence have large values of Kow. In contrast, smaller or polar substances are less hydrophobic and have smaller values of Kow. The range of Kow is many orders of magnitude for variety of organic compounds.

Literature value with log K_ow > 6 may contain significant error and thus should be carefully checked.

Kow is not an accurate determinant of lipophilicity for ionizable compounds because it only correctly describes the partition coefficient of neutral (uncharged) molecules. For example, the parameter is not a good predictor of drug behaviours in the changing pH environments of the body because the majority of them are ionizable. The partition coefficients between leaf/root and water (K_root_water  and K_leaf_water) are governed by the lipid content of the plant organs. For calculating these partition coefficients, empirical relationships are generally proposed, using the Octanol-water partition coefficient as a surrogate for Lipid-water partition.

Kow is not an accurate determinant of lipophilicity for ionizable compounds because it only correctly describes the partition coefficient of neutral (uncharged) molecules. For example, the parameter is not a good predictor of drug behaviors in the changing pH environments of the body because the majority of them are ionizable.

Description of data source

Experimental protocols to measure Kow are well established and standardised, for example, in OECD guidelines (OECD, 2006; OECD, 2004; OECD 1995). When the parameter is measured following such well-established experimental protocols, it can be assumed that the obtained values of Kow are not likely to be very variable. The measurement values of Kow are available for many of well-recognized toxic substances. Therefore, in this document, measurement data were selected in preference for the determination of default Kow values.

The measurement data were obtained from OECD (2004) and from the software called EPI-Suite developed by US-EPA (http://www.epa.gov/opptintr/exposure/pubs/episuite.htm). The measurement data in EPI-Suite were initially entered by junior and senior scientists using many sources for which the data had already been carefully evaluated, and then were checked by senior scientists.  The QSAR approach used in the EPI-Suite (i.e. “KOWWIN”) was applied to estimate Kow in the case where no measurement was available. The approach uses a "fragment constant" methodology to predict Kow.  In the "fragment constant" method, a structure is divided into fragments (atom or larger functional groups) and coefficient values of each fragment or group are summed together to yield the log Kow estimate. The KOWWIN’s methodology is known as an Atom/Fragment Contribution (AFC) method.  Coefficients for individual fragments and groups were derived by multiple regression. Meylan and Howard (1995) presents a more complete description of the KOWWIN’s methodology. Analysis of applicability domain of the KOWWIN model (Nikolova and Jaworska, 2005) was performed with the training set of 2434 compounds (r2 = 0,981 and RMSE (root mean squared error) = 0.22). This analysis revealed 186 different fragments and 322 different correction factors, resulting in a 508-dimensional descriptor space. The validation set consisted of 10 910 substances, in which the log Kow values vary between -4.99 and 11.71.

Assuming identical, independent and normally distributed errors, the uncertainty in a QSAR prediction log Kow can be defined as the predictive distribution by the predictive mean log Kow and standard error of predictions K_ow:

log K_owp~log K_owp+tn-k-1.SE (log K_owp)

Where  is the student t-distribution with n-k-1 degrees of freedom, n is the number of data in the training set, k is the number of descriptors in the model (and k+1 is the intercept plus the number of descriptors).

The QSAR model used here provides an estimation of the standard error of predictions SE (log K_owp) by the Mean Squared error (MSE).

Parameter default value and PDF

Table 21 presents the values of Kow for well-recognized toxic substances with experimental data (when available), simulated data generated by the KOWWIN and uncertainty derived from the approach presented above. Experimental references sited by EPI-Suite are all documented in the software (freely downloadable from the link shown above).

Table 21  Kow of selected substances

| Chemical class | Substance | Log Kow experimental estimate (ref in appendix 1) | Log Kow KOWWIN estimate | 5th-95th percentile (KOWWIN) |
| --- | --- | --- | --- | --- |
| PAH | Anthracene | 4.45 | 4.35 | 3.99 - 4.71 |
|  | Benzo(a)pyrene | 6.13 | 6.11 | 5.75 - 6.47 |
|  | Benzo(b)fluoranthene | 5.78 | 6.11 | 5.75 - 6.47 |
|  | Benzo(k)fluoranthene | 6.11 | 6.11 | 5.75 - 6.47 |
|  | Fluoranthene | 5.16 | 4.93 | 4.57 - 5.29 |
|  | Naphthalene | 3.30 | 3.17 | 2.81 - 3.53 |
| PCB | PCB28 | 5.62 | 5.69 | 5.33 - 6.05 |
|  | PCB 52 | 6.09 | 6.34 | 5.98 - 6.70 |
|  | PCB101 | 6.80 | 6.98 | 6.62 - 7.34 |
|  | PCB118 | 7.12 | 6.98 | 6.62 - 7.34 |
|  | PCB138 | 7.44 | 7.62 | 7.26 - 7.98 |
|  | PCB153 | 7.75 | 7.62 | 7.26 - 7.98 |
|  | PCB180 | - | 8.27 | - |
| Pesticides | Alachlor | 3.37 | 3.52 | 3.01 - 3.73 |
|  | Atrazine | 2.61 | 2.82 | 2.46 - 3.18 |
|  | Chlordane | 6.16 | 6.26 | 5.90 - 6.62 |
|  | Chlorpyrifos | 4.96 | 4.66 | 4.30 - 5.02 |
|  | DDT | 6.91 | 6.79 | 6.43 - 7.15 |
|  | Dieldrin | 5.40 | 5.45 | 5.09 - 5.81 |
|  | Diuron | 2.68 | 2.67 | 2.31 - 3.03 |
|  | Endosulfan | 3.83 | 3.50 | 3.14 - 3.86 |
|  | Hexachlorocyclohexane (lindane) | 3.72 | 4.26 | 3.90 - 4.62 |
|  | Isoproturon | 2.87 | 2.84 | 2.48 - 3.20 |
|  | Malathion | 2.36 | 2.29 | 1.93 - 2.65 |
|  | Parathion | 3.83 | 3.73 | 3.37 - 4.09 |
|  | Pentachlorophenol | 5.12 | 4.74 | 4.38 - 5.10 |
| Brominated flame retardants | Pentabromo diphenylether | - | 7.66 | - |
|  | Hexabromobiphenyl | - | 9.10 | - |
| VOCs | Benzene | 2.13 | 1.99 | 1.63 - 2.35 |
|  | 1,2-Dichloroethane | 1.48 | 1.83 | 1.47 - 2.19 |
|  | Dichloromethane | 1.25 | 1.34 | 0.98 - 1.70 |
|  | Hexachlorobenzene (HCB) | 5.73 | 5.86 | 5.50 -6.22 |
|  | Hexachlorobutadiene | 4.78 | 4.72 | 4.36 - 5.08 |
|  | Pentachlorobenzene | 5.17 | 5.22 | 4.86 - 5.58 |
|  | Trichlorobenzene | 4.05 | 3.93 | 3.57 - 4.29 |
|  | Trichloromethane (chloroform) | 1.97 | 1.52 | 1.16 - 1.88 |
| Phthalate | Dibutylphthalate (DBP) | 5.53 | 4.61 | 4.25 - 4.97 |
|  | Di(2-ethylhexyl)phthalate (DEHP) | 7.60 | 8.39 | 8.03 - 8.75 |
| Dioxins | 2,3,7,8-TCDD | 6.80 | 6.92 | 6.56 - 7.28 |
|  | 1,2,3,7,8-PeCDD | 6.64 | 7.56 | 7.20 - 7.92 |
|  | 1,2,3,6,8-HxCDD | 7.80 | 8.21 | 7.85 - 8.57 |
| Phenols - Alkylphenols | 2,4,6-tri-tert-butylphenol | 6.06 | 6.39 | 6.03 - 6.75 |
|  | Nonylphenol | 5.76 | 5.99 | 5.63 - 6.35 |
|  | 2-Octylphenol | - | 5.50 | - |

#### 5.2.2.4 Water-organic carbon partition coefficient in log10 (log10_K_oc)

Physical/chemical/biological/empirical meaning

Organic carbon is assumed to be the main particulate media interacting with hydrophobic chemicals potentially present in soil. The Water-Organic Carbon partition coefficient represents the ratio at equilibrium of the chemical associated to particulate organic matter and present in water respectively.

The exchanges of contaminants between Pore Water and Soil particles are assumed to be at equilibrium and represented by a Partition coefficient at equilibrium Kd_soil_organic. Interaction of chemicals with particles are assumed to be governed by lipophilic sorption onto organic matter. The K_oc parameter, together with the organic fraction in soil, allows calculating the partition coefficient at equilibrium Kd_soil.

The main limitation of the approach described above is that the variability in the soil composition is only described by the content of organic carbon. The validity of this assumption can be disputable especially for ionizable compounds (Ter Laak et al, 2006). In particular, the organic carbon content alone as the descriptor of soil composition is not sufficient to predict the soil–water distribution of chemicals that do not exclusively adsorb to organic matter. Furthermore, the composition of organic carbon itself can vary substantially and influence sorption. Complexation is another process neglected by the model, although it may have significant impact for some compounds.

Description of data source

Because experimental K_oc data are not available for all chemicals in use, numerous correlations have been developed relating K_oc to molecular descriptors like the Octanol-Water partition coefficient K_ow. Such correlations (called QSAR models) tend to be developed for specific groups or classes of chemicals and can therefore be adapted for some classes of chemicals only. We present here some of the QSAR models that can be used for estimating K_oc values for a given chemical (Table 22).

- A classical hydrophobic approach based on Kow and on a decision tree was proposed by Sablić et al (1995, 1996). Log Koc is estimated by a hierarchical decision tree, offering 20 different equations in total. The first equation applies the topological index 1χ, while the other 19 equations correlate log Koc to log Kow. For non-polar compounds, the more precise but also restricted model is the one with 1χ, if it cannot be applied, a more general, less precise equation is used.

- Schüürmann et al (2006) developed another model for non-ionic organic compounds. Literature data of logKoc for 571 organic chemicals (subdivided in 457 for training set and 114 for predictive set) were fitted to 29 parameters. The general form of the model is:

with 3 variable Pi (molecular weight, bond connectivity, molecular E-state), 21 fragment correction factors Fj, 4 structural indicator variables Ik. The data set compounds are neutral organics (except for partial ionization of acids and bases at soil pH) that include the following atom types: C, H, N, O, P, S, F, Cl, and Br. Because the training set contained no organoiodine compounds, I is not included, but it is likely that a simple extension (see below) will provide reasonable estimates for compounds with iodine attached to aliphatic or aromatic carbon. The range confidence is checked for the molecular correction factors by comparing the frequency of these substructures in the molecule to the training set. The compound will be checked versus the original training set compounds by means of second order ACFs (Atom Centered Fragments). The applicability domain is then classified as: (i) In: All ACFs are matching including the number of occurrences; (ii) Borderline in: Either the frequency of at least one substructure of the compound exceeds the range of occurrences in the training set, or one substructure is not in the training set at all; (iii) Borderline out: More than one substructure is not in the training set at all, but all 1st order ACFs are matching; (iv) Out: There is mismatch even with 1st order ACFs.

- Tao et al (1999) developed another model based on logKoc data for 592 organic chemicals (subdivided in 430 for training set and 162 for predictive set) and on 98 parameters (74 fragment constants and 24 structural factors).

- Huuskonen (2003) developed a model based on atom-type electrotopological state indices, involving 12 parameters (connectivity index 1χ, 11 atom-type E-state indices). It was tested on logKoc data for 201 organic pesticides (subdivided in 143 for training set and 58 for predictive set). The general form of the model is:

where Si are the atom-type E-state values..

- Poole and Poole (1999) developed a solvatation-based model to predict logK_oc. After removal of the outliers, the model is under the form:

where V is the McGowan’s characteristic volume, E is the excess molar refraction, A and B(O)  are the solute’s effective hydrogen-bond acidity and hydrogen-bond basicity.

- Franco et al (2008, 2009) developed a QSAR model for ionizable compounds (monovalent organic acids and bases). The classical Kow model is applied here, but the Kow value accounts for the distribution of the chemical between neutral and ionic forms. The neutral and ionic fractions are calculated from the substance pKa and the surrounding pH, according to the Henderson-Hasselbalch relationship:

for acids;

for bases.

Thus, supplying of a valid pKa and a pH is required for running the model. The models do not work for neutral compounds without specification of pKa.

The QSAR models that are indicated above are based on linear regressions fitted by ordinary least squares. Assuming identical, independent and normally distributed errors, the uncertainty in a QSAR prediction Log Koc,p can be defined as the predictive distribution by the predictive mean  and standard error of predictions :

Where  is the student t-distribution with n-k-1 degrees of freedom, n is the number of data in the training set, k is the number of descriptors in the model (and k+1 is the intercept plus the number of descriptors).

The QSAR models that are indicated above generally provide an estimation of the standard error of predictions  by the Mean Squared error MSE. Since MSE is an expectation value, it is subject to estimation error that could be taken into account. The uncertainty on MSE can be calculated from a Bayesian point of view, assuming that the uncertainty of MSE has a scaled inverse Chi distribution. A re-analysis of raw data used in the training set would however be necessary to calculate this posterior distribution. Therefore, in the present model, the MSE uncertainty is not included and the standard error of predictions  is assumed to be equal to the MSE value provided in each QSAR model description.

Table 22  QSAR available for calculating logKoc

| Source | Descriptors | N. of data in the training set | MSE | Applicability domain | - 5th-95th percentile |
| --- | --- | --- | --- | --- | --- |
| Sablić et al (1995, 1996) | 1: topological index 1χ | 81 | 0.264 | Predominantly hydrophobics - 3 to 22 atoms of carbon or halogenes with 1<logKoc<6,5 | 0.44 |
|  | 1: octanol-water partition coefficient Kow | 81 | 0.451 | Predominantly hydrophobics | 0.75 |
|  |  | 390 | 0.557 | Nonhydrophobics with -2<logKow<8 | 0.92 |
|  |  | 54 | 0.401 | Phenols, Anilines, Benzonitriles, Nitrobenzenes with 1<logKow<5 | 0.67 |
|  |  | 216 | 0.425 | Acetanilides, Carbamates, Esters, Phenylureas, Phosphates, Triazines, Triazoles, Uracils with -1<logKow<8 | 0.70 |
|  |  | 36 | 0.388 | Alcohols, Organic acids with -1<logKow<5 | 0.66 |
|  |  | 21 | 0.339 | Acetanilides | 0.59 |
|  |  | 13 | 0.397 | Alcohols | 0.71 |
|  |  | 28 | 0.491 | Amides | 0.84 |
|  |  | 20 | 0.341 | Anilines | 0.59 |
|  |  | 43 | 0.408 | Carbamates | 0.69 |
|  |  | 20 | 0.242 | Dinitroanilines | 0.42 |
|  |  | 25 | 0.463 | Esters | 0.79 |
|  |  | 10 | 0.583 | Nitrobenzenes | 1.08 |
|  |  | 23 | 0.336 | Organic acids | 0.58 |
|  |  | 24 | 0.373 | Phenols, Benzonitriles | 0.64 |
|  |  | 52 | 0.335 | Phenylureas | 0.56 |
|  |  | 41 | 0.452 | Phosphates | 0.76 |
|  |  | 16 | 0.379 | Triazines | 0.67 |
|  |  | 15 | 0.482 | Triazoles | 0.85 |
| Schüürmann et al, 2006 | 29 descriptors : molecular weight, bond connectivity, molecular E-state, 24 fragment corrections representing polar groups, one indicator for nonpolar and weakly polar compounds | 457 (and 114 compounds used in the prediction set) | 0.467 | Neutral organics (except for partial ionization of acids and bases at soil pH) with atom types C, H, N, O, P, S, F, Cl, Br | 0.77 |
| Tao et al, 1999 | 98 descriptors (74 fragment constants and 24 structural factors) | 430 (and 162 compounds used in the prediction set) | 0.366 | Organics with K_oc over 7.65 log-units | 0.60 |
| Huuskonen, 2003 | 12 structural parameters (connectivity index 1χ, 11 atom-type E-state indices) | 143 (and 58 compounds used in the prediction set) | 0.40 | Organic pesticides (with logKoc ranging from 0.42 to 5.31) | 0.66 |
| Poole et al, 1999 | 4 descriptors | 131 | 0.248 |  | 0.41 |
| Franco et al, 2008, 2009 | 3 descriptors or conditions (octanol-water partition coefficient Kow, pKa, pH) | 44 (10 acids, 12 bases, different pH) | 0.32 | Ionizable monovalent organic acids and bases | 0.54 |

Parameter default value and PDF

According to the approach described above, the default best estimates and PDFs for the Log scaled K_oc parameter are given for several key substances in Table 23. All the calculations were computed on the ChemProp software that is freely available on request (http://www.ufz.de/index.php?en=6738). When the Schüürmann’s approach indicates ‘In’ for the applicability domain, it was the preferred approach because it explicitly checks the validity domain. Instead, the Sablic approach was considered. For pesticides, the Huuskonen’s approach was considered because it was specifically developed for this class of chemicals. The grey lines indicate the methods that are proposed by default in the MERLIN-Expo tool. However, end-users are encouraged to run the ChemProp software with alternative methods (that are not reported comprehensively here) to check the concordance between several approaches and to evaluate the plausibility of estimations.

Table 23 Log10 Koc of selected substances

| Chemical class | Substance | Model | Applicability domain | Best estimate | 5th-95th percentile |
| --- | --- | --- | --- | --- | --- |
| PAH | Anthracene | Sablić – Equ. 1 |  | 4.3 | 3.86-4.74 |
|  |  | Schüürmann | In | 4.08 | 3,31-4,85 |
|  | Benzo(a)pyrene | Sablić – Equ. 1 |  | 5.86 | 5,42-6,3 |
|  |  | Schüürmann | In | 5.7 | 4,93-6,47 |
|  | Benzo(b)fluoranthene | Sablić – Equ. 1 |  | 5.86 | 5,42-6,3 |
|  |  | Schüürmann | In | 5.18 | 4,41-5,95 |
|  | Benzo(k)fluoranthene | Sablić – Equ. 1 |  | 5.86 | 5,42-6,3 |
|  |  | Schüürmann | In | 5.18 | 4,41-5,95 |
|  | Fluoranthene | Sablić – Equ. 1 |  | 4.83 | 4,39-5,27 |
|  |  | Schüürmann | In | 4.23 | 3,46-5 |
|  | Naphtalene | Sablić – Equ. 1 |  | 3.28 | 2,84-3,72 |
|  |  | Schüürmann | In | 3.12 | 2,35-3,89 |
| PCB | PCB28 | Sablić – Equ. 1 |  | 4.43 | 3,99-4,87 |
|  |  | Schüürmann | In | 4.26 | 3,49-5,03 |
|  | PCB 52 | Sablić – Equ. 1 |  | 4.64 | 4,2-5,08 |
|  |  | Schüürmann | In | 4.63 | 3,86-5,4 |
|  | PCB101 | Sablić – Equ. 1 |  | 4.85 | 4,41-5,29 |
|  |  | Schüürmann | In | 5.00 | 4,23-5,77 |
|  | PCB118 | Sablić – Equ. 1 |  | 4.85 | 4,41-5,29 |
|  |  | Tao |  | 6.12 | 5,52-6,72 |
|  |  | Poole |  | 5.61 | 5,2-6,02 |
|  | PCB138 | Sablić – Equ. 1 |  | 5.08 | 4,64-5,52 |
|  |  | Schüürmann | In | 5.36 | 4,59-6,13 |
|  | PCB153 | Sablić – Equ. 1 |  | 5.07 | 4,63-5,51 |
|  |  | Schüürmann | Border In | 5.37 | 4,6-6,14 |
|  |  | Tao |  | 6.63 | 6,03-7,23 |
|  |  | Poole |  | 5.88 | 5,47-6,29 |
|  | PCB180 | Sablić – Equ. 1 |  | 5.29 | 4,85-5,73 |
|  |  | Schüürmann | Border In | 5.73 | 4,96-6,5 |
|  |  | Tao |  | 7.14 | 6,54-7,74 |
|  |  | Poole |  | 6.26 | 5,85-6,67 |
| Pesticides | Alachlor | Huuskonen |  | 2.83 | 2,17-3,49 |
|  | Atrazine |  |  | 2.44 | 1,78-3,1 |
|  | Chlordane |  |  | 5.15 | 4,49-5,81 |
|  | Chlorpyrifos |  |  | 3.6 | 2,94-4,26 |
|  | DDT |  |  | 4.72 | 4,06-5,38 |
|  | Dieldrin |  |  | 4.49 | 3,83-5,15 |
|  | Diuron |  |  | 2.29 | 1,63-2,95 |
|  | Endosulfan |  |  | 4.04 | 3,38-4,7 |
|  | Hexachlorocyclohexane (lindane) |  |  | 3.7 | 3,04-4,36 |
|  | Isopruturon |  |  | 2.05 | 1,39-2,71 |
|  | Malathion |  |  | 2.4 | 1,74-3,06 |
|  | Parathion |  |  | 2.82 | 2,16-3,48 |
|  | Pentachlorophenol |  |  | 3.46 | 2,8-4,12 |
| Brominated flame retardants | Pentabromo diphenylether | Sablić – Equ. 3 |  | 4.74 | 3.82-5,66 |
|  |  | Schüürmann | Border Out | 5.34 | 4,57-6,11 |
|  | Hexabromobiphenyl | Sablić – Equ. 1 |  | 5.08 | 4,64-5,52 |
|  |  | Schüürmann | Border Out | 6.32 | 5,55-7,09 |
| VOCs | Benzene | Sablić – Equ. 1 |  | 2.26 | 1,82-2,7 |
|  |  | Schüürmann | In | 2.18 | 1,41-2,95 |
|  | 1,2-Dichloroethane | Sablić – Equ. 1 |  | 1.70 | 1,26-2,14 |
|  |  | Schüürmann | In | 1.85 | 1,08-2,62 |
|  | Dichloromethane | Sablić – Equ. 1 |  | 1.44 | 1-1,88 |
|  | Hexachlorobenzene (HCB) | Sablić – Equ. 1 |  | 3.54 | 3,1-3,98 |
|  |  | Schüürmann | Border In | 4.37 | 3,6-5,14 |
|  | Hexachlorobutadiene | Sablić – Equ. 1 |  | 3.02 | 2,58-3,46 |
|  | Pentachlorobenzene | Sablić – Equ. 1 |  | 3.32 | 2,88-3,76 |
|  |  | Schüürmann | In | 4.01 | 3,24-4,78 |
|  | Trichlorobenzene | Sablić – Equ. 1 |  | 2.89 | 2,45-3,33 |
|  |  | Schüürmann | In | 3.28 | 2,51-4,05 |
|  | Trichloromethane (chloroform) | Sablić – Equ. 1 |  | 1.6 | 1,16-2,04 |
| Phthalate | Dibutylphthalate (DBP) | Sablić – Equ. 1 | Chemical domain mismatch : at least one substructure not represented | 3.03 | 2,59-3,47 |
|  | Di(2-ethylhexyl)phthalate (DEHP) | Sablić – Equ. 1 |  | 4.54 | 4,1-4,98 |
|  |  | Schüürmann | In | 4.15 | 3,38-4,92 |
| Dioxins | 2,3,7,8-TCDD | Sablić – Equ. 1 |  | 3.91 | 3,47-4,35 |
|  |  | Schüürmann | In | 4.62 | 3,85-5,39 |
|  | 1,2,3,7,8-PeCDD | Sablić – Equ. 1 |  | 4.26 | 3,82-4,7 |
|  |  | Schüürmann | Border Out | 4.98 | 4,21-5,75 |
|  | 1,2,3,6,8-HxCDD | Sablić – Equ. 3 |  | 4.62 | 3,7-5,54 |
| Phenols - Alkylphenols | 2,4,6-tri-tert-butylphenol | Sablić – Equ. 3 | Chemical domain borderline approached : at least one substructure occurrence outside thresholds | 4.35 | 3,43-5,27 |
|  | Nonylphenol | Sablić – Equ. 3 |  | 3.82 | 2,9-4,74 |
|  |  | Schüürmann | Border Out | 3.62 | 2,85-4,39 |
|  | 2-Octylphenol | Sablić – Equ. 3 |  | 3.58 | 2,66-4,5 |
|  |  | Schüürmann | Border Out | 3.46 | 2,69-4,23 |

#### 5.2.2.5 Empirical correction factors

Physical/chemical/biological/empirical meaning

Several empirical correction factors are used in the Leaf model to correct differences between:

- solubility in octanol and sorption to leaf lipids (delta_solubility_lipids_leaf)

- solubility in octanol and sorption to root lipids (delta_solubility_lipids_root)

- densities of water and of octanol (delta_density_OW)

Description of data source and Parameter default value

A literature survey was conducted to collect the data of the parameters. The following table presents the parameter values collected from the literature survey and their sources. Only single value was found for each parameter and hence the value was chosen as a best estimate.

Table 24 Default values of delta_solubility_lipids_leaf, delta_solubility_lipids_root and delta_density_OW

| Abbreviation of parameter | Unit | Best estimate | Remark | Original Source |
| --- | --- | --- | --- | --- |
| delta_solubility_lipids_leaf | unitless | 0.95 | Briggs et al (1983) present the correction factor for leaf lipids | Briggs et al (1983) |
| delta_solubility_lipids_root | unitless | 0.77 | Trapp (2002) presents the value 0.77 originated from Briggs et al (1982). | Trapp (2002), Briggs et al (1982) |
| Delta_density_OW | L kg-1 | 1.22 | Equals to 1 divided by octanol density (0.82 kg L-1). | - |

#### 5.2.2.6 Universal gas constant (R)

Physical/chemical/biological/empirical meaning

R is the gas constant in the ideal gas equation, pV = nRT. In the current model, it is used to convert Henry’s law constant into a dimensionless value, which corresponds to the partition coefficient between air and water (K_air_water).

Parameter default value and PDF

R has a value of 8.314 in the unit of Pa m3 mol-1 K-1.

### 5.2.3 Parameters related to the xylem influx

#### 5.2.3.1 Extinction factor for partitioning of evapotranspiration to plant transpiration (alpha_extinction)

Physical/chemical/biological/empirical meaning

alpha_extinction represents the extinction factor for total solar irradiance which functions as partitioning radiant energy between canopy and soil surface, that is, as partitioning evapotranspiration between soil evaporation and plant transpiration.

Description of data source

A literature survey was conducted to collect the data of the parameters. The following table presents the parameter values collected from the literature survey and their sources.

Table 25 Values of alpha_extinction collected from the literature survey

| Abbreviation of parameter | Unit | Value | Remark | Original Source |
| --- | --- | --- | --- | --- |
| alpha_extinction | unitless | 0.5 | The authors recommend, in their conclusions, testing lower values (close to 0.5) in a wide range of crops and environments. | Sau et al (2004) |
|  |  | Mid-value = 0.9, Min = 0.3, Max = 1.5 | Tahiri (2006) mentions that measured values of the extinction coefficient vary from about 0.3 to 1.5 depending on crop species and stand types (external structure of plants). The mid-value was then added to the two values. | Tahiri et al (2006) |

Parameter default value and PDF

A mid value between the value from Sau et al (2004) and the mid-value calculated based on Tahiri et al (2006) was selected as a best estimate because there is no proof to decide which is better. A PDF was proposed according to the range suggested by Tahiri et al (2006).

Best estimate (alpha_extinction) = 0.7 (unitless)

PDF (alpha_extinction) = U(min=0.3 , max=1.5) (unitless)

#### 5.2.3.2 Leaf Area Index at harvest for leafy crops (LAI_leaf_harvest)

Physical/chemical/biological/empirical meaning

The LAI_harvest_leaf represents one-sided leaf area (m2) per unit ground surface area (m2) at harvest time. Leaf Area Index is an important parameter in the Leaf model, as it influences transpiration rates.

Description of data source

Leaf Area Index has been estimated for a variety of plants and trees. Asner et al (2003) built a database of LAI, categorizing the data into different biomes (such as crops, deserts, grasslands, plantations, wetlands, and so on), and then made Statistical analyses for each biome. Breuer et al (2003) collected many LAI values over different plant crops. However neither of the two studies records or specifies LAI values of leafy crops.

For the LAI data in Breuer et al (2003), goodness-of-fit tests (Kolmogorov-Smirnov test and Chi-square test) were performed with 5% of significance level to derive the PDFs.

Table 26 summarizes the statistical values of the LAI from these three studies.

Table 26 Leaf Area Index at harvest

| Plant category | Number of data | Mean | SD | Source |
| --- | --- | --- | --- | --- |
| Plantation | 77 | 8.72 | 4.32 | Asner et al, 2003 |
| Crops (crop type are not specified) | 83 | 3.62 | 2.06 |  |
| All | 878 | 4.51 | 2.52 |  |
| All crops (fruits are not included) | 37 | GM = 3.6 | GSD = 1.5 | Breuer et al, 2003 * The estimations of GM and GSD were conducted by the document authors |
| Kale (similar to cabbage) | 1 | 3.1 | - |  |

Parameter default value and PDF

Only one value (for kale) is relevant for the parameter of interest. This poor information does not tell us about the margin of uncertainty potentially residing in this parameter. Considering the value of kale (3.1) is close to the generic mean calculated over all crops including kale (3.6), it may be enough rational to let the generic information represent the mean and range of LAI for leafy crops. Therefore, the generic information was used here as default values.

Best estimate (LAI_leaf_harvest) = 3.6 (unitless)

PDF (LAI_leaf_harvest) = LN(GM = 3.6, GSD = 1.5) (unitless)

### 5.2.4 Parameters related to the chemical degradation

#### 5.2.4.1 Degradation rate in leaf /in root (lambda_deg_leaf, lambda_deg_root)

Physical/chemical/biological/empirical meaning

It represents first order decay rate of pollutants in leaf (root). The value varies depends on chemicals of interest.

Description of data source

A literature survey was conducted to collect the data of the parameters. The following table presents the parameter values collected from the literature survey and their sources.

Table 27 Values of lambda_deg_leaf (root) collected from the literature survey

| Abbreviation of parameter | Unit | Value | Remark | Original Source |
| --- | --- | --- | --- | --- |
| lambda_deg_Leaf | d-1 | 0 | Rein et al (2011) present 0 as a generic value for root, stem, leaf, and corn degradation rates. | Rein et al (2011) |
| lambda_deg_root | d-1 | 0.39 | Trapp et al (1994) determined the metabolism rate from measured data (Bromacil in soybeans). The measurements consist of data for stems and roots. The average half-life over stems and roots was found to be 1.8 day. Then metabolism rate was determined by the equation, t_1/2 (half life) = ln2 / K_deg (degradation rate). | Trapp et al (1994) |
|  |  | 0.15 | Legind et al (2011) present a plant metabolism rate (applied to root, stem, leaf, and fruit) for methomyl (at 20°C) which corresponds to a half-life of 4.6 days. The value is representative of a range of measured values. | Legind et al (2011) |
|  |  | 0 | Rein et al (2011) present 0 as a generic value for root, stem, leaf, and corn degradation rates. | Rein et al (2011) |

Parameter default value and PDF

The following table lists, for each parameter, a recommended value selected from the previous table and also presents a probability density function (PDF). The null values that are proposed correspond to conservative assumptions because they express no degradation in the plant.

Table 28 Default values of lambda_deg_leaf (root)

| Abbreviation of parameter | Unit | Best estimate | PDF | Comment |
| --- | --- | --- | --- | --- |
| lambda_deg_leaf | d-1 | 0 | - |  |
| lambda_deg_root | d-1 | 0 | - | A generic value was selected because the parameter is substance-specific and thus the measured values for methomyl and bromacil can not represent all other organic substances. |

### 5.2.5 Parameters related to the diffusion between leaf and air

#### 5.2.5.1 Molar masses of the contaminant/dioxygen/water (M_molar, M_O2, M_H2O)

Physical/chemical/biological/empirical meaning

The molar mass of each contaminant, oxygen and water are determined by the molecular formula.

Parameter default value and PDF

The molar mass of several key substances is presented in Table 29. The molar masses of di-oxygen and water are 32 g mol-1 and 18 g mol-1, respectively.

Table 29  Molar mass of selected substances

| Chemical class | Substance | Molar mass (g mol-1) |
| --- | --- | --- |
| PAH | Anthracene | 178 |
|  | Benzo(a)pyrene | 252 |
|  | Benzo(b)fluoranthene | 252 |
|  | Benzo(k)fluoranthene | 252 |
|  | Fluoranthene | 202 |
|  | Naphtalene | 128 |
| PCB | PCB28 | 257.5 |
|  | PCB 52 | 292 |
|  | PCB101 | 326.5 |
|  | PCB118 | 326.5 |
|  | PCB138 | 361 |
|  | PCB153 | 361 |
|  | PCB180 | 503.5 |
| Pesticides | Alachlor | 269.5 |
|  | Atrazine | 215.5 |
|  | Chlordane | 410 |
|  | Chlorpyrifos | 350.6 |
|  | DDT | 354.5 |
|  | Dieldrin | 358.5 |
|  | Diuron | 233 |
|  | Endosulfan | 407.1 |
|  | Hexachlorocyclohexane | 291 |
|  | Isopruturon | 206 |
|  | Malathion | 447.2 |
|  | Parathion | 291.1 |
|  | Pentachlorophenol | 266.5 |
| Brominated flame retardants | Pentabromo diphenylether | 564.5 |
|  | Hexabromobiphenyl | 627.4 |
| VOCs | Benzene | 78 |
|  | 1,2-Dichloroethane | 99 |
|  | Dichloromethane | 85 |
|  | Hexachlorobenzene (HCB) | 285 |
|  | Hexachlorobutadiene | 261 |
|  | Pentachlorobenzene | 250.5 |
|  | Trichlorobenzene | 181.5 |
|  | Trichloromethane (chloroform) | 119.5 |
| Phthalate | Dibutylphthalate (DBP) | 276 |
|  | Di(2-ethylhexyl)phthalate (DEHP) | 390 |
| Dioxins | 2,3,7,8-TCDD | 322 |
|  | 1,2,3,7,8-PeCDD | 356.5 |
|  | 1,2,3,7,8-HxCDD | 391 |
| Phenols - Alkylphenols | 2,4,6-tri-tert-butylphenol | 262 |
|  | Nonylphenol | 220 |
|  | 2-Octylphenol | 206 |

#### 5.2.5.2 Diffusion coefficient of oxygen in water (D_O2_water)

Physical/chemical/biological/empirical meaning

The diffusion coefficients of oxygen in water depend on temperature.

Description of data source

A literature survey was conducted to collect the data of the parameters. The following table presents the parameter values collected from the literature survey and their sources.

Table 30 Values of D_O2_water collected from the literature survey

| Abbreviation of parameter | Unit | Value | Remark | Original Source |
| --- | --- | --- | --- | --- |
| D_O2_water | m2 d-1 | 1.73E-04 | No indication about temperature. | Trapp and Matthies (1998) |
|  |  | 1.70E-04 | At 20°C, calculated based on the formula developed by Wilke and Chang (1955) and the unit was converted from cm2.S-1 into m2.d-1. | Richard (1996), Wilke and Chang (1955) |

Parameter default value and PDF

The following table lists, for each parameter, a recommended value selected from the previous table and also presents a probability density function (PDF).

Table 31 Default values of D_O2_water

| Abbreviation of parameter | Unit | Best estimate | PDF | Comment |
| --- | --- | --- | --- | --- |
| D_O2_water | m2 d-1 | 1.70E-04 | - | The value from Richard (1996), Wilke and Chang (1955) was selected because the value from Trapp and Matthies (1998) is not presented with temperature. |

#### 5.2.5.3 Thickness of the water layer in leaf (Delta_x_leaf)

Physical/chemical/biological/empirical meaning

It is used to calculate the permeability of water inside plant (P_water).

Description of data source

Van Hove and Adema (1996) calculated the values for the effective thickness of the water film on the leaf surface. The thickness varies from 10 μm at low relative humidity to 100 μm at high relative humidity.

Parameter default value and PDF

The range of thickness estimated by Van Hove and Adema (1996) was used to derive a PDF of this parameter. The mid-value over the range was then used here as a default value.

Best estimate (Delta_x_leaf) = 5.50E-05 (m)

PDF (Delta_x_leaf) = U(Min = 1.0E-5, Max = 1.0E-4) (m)

#### 5.2.5.4 Permeability of the cell wall (P_cell_wall)

Physical/chemical/biological/empirical meaning

See the section 4.5.

Description of data source

Trapp (2000) presents 2.5×10-4 m.s-1 (21.6 in m.d-1)

Parameter default value and PDF

The value found in Trapp (2000) was selected as a default value.

Best estimate (P_cell_wall) = 21.6 (m d-1)

### 5.2.6 Parameters related to the dry/wet depositions of aerosols intercepted by leaves and related to the interception of irrigated water

#### 5.2.6.1 Interception coefficient for dry/wet deposits (mu_dry, mu_wet)

Physical/chemical/biological/empirical meaning

They represent the empirically derived interception (absorption) coefficient for dry/wet deposits. The interception coefficient for wet deposits are used to calculate both the interception of wet deposits of aerosols and the interception of irrigated water.

Description of data source

Queguiner et al (2009) conducted an exhaustive literature review about foliar interception coefficients for particulate metals (dry deposits) and for dissolved metals (wet deposits). As it was found that the quality of data obtained from the literature review was so heterogeneous in terms of the data sources, each data point was assigned a score according to its quality. The generation of PDFs was then made through a direct weighted bootstrap procedure.

No other references provide the data of the interception coefficient for dry/wet deposits.  Queguiner et al (2009) derived the PDFs of those parameters solely for leaves and for metals but neither for other types of crops nor organic substances.

Parameter default value and PDF

The data given by Queguiner et al (2009) were used to derive the best estimates and PDFs for leafy crops.

Table 32 Default values of mu_dry and mu_wet

| Abbreviation of parameter | Unit | Best estimate | PDF | Comment |
| --- | --- | --- | --- | --- |
| mu_dry | m2 kgdw-1 | 1.51 | T(Min = 0.16, Max = 14, Mode = 1.51) | Queguiner et al (2009) assume the log-normality in the distributions of the parameters. However, it was found that the values of mean, 5%, and 95% which are presented in Queguiner et al (2009) don't match the shape of log-normality. Therefore, the type of distribution was transformed from log-normal into triangular, by making the values of 5%, mean, and 95% fit to respectively min, max, and mode of a triangular distribution. |
| mu_wet | m2 kgdw-1 | 1.68 | T(Min = 0.25, Max = 11.5, Mode = 1.68) |  |

### 5.2.7 Parameters related to the transfer from soil to leaf governed by the equilibrium transfer factor

#### 5.2.7.1 Transfer factor from soil to leaf (TF_soil_leaf)

Physical/chemical/biological/empirical meaning

The transfer factor from soil to plant is the equilibrium ratio of the metal concentration in leaves to the metal concentration in soil. This parameter is then calculated by:

TFsoil_leaf=CleafCsoil

where Cleaf denotes the concentration of metal in the edible part of a leafy crop (mg kgdw-1) and Csoil denotes the total concentration of metal in soil (mg kgdw-1).

When Cleaf is obtrained on a basis of fresh weight, it is necessary to convert the unit from fresh into dry weight by using the data of dry matter fraction in a leaf of interest.

TF_soil_leaf may depend on many factors such as soil texture, pH in soil, type of plants, and the experimental conditions. Precise estimation of TF_soil_leaf based on these factors remains uncertain, due to the high variability observed in experimental data.

Description of data source

A literature survey was conducted to collect the data of TF_soil_leaf. In addition of the literature survey, the database called BAPPET (Base de données sur les teneurs en Eléments Traces métalliques de Plantes Potagères, http://www.developpementdurable. gouv.fr/BAPPET-BAse-de-donnees-sur-les.html) was used to obtain the values. The BAPPET stores many experimental data of metal concentration in plants and in soil and those concentrations allows the calculation of TF_soil_leaf by the equation above. The concentration of metal in a plant is given in the database in function of the metal considered, of factors associated with the plant/soil (e.g. type of plant/soil), of environmental context (e.g. industrial, rural, urban, and artificial), and of experimental conditions (e.g. mode of extraction of metal: total extraction, semi-total extraction, and partial). The BAPPET compiles the data together with the information about such differing factors.

Parameter default value and PDF

Most of values of TF_soil_leaf were calculated. The data of Cleaf and Csoil, which are used to calculate the parameter, were obtained from individual scientific article and the BAPPET. Some of the parameter values originated from the BAPPET were a priori filtered out considering the following factors:

- Environmental contexts: The environment where measurements were carried out. The database categorizes the contexts into ‘industrial’ (industrial zone, zone neighboring factories..), rural (zone away from industrial or urban zones), ‘urban’, ‘artificial’ (zone where experiments are conducted with metals in chemical form), and ‘agricultural’.

-  The data obtained in the context of ‘industrial’ were excluded to filter out the studies made in the regions receiving air deposits originated from industrial emissions.

- Mode of metal extraction: The mode of metal extraction in soil. The database categorizes the mode into ‘total’, ‘semi-total’ extractions (using strong acids) and ‘partial’ extraction (using weak solvants).

-  The data associated with ‘partial’ were removed because the equation above considers the total concentration in soil.

- Organ: Part of plant analysed. The database categorizes the organ into ‘consumable (edible) part’ and ‘whole part’.

-  The data associated with ‘whole part’ were removed because the equation above considers the total concentration in soil.

These three factors were in the same way taken into account for individual articles.

The generated parameter values were all transformed into log-scaled values (natural logarithm). Assuming that the transformed values follow the normality, a standard deviation (SD) method was applied to detect outliers according to Seo (2006). In the method, the values greater than X + 2SD or the values smaller than X – 2SD were regarded as outliers (X : mean, SD : standard deviation).

After removing the outliers, the Goodness-of-fit test was used with significant level 5% to evaluate the adjustment of distributions on the parameter values. Log-normal distribution was a priori chosen as the form of PDF. When the log-normality was found to be invalid, the Gamma and Weibull distributions were tested. Table 33 presents a best estimate and a PDF of TF_soil_leaf for each selected metal.

Table 33  Default values of TF_soil_leaf for selected metals

| Metal | Best estimate | PDF | Number of data |
| --- | --- | --- | --- |
| Al | 8.32E-04 | LN(8.32E-04, 6.85)* | 31 |
| As | 1.00E-02 | LN(1.00E-02, 2.29) | 12 |
| B | 9.88 | LN(9.88, 4.21) | 47 |
| Cd | 1.22 | LN(1.22, 2.92) | 227 |
| Cr | 1.07E-02 | LN(1.07E-02, 3.95) | 46 |
| Cu | 1.50E-01 | LN(1.50E-01, 2.63) | 180 |
| Fe | 7.25E-03 | LN(7.25E-03, 11.0) | 39 |
| Mn | 3.98E-01 | LN(3.98E-01, 9.49) | 59 |
| Pb | 1.73E-02 | LN(1.73E-02, 5.01) | 137 |
| Zn | 5.06E-01 | LN(5.06E-01, 2.70) | 180 |

* LN signifies log-normal distribution. The first and second values in bracket following the symbol represent geometric mean and geometric standard deviation, respectively.

### 5.2.8 Parameters related to the transfer from leaves to soil by weathering

#### 5.2.8.1 Weathering (wash-off) loss rate from leaf surface to soil (lambda_weathering_leaf)

Physical/chemical/biological/empirical meaning

The parameter represents the loss rate of the pollutant from leaf surface into soil by the wash-off processes (climate process like rain and wind).

Description of data source

Loss of activity from the foliar surface has several origins, such as wind removal and water removal. All these losses are represented by a global loss constant for foliar surface. Weathring was especially studied for radionuclides. Miller and Hoffman (1983) reviewed 78 values determined under various experimental conditions for several elements (Mn, Co, Sr, Ag, and Cs). Leclerc and Choi (2009) extended this literature review for other elements (e.g. I, Ru) and several crops (grass, cereals, rice, cabbage, etc). These data are in general agreement with those previously collected by Miller and Hoffman (1983). Over some elements, Miller and Hoffman (1983) proposed a generic PDF.

The weathering process is a physical process which is assumed to take place for all types of chemicals, either metals or organic compounds. Therefore, this process should be considered for organic compounds, even though no data on this parameter have been found for them.

Parameter default value and PDF

The PDF proposed by Miller and Hoffman (1983) was used as default values for metals. As for organic substances, we propose the null value as a default due to the lack of knowledge. Miller and Hoffman (1983) present the PDF with the values of 5 and 95 percentiles. Those values were then transformed into a geometric mean and a geometric standard deviation.

Best estimate (lambda_weathering_leaf) = 4.11E-02 for metals, 0 for organics (d-1)

PDF (lambda_weathering_leaf) = LN(GM = 4.11E-02, GSD = 1.6) for metals (d-1)

End of Level 3 documentation (end-user with expertise in parameterization)

Level 4 documentation (mathematical information)

# Mass balance equation

The Leaf model developed for organic substances consists of root and leaf compartments whereas the model for metals considers only the leaf compartment.  In each compartment, a mass balance, which accounts for loading, loss, and accumulation of the pollutant, is taken into account. Following equations express mass balances in the root (‘leaf root’) and leaf compartments. It should be noted that the loss by harvest is not included in the mass balance equations because the loss process is considered only at the end of the plant growing period during which pollutants are accumulated in plant.

- Mass balance equations used for organic substances

For the root compartment:

For the leaf compartment:

- Mass balance equations used for metals

Where

- Qroot_leaf (mg) : Total quantity of the pollutant in leaf roots

- QLeaf (mg) : Total quantity of the pollutant in leaf

- Xylem_influx (mg d-1) : Transfer from soil to root by xylem influx

- Xylem_ outflux (d-1) : Loss rate from root to leaf by xylem outflux

- Loss_deg_root/Loss_deg_leaf (d-1) : Loss by degradation in root/leaf

- Irrigation_intercepted (mg d-1) : Transfer to leaf by the leaf interception of irrigated water

- Diffusion_upwards (d-1) : Loss rate from leaf to air by diffusion

- Diffusion_downwards (mg d-1) : Transfer from air to leaf by diffusion

- Dry_deposition_intercepted/Wet_deposition_aerosol_intercepted (mg d-1): Transfer from air to leaf by the leaf interception of the dry/wet depositions of contaminated aerosols

- Uptake_metals (mg d-1) : Transfer from soil to leaf by the equilibrium transfer factor

- Weatheing (d-1) : Loss from leaf to soil by weathering (wash-off) process

# Calculation of state variables

This chapter presents the equations to calculate the state variables used in the Leaf model. Those equations are grouped by differing transfer processes shown in the equations (1), (2), and (3). Together with the equations, state variables are depicted by the following three symbols:

## Xylem influx (Xylem_influx)

Xylem_influx represents the transfer of the pollutant from soil to root, driven by transpiration stream.

(4)

Where

- Xylem_influx (mg d-1) : Transfer from soil to root by xylem influx

- Transpiration (m3 m-2 d-1) : Transpiration stream

- Kdsoil (m3 g-1) :  Distribution coefficient of the pollutant between soil and water

- Csoil (mg kgdw-1 ) : Concentration of the pollutant in root zone of soil (on dry mass basis)

- Sfield (m2) : Surface area of field

- 1000 is the unit correction factor with the unit of g kg-1

### Transpiration stream (Transpiration)

The plant transpiration can be estimated from actual evapotranspiration as a function of LAI (leaf area index) and an extinction factor:

(5)

Where

- Transpiration (m3 m-2 d-1) : Transpiration stream

- ETa (mm d-1): Actual evapotranspiration

- αextinction (unitless) :  Extinction factor for partitioning of evapotranspiration to plant transpiration

- LAILeaf (unitless or m2 leaf.m-2 soil): Leaf area index for leafy crops

- 0.001 is the unit correction factor with the unit of m mm-1

### Leaf area index for leaf (LAIleaf)

LAI is defined as the one-sided green leaf area per unit ground surface area for Leafy crops. The canopy area is assumed to grow linearly over the growing period of Leaf:

(6)

Where

- LAIleaf (unitless or m2 leaf.m-2 soil): Leaf area index for leafy crops

- tharv_leaf (d) : Date of harvest of a leafy crop

- tgerm_leaf (d) : Date of germination of a leafy crop

- LAIleaf_harvest (m2 m-2) : leaf area index at harvest for leafy crops

- t (d) : The current day of the year

### Distribution coefficient of the pollutant between soil and water (Kd_soil)

Kd_soil represents equilibrium partitioning between concentrations in soil pore water (mg m-3) and in soil solids (mg g-1). It is derived from the fraction of organic matter in soil and the partition coefficient between water and organic carbon:

(7)

Where

- Kdsoil (m3 g-1) :  Distribution coefficient of the pollutant between soil and water

- fOM_soil (g g-1) : Fraction of organic matter in soil

- logKoc (unitless) : Water-organic carbon partition coefficient (in log10)

- 0.001 is the unit correction factor with the unit of m3 L-1

- has the unit of L kg-1

- 10-6 is the unit correction factor with the unit of m3 L-1. kg g-1

## Xylem outflux (Xylem_ Outflux)

Xylem_ outflux represents the loss rate from root to leaf due to xylem and phloem flows (d-1):

(8)

Where

- Transpiration (m3 m-2 d-1) : Transpiration stream

- Kroot_water (L kgfw-1) : Partition coefficient between root and water

- mroot_leaf (kgfw m-2) : Mass of root for a leafy crop per unit area of soil

- 0.001 is the unit correction factor with the unit of m3 L-1

### Mass of root for a leafy crop per unit area of soil (m_root_leaf)

The mass of leaf is assumed to grow linearly over the growing period of leaf:

(9)

Where

- mroot_leaf (kgfw m-2): Mass of root for a leafy crop per unit area of soil

- mroot_leaf_harvest (kgfw m-2) : Mass of root for a leafy crop per unit area of soil at harvest

- tharv_leaf (d) : Date of harvest of a leafy crop

- tgerm_leaf (d) : Date of germination of a leafy crop

### Partition coefficient between root and water (K_root_water)

K_root_water is the equilibrium partitioning between concentrations in roots (mg kg fw-1) and in water or xylem sap (mg L-1). K_root_water considers the sorption to root, the dissolution into the aqueous solution of root cells, and the partition to the gas phase of root.

(10)

Where

- Kroot_water (L kgfw-1) : Partition coefficient between root and water

- θroot (L kgfw-1) : Water content of root

- Lroot (kg kgfw-1) : Lipid content of root

- δdensity_OW (L kg-1) : Correction for density difference between water and n-octanol

- logKow (unitless) : Octanol-water partition coefficient (in log 10)

- δsolubility_lipids_root (unitless) : Empirical correction factor for differences between solubility in octanol and sorption to root lipids

- Groot (L kgfw-1) : Air content of root

- Kair_water (m3 m-3) : Partition coefficient between air and water

- has the unit of m3 m-3

### Partition coefficient between air and water (K_air_water)

K_air_water is the equilibrium partitioning between concentrations in air and in water. It is an expression of Henry’s law constant in terms of a dimensionless ratio concentration:

(11)

Where

- Kair_water (m3 m-3) : Partition coefficient between air and water

- H (Pa m3 mol-1) : Henry’s law constant

- R (Pa m3 mol-1 K-1) : Universal gas constant

- Tair_Kelvin (K) : Temperature of air (in Kelvin)

(12)

Where

- Tair_Kelvin (K) : Temperature of air (in Kelvin)

- Tair (°C) : Temperature of air (in Celsius)

## Loss by degradation in root/leaf (Loss_deg_root/Loss_deg_Leaf)

The degradation of the pollutant in root or leaf is expressed by the first order decay rate (d-1):

- λdeg_root (d-1) : Degradation rate in root (See equation (1))

- λdeg_leaf (d-1) : Degradation rate in Leaf (See equation (2))

## Diffusion upwards/downwards (Diffusion_upwards/Diffusion_downwards)

Loss rate of the pollutant from leaf to air by diffusion (Diffusion_upwards, d-1) is calculated as follows:

(13)

Where

- Diffusion_upwards (d-1) : Loss rate from Leaf to air by diffusion

- mleaf (kgfw m-2) : Mass of leaf per unit area of soil

- LAILeaf (m2 m-2) : Leaf area index for leafy crops

- gleaf (m d-1) : Conductance of leaf

- Kleaf_air (m3 kgfw-1) : Partition coefficient between leaf and air

Note that 2 × LAIleaf in the equation represents the up and downside leaf surface area.

Transfer of the pollutant from air to leaf by diffusion (Diffusion_downwards, mg d-1) is calculated as follows:

(14)

Where

- Diffusion_downwards (mg d-1) : Transfer from air to leaf by diffusion

- Cgas_atm (mg m-3) : Gaseous concentration in the atmosphere

- Sfield (m2) : Surface area of field

### Mass of leaf per unit area of soil (m_leaf)

The mass of leaf is assumed to grow linearly over the growing period of leaf:

(15)

Where

- mleaf (kgfw m-2): Mass of leaf per unit area of soil

- mleaf_harvest (kgfw m-2) : Mass of leaf per unit area of soil at harvest

- tharv_leaf (d) : Date of harvest of a leafy crop

- tgerm_leaf (d) : Date of germination of a leafy crop

### Partition coefficient between leaf and air (K_leaf_air)

The phase equilibrium between leaf and air is found from the partition coefficient between leaf and water (K_leaf_water) and the partition coefficient between air and water (K_air_water):

(16)

Where

- Kleaf_air (m3 kgfw-1) : Partition coefficient between leaf and air

- Kleaf_water (L kgfw-1) : Partition coefficient between leaf and water (presented in the equation (29))

- Kair_water (m3 m-3) : Partition coefficient between air and water

- 0.001 is the unit correction factor with the unit of m3 L-1

### Conductance of leaf (g_leaf)

The conductance of leaf is related to the permeability of leaf. The difference is that the permeability is related to concentrations in water, while conductance is related to concentrations in the gas phase:

(17)

Where

- gleaf (m d-1) : Conductance of leaf

- Pleaf (m d-1) : Permeability of leaf

- Kair_water (m3 m-3) : Partition coefficient between air and water

### Permeability of leaf (P_leaf)

The leaf permeability is the sum of the total cuticle and the stomata permeabilities:

(18)

Where

- PLeaf (m d-1) : Permeability of Leaf

- Pcuticle_tot (m d-1) : Total permeability of the cuticle pathway

- Pstomata (m d-1) : Permeability of the stomata

### Total permeability of the cuticle pathway (P_cuticle_tot)

The total cuticle permeability is assumed to result from four permeabilities in parallel:

(19)

Where

- Pcuticle_tot (m d-1) : Total permeability of the cuticle pathway

- Pair (m d-1) : Permeability of the air boundary layer around plant

- Pcuticle (m d-1) : Permeability of the cuticle

- Pwater (m d-1) : Permeability of water inside plant

- Pcell_wall (m d-1) : Permeability of the cell wall

### Permeability of the air boundary layer around plant (P_air)

A resistance of 200 s m-1 was estimated as typical for a chemical with a molecular mass of 300 g mol-1 for the air boundary layer (Thompson 1983). The ratio of square molar masses is used to derive the permeability in air for any chemicals:

(20)

Where

- Pair (m d-1) : Permeability of the air boundary layer around plant

- Kair_water (m3 m-3) : Partition coefficient between air and water

- Mmolar (g mol-1) : Molar mass of the contaminant

- 86400 is the unit correction factor with the unit of second day-1

- has the unit of m second-1

### Permeability of the cuticle (P_cuticle)

A regression equation was derived for the permeability of citrus cuticles (Kerler and Schonherr 1988):

(21)

Where

- Pcuticle (m d-1) : Permeability of the cuticle

- 86400 is the unit correction factor with the unit of second day-1

- has the units of m second-1

### Permeability of water inside plant (P_water)

The permeability of water inside plant is calculated from the effective diffusion coefficient of a chemical in water:

(22)

Where

- Pwater (m d-1) : Permeability of water inside plant

- Dwater (m2 d-1) : Effective diffusion coefficient in water

- Δx_Leaf (m) : Thickness of the water layer in leaf

### Effective diffusion coefficient in pure water (D_water)

For organic substances, the effective diffusion coefficient in water is related to the square root of the molar mass and also related to the diffusion coefficient of oxygen in water whereas the diffusion coefficient in air is related to the square root of the molar mass and also related to the diffusion coefficient of oxygen in water:

(23)

Where

- Dwater (m2 d-1) : Effective diffusion coefficient in pure water

- DO2_water (m2 d-1) : Diffusion coefficient of oxygen in water

- MO2 (g mol-1) : Molar mass of dioxygen

- Mmolar (g mol-1) : Molar mass of the contaminant

### Permeability of the stomata (P_stomata)

The permeability of the stomatal pathway, related to water, is the stomatal conductance times partition coefficient air-water (non-dimensional Henry’s law constant):

(24)

Where

- Pstomata (m d-1) : Permeability of the stomata

- gstomata (m d-1) : Conductance of the stomatal pathway for the chemical

- Kair_water (m3 m-3) : Partition coefficient between air and water

### Conductance of the stomatal pathway for the chemical (g_stomata)

The conductance of the chemical in stomata depends on the molar mass according to the following relationship:

(25)

Where

- gstomata (m d-1) : Conductance of the stomatal pathway for the chemical

- gH2O (m d-1) : Conductance of the stomatal pathway for water

- MH2O (g mol-1) : Molar mass of water

- Mmolar (g mol-1) : Molar mass of the contaminant

### Conductance of the stomatal pathway for water (g_H2O)

The loss of water from leaf through transpiration stream is assumed to follow Fick’s law and the interior of leaf is assumed to be water-saturated. If the leaf surface area (up and downside) is known:

(26)

Where

- gH2O (m d-1) : Conductance of the stomatal pathway for water

- Transpiration (m3 m-2 d-1) : Transpiration stream

- LAILeaf (m2 m-2) : Leaf area index for leafy crops

- rh (unitless) : Relative humidity

- CH2O_sat (kg m-3) : Concentration of water vapour in air at saturation

- 1000 is the unit correction factor with the unit of kg m-3 (identical to the density of water)

### Concentration of water vapour in air at saturation (C_H2O_sat)

On the basis of the Ideal Law of Gases, the saturation water concentration at a given temperature (in Kelvin) is calculated by the saturation vapour pressure of water:

(27)

Where

- CH2O_sat (kg m-3) : Concentration of water vapour in air at saturation

- MH2O (g mol-1) : Molar mass of water

- Pwater_sat (Pa) : Saturation vapour pressure of water at given temperature

- R (Pa m3 mol-1 K-1) : Universal gas constant

- Tair_Kelvin (K) : Temperature of air (in Kelvin)

- 0.001 is the unit correction factor with the unit of kg g-1

### Saturation vapour pressure of water at given temperature (p_water_sat)

The saturation vapour pressure of water at given temperature (in Celsius) can be calculated by the empirical Magnus-equation:

(28)

Where

- Pwater_sat (Pa) : Saturation vapour pressure of water at given temperature

- Tair (°C) : Temperature of air (in Celsius)

### Partition coefficient between leaf and water (K_leaf_water)

The phase equilibrium between leaf and water considers sorption to leaf lipids, dissolution into the aqueous solution of leaf cells, and gaseous phase in leaf:

(29)

Where

- Kleaf_water (L kgfw-1) : Partition coefficient between leaf and water

- ϴleaf (L kgfw-1) : Water content of leaf

- Lleaf (kg kgfw-1) : Lipid content of leaf

- δdensity_OW (L kg-1) : Correction for density difference between water and n-octanol

- logKow (unitless) : Octanol-water partition coefficient (in log 10)

- δsolubility_lipids_leaf (unitless) : Empirical correction factor for differences between solubility in octanol and sorption to leaf lipids

- Gleaf (L kgfw-1) : Air content of leaf

- Kair_water (m3 m-3) : Partition coefficient between air and water

- has the unit of m3 m-3

## Dry/wet deposition from air to leaf (Dry_deposition_intercepted/Wet_deposition_aerosol_intercepted)

Transfer from air to leaf by the leaf interception of the dry/wet depositions of contaminated aerosols is calculated as follows:

(30)

(31)

Where

- Dry_deposition_intercepted (mg d-1) : Dry deposition aerosols intercepted

- Wet_deposition_aerosol_intercepted (mg d-1) : Wet deposition aerosols intercepted

- fdry_Interception_leaf (unitless) : Dry interception fraction onto leaves

- fwet_Interception_leaf (unitless) : Wet interception fraction onto leaves

- Drydeposition (mg d-1 m-2) : Surface dry deposition flux of contaminated aerosols

- Wetdeposition_ aerosol (mg d-1 m-2) : Surface wet deposition flux of contaminated aerosols

- Sfield (m2) : Surface area of field

### Dry/wet interception fraction onto leaves (f_dry_interception_leaf/f_wet_interception_leaf)

The fraction of pollutants intercepted by leaves is calculated from the leaf biomass and absorption coefficients.

(32)

(33)

Where

- fdry_Interception_leaf (unitless) : Dry interception fraction onto leaves

- fwet_Interception_leaf (unitless) : Wet interception fraction onto leaves

- µdry (m2 kgdw-1) : interception coefficient for dry deposits

- µwet (m2 kgdw-1) : interception coefficient for wet deposits

- mLeaf (kgfw m-2) : Mass of leaf per unit area of soil

- ϴLeaf (L kgfw-1) : Water content of leaf

## Interception of irrigated water (Irrigation_intercepted)

The transfer of chemical substances from irrigated water (coming from river water) to leaves due to the interception of irrigated water is calculated as follows:

(34)

Where

- Irrigation_intercepted (mg d-1) : Transfer to leaf by the leaf interception of irrigated water

- Irrigationrate (m d-1) : Irrigation rate of cultivated fields

- Sfield (m2) : Surface area of field

- fwet_interception_leaf: Wet interception fraction onto plant leaves (presented in the equation (33))

- Cwater: Concentration in raw water

## Uptake of metals (Uptake_metals)

The uptake of metals from soil into leaves is governed by a substance-specific equilibrium transfer factor:

(35)

Where

- Uptake_metals (mg d-1) : Uptake of metals

- TFsoil_leaf (kgdw kgdw-1) : Transfer factor from soil to leaf

- ϴleaf (L kgfw-1) : Water content of leaf

- tharv_leaf (d) : Date of harvest of a leafy crop

- tgerm_leaf (d) : Date of germination of a leafy crop

- mleaf_harvest (kgfw m-2) : Mass of leaf per unit area of soil at harvest

- Csoil (mg kgdw-1 ) : Concentration in soil (on dry mass basis)

- Sfield (m2) : Surface area of field

## Loss from leaf to soil by weathering (Weathering)

The Leaf model calculates the weathering (wash-off) process by a constant loss rate (lambda_weathering_leaf), expressed in day-1.

- λweathering_leaf (d-1) : Loss rate of the pollutant from leaf surface into soil by the wash-off process (climate process like rain and wind) (See equation (3))

## Concentration in leaf at harvest (C_leaf)

The concentration of the pollutant in leaf at harvest time (C_leaf) is calculated as follows:

(36)

Where

- Cleaf (mg kgfw-1) : Concentration in leaf at harvest

- Qleaf_harvest (mg) : Quantity in leaf at harvest, which is derived from Qleaf

Reference

Abraham MH, Andonian-Haftvan J, Whiting GS, Leo A, Taft RS (1994) Hydrogen bonding. Part 34. The factors that influence the solubility of gases and vapors in water at 298 K, and a new method for its determination. J. Chem. Soc. Perkin Trans. 2 1777-1791.

Allen RG, Pereira LS, Raes D, Smith M (1998) Crop evapotranspiration – Guidelines for computing crop water requirements – FAO Irrigation and drainage paper 56. FAO – Food and Agriculture Organization of the United Nations, Rome. Asner G, Scurlock J, Hicke J (2003) Global synthesis of leaf area index observations: implications for ecological and remote sensing studies. Global Ecology & Biogeography 12: 191– 20.

Baes CF, Sharp RD (1983) A proposal for estimation of Soil Leaching and Leaching Constants for Use in Assessment Models. Journal of Environment Quality 12 (1): 17-28. Barber JL, Thomas GO, Kerstiens G, Jones KC (2004) Current issues and uncertainties in the measurement and modelling of air–vegetation exchange and within-plant processing of POPs. Environmental Pollution 128 (1-2): 99-138.

Brand E, Otte PF, Lijzen JPA (2007) CSOIL 2000: an exposure model for human risk assessment of soil contamination. RIVM Report 711701054/2007. Bilthoven, NL, National Institute of Public Health and the Environment.

Brandes L J, des Hollander H, van de Meent D. (1996) SimplBox 2.0: A nested multimedia fate model for evaluating the environmental fate of chemicals. RIVM report no 719101029.

Breuer L, Eckhardt K, Frede HG (2003) Plant parameter values for models in temperate climates. Ecological Modelling 169: 237-293.

Briggs GG, Bromilow RH, Evans AA (1982) Relationships between lipophilicity and root uptake and translocation of non-ionised chemicals by barleya. Pestic. Sci. 13: 495-504.

Briggs GG, Bromilow RH, Evans AA, Williams M (1983) Relationships between lipophilicity and the distribution of non-ionised chemicals in barley shoots following uptake by the roots. Pestic. Sci. 14: 492-500.

Chamberlain AC (1970) Interception and retention of radioactive aerosols by vegetation. Atmospheric Environment 4: 57-78.

Diamond ML, Priemer DA, Law NL (2001) Developing a multimedia model of chemical dynamics in an urban area. Chemosphere 44 (7): 1655-1667.

EC(European Commission) (2003) Technical Guidance Document on Risk Assessment in support of Commission Directive 93/67/EEC on Risk Assessment for new notified substances, Commission Regulation (EC) No 1488/94 on Risk Assessment for existing substances, and Directive 98/8/EC of the European Parliament and of the Council concerning the placing of biocidal products on the market. Italy, European Communities.

Francisco X, Kenneth LC, Boote J, Pineiro J, Sau F (2008) Improving the CERES-Maize Model ability to simulate water deficit impact on maize production and yield components. Agronomy Journal 100 (2): 296-307.

Franco A, Fu W, Trapp S (2009) Influence of soil pH on the sorption of ionizable chemicals: Modeling advances. Environ. Toxicol. Chem. 28: 458-464.

Franco A, Trapp S (2008) Estimation of the soil-water partition coefficient normalized to organic carbon for ionizable organic chemicals. Environ. Toxicol. Chem. 27: 1995-2004. Hoffman FO, Thiessen KM, Frank ML, Blaylock BG (1992) Quantification of the interception and initial retention of radioactive contaminants deposited on pasture grass by simulated rain. Atmospheric Environment 26A (18): 3313-3321. Hoffman FO, Thiessen KM, Rael RM (1995) Comparison of interception and initial retention of wet-deposited contaminants on leaves of different vegetation types. Atmospheric Environment 29 (15): 1771-1775.

Huuskonen J (2003) Prediction of soil sorption coefficient of organic pesticides from the atom-type electrotopological state indices. Environ. Toxicol. Chem. 22: 816-820.

IAEA (2010) Handbook of parameter values for the prediction of radionuclide transfer in terrestrial and freshwater environments. Technical Reports Series No.472.

Jeffries J, Martin I (2008) Updated technical background to the CLEA model. SC050021/SR3. Bristol, UK, Environment Agency. Science Report.

Kool D, Agam N, Lazarovitch N, Heitman JL, Sauer TJ, Ben-Gal A (2014) A review of approaches for evapotranspiration partitioning. Agricultural and Forest Meteorology 184: 56-70. Kerler F, Schonherr J (1988) Permeation of lipophilic chemicals across plant cuticles - prediction from partition-coefficients and molar volumes. Arch. Environ. Con. Tox. 17(1): 7-12.

Kühne R, Ebert R-U, Schüürmann G (2005) Prediction of the temperature dependency of Henry's Law constant from chemical structure. Environ. Sci. Technol. 39: 6705-6711. Leclerc E, Choi YH (2009) Quantification of Radionuclide Transfer in Terrestrial and Freshwater Environments for Radiological Assessments. IAEA-TECDOC-1616 P45-49.

Legind CN, Trapp S (2009) Modeling the exposure of children and adults via diet to chemicals in the environment with crop-specific models. Environ. Pollut. 157: 778-785. Legind CN, Trapp S (2009) Modeling the exposure of children and adults via diet to chemicals in the environment with crop-specific models. Environmental Pollution 157, Supporting information.

Legind CN, Kennedy CM, Rain A, Snyder N, Trapp S (2011) Dynamic plant uptake model applied for drip irrigation of an insecticide to pepper fruit plants. Pest. Manag. Sci. 67: 521-527.

Legind CN, Trapp S (2010) Review of features, events and processes incorporated in existing models for chemical uptake into plants and animals – Proposal of the conceptual and mathematical 2-FUN model for assessing transfer of chemicals to plants and animals. Written as Deliverable 2.4 in the framework of the 2-FUN project (FP6 Project-2005-Global-4 Integrated Project - Contract n°: 036976).

Mckone TE (1993) CalTOX: a multimedia total-exposure model for hazardous-wastes sites: Part II: Multimedia transport and transformation model. Prepared for the State of California, UCRL-CR-111456PtII. Lawrence Livermore National Laboratory, Livermore, CA, USA.

Meylan WM, Howard PH (1991) Bond contrlbution method for estimating Henry’s law constants. Environmental Toxicology and Chemistry 10: 283-1293. Meylan WM, Howard PH (1995) Atom/fragment contribution method for estimating octanol-water partition coefficients. J. Pharm. Sci. 84: 83-92. Miller CW, Hoffman FO (1983) An examination of the environmental half-time for radionuclides deposited on vegetation. Health Physics 45(3): 731-744. Nikolova N, Jaworska J (2005) An approach to determining applicability domains for QSAR group contribution models: An analysis of SRC KOWWIN. ATLA 33: 461-470. OECD (1995) OECD guidelines for the testing of chemicals - Partition Coefficient (n-Octanol/Water): Shake Flask Method. OECD (2004) OECD guidelines for the testing of chemicals - Partition Coefficient (n-Octanol/Water): High Performance Liquid Chromatography (HPLC) Method. OECD (2006) OECD guidelines for the testing of chemicals - Partition Coefficient (1-Octanol/Water): Slow-Stirring Method.

Panagos P, Hiederer R, Van Liedekerke M, Bampa F (2013) Estimating soil organic carbon in Europe based on data collected through an European network. Ecological Indicators 24: 439–450. Peach L, Benjamin LR, Mead A (2000) Effects on the growth of carrots (Daucus carota L.), cabbage (Brassica oleracea var. capitata L.) and onion (Allium cepa L.) of restricting the ability of the plants to intercept resources. Journal of Experimental Botany 51(344): 605-615. Pinder JE, Ciravolo TG, Bowling JW (1988a) The interrelationships among plant biomass, plant surface area and the interception of particulate deposition by grasses. Health Physics 55(1): 51-58., Pinder JE, McLeod KW (1988b) Contaminant transport in agroecosystems through retention of soil particles on plant surfaces. J. Environ. Qual. 17(4): 602-607. Poole SK, Poole CF (1999) Chromatographic models for the sorption of neutral organic compounds by soil from air and water. J. Chromatogr. A. 845: 381-400.

Pröhl G, Hoffman FO (1996) Radionuclide interception and loss processes in vegetation. In Modelling radionuclide interception and loss processes in vegetation and of transfer in semi-natural ecosystem. IAEA-TECDOC-857.

Queguiner S, Ciffroy P, Roustan Y, Musson-Genon L (2009) Multimedia modelling of the exposure to cadmium and lead released in the atmosphere - Application to industrial releases in a mediterranean region and uncertainty/sensitivity analysis. Water Air Soil Pollut. 198: 199-217.

Rein A, Legind CN, Trapp S (2011) New concepts for dynamic plant uptake models. SAR and QSAR in Environmental Research 22: 191-215.

Richard T (1996) Calculating the oxygen diffusion coefficient in water. http://compost.css.cornell.edu/oxygen/oxygen.diff.water.html, checked in 01/03/2014.

Riederer M (1995) Partitioning and transport of organic chemicals between the atmospheric environment and leaves. In S Trapp and JC McFarlane (Eds.), Plant contamination. Modeling and simulation of organic chemical processes, Florida, CRC Press.

Rikken MGJ, Lijzen JPA, Cornelese AA (2001) Evaluation of model concepts on human exposure. RIVM Report 711701022/2007. Bilthoven, NL, National Institute of Public Health and the Environment.

Ritchie JT (1972) Model for predicting evapotranspiration from a row crop with incomplete cover. Water Resour. Res. 8: 1204–1213.

Ritchie JT (1985) A user-oriented model of the soil water balance in wheat. p. 293–305. In W Day and RK Atkin (ed.) Wheat growth and modelling. NATO-ASI Science, New York.

Sabljic A, Güsten H, Verhaar H, Hermens J (1995) QSAR modelling of soil sorption. Improvements and systematics of log KOC vs. log KOW correlations. Chemosphere 31: 4489- 4514, Corrigendum: Vol. 33 (1996): p2577.

Sau F, Boote KJ, Bostick WM, Jones JW, Minguez MI (2004) Testing and Improving Evapotranspiration and Soil Water Balance of the DSSAT Crop Models. Agronomy Journal 96: 1243-1257.

Schüürmann G, Ebert R-U, Kühne R (2006) Prediction of the sorption of organic compounds into soil organic matter from molecular structure. Environ. Sci. Technol. 40: 7005-7011.

Seo S (2006) A Review and Comparison of Methods for Detecting Outliers in Univariate Data Sets. University of Pittsburgh, United States.

Seuntjes P, Steurbaut W, Vangronsveld J (2006) Chain model for the impact analysis of contaminants in primary food products. D/2006/1191/29. Brussels, Belgian Science Policy. Smith KEC, Jones KC (2000) Particles and vegetation: implications for the transfer of particle-bound organic contaminants to vegetation. The Science of The Total Environment 246 (2-3): 207-236.

Tahiri AZ, Anyoji H, Yasuda H (2006) Fixed and variable light extinction coefficients for estimating plant transpiration and soil evaporation under irrigated maize. Agricultural water management 84: 186-192.

Tao S, Piao H, Dawson R, Lu X, Hu H (1999) Estimation of organic carbon normalized sorption coefficient (Koc) for soils using the fragment constant method. Environ. Sci. Technol. 33: 2719-2725. Thompson N (1983) Diffusion and uptake of chemical vapor volatilizing from a sprayed target area. Pest.Sci. 14(1): 33-39.

Trapp S (2000) Modelling uptake into roots and subsequent translocation of neutral and ionisable organic compounds. Pest Manag.Sci. 56(9): 767-778.

Trapp S (2002) Dynamic root uptake model for neutral lipophilic organics. Environmental Toxicology and Chemistry 21: 203-206. Trapp S (2004) Plant uptake and transport models for neutral and ionic chemicals. Environ Sci & Pollut Res 11(1): 33-39.

Trapp S, Matthies M (1995) Generic one-compartment model for uptake of organic-chemicals by foliar vegetation, Environ. Sci. Technol. 29: 2333–2338, Erratum Vol.30: p360.

Trapp S, Matthies M (1998) Chemodynamics and Environmental Modeling. An Introduction. Springer, berlin, Germany.

Trapp S, McFarlane C, Matthies M (1994) Model for uptake of xenobiotics into plants: validation with bromacil experiments. Environmental Toxicology and Chemistry 13: 413-422.

Trapp S, Schwartz S (2000) Proposals to overcome limitations in the EU chemical risk assessment scheme. Chemosphere 41(7): 965-971.

U.S. Environmental Protection Agency (2002) TRIM: Total Risk Integrated Methodology. Volume II-Description of Chemical Transport and Transformation Algorithms. EPA-453/R-02-011b. Washington, DC. Vandecasteele CM, Baker S, Förstel H., Muzinsky M, Millan R, Madoz-Escande C, Tormos J, Sauras T, Schulte E, Colle C (2002) Interception, retention and translocation under greenhouse conditions of radiocaesium and radiostrontium from a simulated accidental source. Sci. Total Environment 278: 199-214.

Van Hove LWA, Adema EH (1996) Short communication: The effective thickness of water films on leaves. Atmospheric Environment 30: 2933-2936.

Viswanadhan VN, Ghose AK, Singh UC, Wendoloski JJ (1999) Prediction of solvation free energies of small organic molecules. Additive-constitutive models based on molecular fingerprints and atomic constants. J. Chem. Inform. Comput. Sci. 39: 405-412.

Wang M, Jones KC (1994) Uptake of chlorobenzenes by carrots from spiked and sewage sludgeamended soil. Environ. Sci. Technol. 28:1260-1267.

Wilke CR and Chang P (1955) A.E.CH.E. Journal 1: 264-270.

2Chemical classes in the Meylan’s database with indication of the number of chemicals: Alkanes 16 ; Alkenes 20 ; Alkynes 7 ; Acids, aliphatic 6; Alcohols 18; Aldehydes 17; Esters 27; Ethers 16; Epoxides 2; Ketones 9; Halomethanes 22; Haloethanes 20; Halopropanes 11; Halobutanes 9; Other haloalkanes 4; Haloalcohols 5; Haloalkenes 12; Aliphatic amines 13; Nitriles 5; Other aliphatic nitrogen compounds 11; Aliphatic sulfur compounds 8; Five-member aromatic rings 3; P yridines 12; Benzene and alkylated; benzenes 13; Halogenated benzenes 12; Anilines 3; Phenols 8; Biphenyls 3; Polyaromatics 13; Other aromatics 14; Pesticides 6

The topological index was introduced by Kier et Hall (1990, 1999) following the suggestions of Randic (1975). nχ is a n-order topological index where n represents the number of atoms (except H) linked to each atom (except H) belonging to the molecule.

Defined as molecules containing only C, H and halogen (F, Cl, Br, I)

Defined as all the molecules that contains other atoms than C, H and halogen (F, Cl, Br, I). Does not imply anything about their lipophilicity.